3GPP TSG-SA5 Meeting #161	S5-252635

Fukuoka, Japan, 19 - 23 May 2025

Source:	Huawei

Title:	Rel-19 pCR TS 28.561 Update verification of network response to events

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS 28.561

Version:	0.2.0

Work Item:	NDT

Comments

This contribution is proposed to update the use case for Verification of network response to events.

The main purpose of this use case is to show that “The NDT can be used to evaluate how the network responds any of these events. The NDT can model network behaviors and provide information on potential impact of network events.”

But the example as provided does not align with this use case.

Proposed Changes

* * * First Change * * * *

#### 5.3.2.2	Verification of network response to events – NDTVER_02

The network can experience different kinds of events, including failure and non-failure events. A failure event refers to the situation where the network does not behave as expected, e.g., where users in the network are completely unable to get service from the network. A non-failure event refers to the situation where the network experiences an unusual occurrence but users are still able to get service. The NDT can be used to evaluate how the network responds any of these events. The NDT can model network behaviors and provide information on potential impact of network events. The NDT can be used to evaluate if a particular event will cause a network failure or whether the network can withstand the event.

The following is an example of events that can be supported:

- 	Signaling storms: A signaling storm is a failure events where a large number of signaling messages suddenly surge in the mobile communication network, resulting in the network processing capacity overload, thus affecting the network performance and stability. During this period, users will repeatedly try to establish the connection until reconnected, thus generating a large number of signaling messages surge suddenly, causing signaling storm. In this scenario, NDT is used to simulate/emulate the network behaviour of surges in signaling requests on the network and report whether a signaling storm may occur. The information can enable the MnS consumer to evaluate the network resilience capability .

* * * End of Changes * * * *