3GPP TSG-SA5 Meeting #161	S5-252650

Fukuoka, Japan, 19 - 23 May 2025

Source:	Ericsson France S.A.S

Title:	Rel-19 pCR TS 28.561 Add attributes to NDTFunction and NDTJob and NDTReport

Document for:	Agreement

Agenda item:	6.19.5

Spec:	TS 28.561

Version:	0.2.0

Work Item:	NDT

Comments

This pCR adds attributes for IOC NDTFunction, NDTJob, NDTReport.

Proposed Changes

* * * First Change * * * *

#### 6.2.1.3	Class definition

##### 6.2.1.3.1	NDTFunction <<InformationObjectClass>>

###### 6.2.1.3.1.1	Definition

This IOC represents the properties of an NDT Function.

The attribute "supportedNDTCapabilities" indicates the types of modelling and analysis that are supportedby the NDT instance.

###### 6.2.1.3.1.2	Attributes

The NDTFunction IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedNDTCapabilities | M | T | F | T | T |
| Attribute related roles |  |  |  |  |  |

###### 6.2.1.3.1.3	Attribute constraints

None.

###### 6.2.1.3.1.4	Notifications

None.

##### 6.2.1.3.2	NDTJob <<InformationObjectClass>>

###### 6.2.1.3.2.1	Definition

This IOC represents the properties of an NDT job created by an MnS consumer.

The attribute "nDTJobType" indicates the type of modelling and analysis requested by the MnS consumer.

The attribute "nDTNetworkScope" indicates the scope of mobile network to be modelled by the NDT job.

The attribute "nDTNetworkConfigurations" indicates the network configurations that are defined by MnS consumer to be considered during the modelling and analysis process.

###### 6.2.1.3.2.2	Attributes

The NDTJob IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTJobType | M | T | T | T | T |
| nDTNetworkScope | M | T | T | F | T |
| nDTNetworkConfigurations | M | T | T | F | T |
| Attribute related roles |  |  |  |  |  |
| nDTFunctionRef | M | T | T | F | T |
| nDTReportRefList | M | T | T | F | T |

###### 6.2.1.3.1.3	Attribute constraints

None.

###### 6.2.1.3.1.4	Notifications

None.

##### 6.2.1.3.3	NDTReport <<InformationObjectClass>>

###### 6.2.1.3.3.1	Definition

This IOC represents the properties of an NDT report corresponding to an NDT job.

The attribute "nDTExecutionReport" contains performance metrics and/or alarm results that are collected by NDT after the network behaviour is modelled.

###### 6.2.1.3.3.2	Attributes

The NDTReport IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTExecutionReport | M | T | T | F | T |
| Attribute related roles |  |  |  |  |  |
| nDTJobRef | M | T | T | F | T |
| nDTFunctionRef | M | T | T | F | T |

###### 6.2.1.3.1.3	Attribute constraints

None.

###### 6.2.1.3.1.4	Notifications

None.

## 6.3	Attribute definitions

### 6.3.1	Attribute properties

Table 6.3.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| nDTJobRef | It indicates an DN of a NDTJob Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nDTFunctionRef | It indicates an DN of a NDTFunction Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nDTReportRefList | It indicates a list of DN of NDTReport Instances. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTExecutionReport | It indicates performance metrics and/or alarm results that are reported by NDT after the execution of the NDT job | type: string multiplicity: * isOrdered: False isUnique: False defaultValue: None isNullable: False |
| supportedNDTCapabilities | It indicates the different types of modelling and analysis that are supported by the NDT instance. allowedValues: NETWORK_AUTOMATION, VERIFICATION, DATA_GENERATION The list of allowed values can be extended with vendor-specific values | type: ENUM multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTJobType | It indicates the NDT job type requested by MnS consumer. allowedValues: NETWORK_AUTOMATION, VERIFICATION, DATA_GENERATION The list of allowed values can be extended with vendor-specific values | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nDTNetworkScope | It indicates the scope of mobile network to be modelled by the NDT job. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nDTNetworkConfigurations | It indicates the network configurations that are defined by MnS consumer to be considered during the modelling and analysis process. | type: string multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

* * * End of Changes * * * *