3GPP TSG-SA5 Meeting #161	S5-252708

Fukuoka, Japan, 19 - 23 May 2025

# Source:	Ericsson

# Title:	Rel-19 pCR TS 28.572 Plan Configuration Examples

# Document for:	Approval

Agenda Item:	6.19.9.1 - Management of planned configurations

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]		3GPP TS 28.572: "Management of planned configurations"

# 3	Rationale

Enhance the examples.

# 4	Detailed proposal

The following changes are proposed for TS 28.572[1].

Begin of modifications

### 7.1.2	Information Elements

The planConfigContentType specifies the type of the content in the 'planConfig'.  This is required so that the MnS Producer may correctly interpret the provided plan configuration data.  The configurationContentType application/3gpp-yang-patch shall be one such supported content type.  This type is based on the yang patch specification as per RFC 8072 but with the relaxation of the specification for the following properties as defined in RFC-8072:

'target' : As per section 2.4 of RFC-8072 but with the relaxation of the definition of the target to allow for the omission of the module prefix (e.g. it is possible to specify a target like '/_3gpp-common-subnetwork:SubNetwork=Ireland /_3gpp-common-mecontext:MeContext=Dublin-1/_3gpp-common-managed-element:ManagedElement=Dublin-1/_3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1' or in the relaxed form '/SubNetwork=Ireland/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1').

'value' : Encoded according to RFC-7951 but with the relaxation of the definition of the value to allow for the omission of the module prefix.  The MnS Producer shall accept this relaxed form of the value if it determines the overall value is unambiguous.

Next modification

## A.1	RESTful HTTP-based solution set

Resource structure

The resource structure on the MnS producer is as follows:

…/{MnSName}/{/MnSVersion}/plan-descriptors/{id}

…/{MnSName}/{/MnSVersion}/plan-descriptors/{id}/plan-config

…/{MnSName}/{/MnSVersion}/plan-group-descriptors/{id}

…/{MnSName}/{/MnSVersion}/plan-validation-jobs/{id}

…/{MnSName}/{/MnSVersion}/plan-validation-jobs/{id}/status

…/{MnSName}/{/MnSVersion}/plan-validation-jobs/{id}/validation-details

…/{MnSName}/{/MnSVersion}/plan-activation-jobs/{id}

…/{MnSName}/{/MnSVersion}/plan-activation-jobs/{id}/status

…/{MnSName}/{/MnSVersion}/plan-activation-jobs/{id}/activation-details

Creating, reading, updating and deleting planned configurations

The data node tree on the MnS producer is as follows upon system start up.

{ "plan-descriptors": {}, "plan-group-descriptors": {}, "fallback-config-descriptors": {}, "trigger-condition-descriptors": {}, "plan-validation-jobs": {}, "plan-activation-jobs": {} }

### A.1.1  Plan Descriptor Examples

A new item of the collection resource "plan-descriptors" is created by MnS consumers using HTTP POST.

POST 3gpp/ProvMnS/v1/plan-descriptors HTTP/1.1 Host: example.org Content-Type: application/json { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "planConfigContentType": "application/vnd.3gpp.json-patch+json", "currentConfigAddress": "http://example.org/3gpp/ProvMnS/v1", "applyMode": "ATOMIC", "planConfig": [ { "key": "op1", "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] }

The MnS producer allocates the identifier "p1" for the new resource and returns the response. The location header contains the URI of the new resource. The response body contains the representation of the new resource which is equal to the representation received in the request with the "lastModifiedAt" and "validationState" properties added.

HTTP/1.1 201 Created Date: Wed, 21 Aug 2024 15:39:57 GMT Location: http://example.org/3gpp/ProvMnS/v1/plan-descriptors/p1 Content-Type: application/json { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "planConfigContentType": "application/vnd.3gpp.json-patch+json", "currentConfigAddress": "http://example.org/3gpp/ProvMnS/v1", "applyMode": "ATOMIC", "lastModifiedAt": " 2025-03-06T16:50:26-08:00 ", "validationState": "NOT_VALIDATED", "planConfig": [ { "key": "op1", "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] }

The resource structure on the MnS producer contains the new resource.

{ "plan-descriptors": { "p1": { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "planConfigContentType": "application/vnd.3gpp.json-patch+json", "currentConfigAddress": "http://example.org/3gpp/ProvMnS/v1", "applyMode": "ATOMIC", "lastModifiedAt": " 2025-03-06T16:50:26-08:00 ", "validationState": "NOT_VALIDATED", "planConfig": [ { "key": "op1", "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] } }, "plan-group-descriptors": {}, "fallback-config-descriptors": {}, "trigger-condition-descriptors": {}, "plan-validation-jobs": {}, "plan-activation-jobs": {} }

The next example shows how the value of the "location" attribute in the operation can be changed from "Castle Charlottenburg" to "Summer palace Charlottenburg".

PATCH 3gpp/ProvMnS/v1/plan-descriptors/p1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "replace", "path": "/planConfig/0/value/attributes/location", "value": "Summer palace Charlottenburg" } ]

To delete the planned configuration "p1" a MnS consumer might send.

DELETE 3gpp/ProvMnS/v1/plan-descriptors/p1 HTTP/1.1 Host: example.org

In case of success, the MnS producer returns the following message.

HTTP/1.1 204 No Content Date: Wed, 21 Aug 2024 16:12:45 GMT

The next example shows how a planned configuration can be created for the case where the current configuration data node tree is specified with YANG.

POST 3gpp/ProvMnS/v1/plan-descriptors HTTP/1.1 Host: example.org Content-Type: application/json Accept : application/json,application/problem+json { "name" : "planxyz", "applyMode" : "BEST_EFFORT", # Execute in a best effort manner trying all operations "configurationContentType" : "application/3gpp-yang-patch+json" "planConfig": { "patchId" : "NRCellDU", "edit" : [ { "operation": "create", "editId" : "opId-001", "target": "/_3gpp-common-subnetwork:SubNetwork=Irl/3gpp-common-mecontext:MeContext=Dublin-1/3gpp-common-managed-element:ManagedElement=Dublin-1/3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1", "value": { "NRCellDU": [ { "id": "1", "attributes": { "administrativeState" : "LOCKED", ... } } } }, { "operation": "merge", "editId" : "opId-002", "target": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1/NRCellDU=2", "value": { "attributes": { "administrativeState": "LOCKED", } } }, { "operation": "remove", "editId" : "opId-003", "target": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1/NRCellDU=3" } ] } } }

To get the planConfig :

GET 3gpp/ProvMnS/v1/plan-descriptors/p1/plan-config HTTP/1.1 Host: example.org Content-Type: application/json

To get a specific operation/edit in a planConfig :

GET 3gpp/ProvMnS/v1/plan-descriptors/p1/plan-config/editId-001 HTTP/1.1 Host: example.org Content-Type: application/json

The next example shows how the planConfig edit / operation with editId=opId-001 can be modified to change the value of the "administrativeState" attribute from "LOCKED" to "UNLOCKED".  The 'editId' may be used to uniquely identify the specific configuration operation/edit to be modified in the planConfig.

PATCH 3gpp/ProvMnS/v1/plan-descriptors/p1/planConfig/opId-002 HTTP/1.1 Host: example.org Content-Type: application/json { "value": { "attributes": { "administrativeState": "LOCKED", } } }

To create a new edit entry to the end of the edits :

PUT 3gpp/ProvMnS/v1/plan-descriptors/p1/planConfig/opId-004 HTTP/1.1 Host: example.org Content-Type: application/json { "operation": "merge", "editId" : "opId-004", "target": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1/NRCellDU=4", "value": { "attributes": { "administrativeState": "UNLOCKED", } } }

To create a new edit entry before/after an existing edit use 'before' or 'after' parameter options:

PUT 3gpp/ProvMnS/v1/plan-descriptors/p1/planConfig/opId-004?after=opId-001 HTTP/1.1 Host: example.org Content-Type: application/json { "operation": "merge", "editId" : "opId-004", "target": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1/NRCellDU=4", "value": { "attributes": { "administrativeState": "UNLOCKED", } } }

### A.1.1  Plan Activation Job Examples

This section shows examples for plan activation job related resources.  This example uses the 'inline' form of the plan descriptor (planDescriptor) :

# Create plan-activation-job POST {apiRoot}/ProvMnS/1900/plan-activation-jobs HTTP/1.1 Host: example.org Content-Type: application/json Accept : application/json,application/problem+json { "name" : "job-xyz", "description" : "optimize the Dublin area network", "isFallbackEnabled" : true, "planConfigDescrId" : "planxyz" } Response : { "id": "myjob-111", "name" : "job-xyz", "description" : "optimize the Dublin area network", "isFallbackEnabled" : true, "planConfigDescrId" : "planxyz", "jobState" : "RUNNING", "startedAt" : "<some-time>", "activationState" : "NOT_STARTED", "activationDetails": { "href" : "{apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/activation-details" }, "cancelRequest" : false, "links" : { "self" : { "href" : "{apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111"}, "planDescriptor" : "{apiRoot}/ProvMnS/1900/…/planxyz"}, "status": { "href" : "{apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/status"}, "fallback": { "href" : "…"} } } # GET plan-activation-job status (job COMPLETE). Report all the relevant status information related to the plan activation job GET {apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/status { "jobState": "COMPLETE", "activationState": "ACTIVATED", "startedAt": "2024-12-02T13:16:54.088Z", "stoppedAt": "2024-12-02T13:16:58.088Z" } # GET plan-activation-job activation-details # - jobState=COMPLETED / activationState=ACTIVATED # - All configuration edits/operations are successfully activated # - no detailed info on failed edits/operations are reported (none failed) GET {apiRoot}/ProvMnS /1900/plan-activation-jobs/myjob-111/activation-details { "summary" : { "total": 3, "succeeded": 3, "failed": 0 } } # GET plan-activation-job activation-details with verbose option (expand=all) # - jobState=COMPLETED / activationState=ACTIVATED # - All configuration edits/operations are successfully activated # - no detailed info on failed edits/operations are reported (none failed) GET {apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/activation-details?expand=all { "editStatus" : { "patchId" : "patch-001", "edit" : [ { "editId" : "opId-001", "state" : "SUCCEEDED" }, { "editId" : "opId-002", "state" : "SUCCEEDED" }, { "editId" : "opId-003", "state" : "SUCCEEDED" } ] }, "summary" : { "total": 3, "succeeded": 3, "failed": 0 } } # GET plan-activation-job result-details with complete configuration edit/operation failure # - jobState=COMPLETED / activationState=ACTIVATION_FAILED # - All configuration edits/operations failed to activate # - Activation details on failed edits/operations are reported GET {apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/activation-details { "editStatus" : { "patchId" : "patch-001", "edit" : [ { "editId" : "opId-001", "state" : "FAILED", "errors" : { "error" : [ { "type": "application", "reason": "data-exists", "path": "/_3gpp-common-subnetwork:SubNetwork=Irl/_3gpp-common-mecontext:MeContext=Dublin-1/_3gpp-common-managed-element:ManagedElement=Dublin-1/_3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1/_3gpp-nr-nrm-nrcelldu:NRCellDU=1", "message": "Data already exists; cannot be created" } ] } }, { "editId" : "opId-002", "state" : "FAILED", "errors" : { "error" : [ { "type": "application", "reason": "data-exists", "path": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1/NRCellDU=2", "message": "Data already exists; cannot be created" } ] } }, { "editId" : "opId-003", "state" : "FAILED", "errors" : { "error" : [ { "type": "application", "reason": "data-exists", "path": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1/NRCellDU=3", "message": "Data already exists; cannot be created" } ] } } ] }, "summary" : { "total": 3, "succeeded": 0, "failed": 3 } } # GET plan-activation-job result-details with operation/edit failure (with expand=all option) # - jobState=COMPLETED / activationState=PARTIALLY_ACTIVATED # - Some configuration edits/operations failed to activate # - Activation details on failed edits/operations are reported # - Activation details on successful edits/operations are reported (expand=all) GET {apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/activation-details?expand=all { "editStatus" : { "patchId" : "patch-001", "edit" : [ { "editId" : "opId-001", "state" : "FAILED", "errors" : { "error" : [ { "type": "application", "reason": "data-exists", "path": "/_3gpp-common-subnetwork:SubNetwork=Irl/_3gpp-common-mecontext:MeContext=Dublin-1/_3gpp-common-managed-element:ManagedElement=Dublin-1/3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1/_3gpp-nr-nrm-nrcelldu:NRCellDU=1", "message": "Data already exists; cannot be created" } ] } }, { "editId" : "opId-002", "state" : "SUCCEEDED" }, { "editId" : "opId-003", "state" : "SUCCEEDED" } ] }, "summary" : { "total": 3, "succeeded": 2, "failed": 1 } }

End of modifications