3GPP TSG-SA5 Meeting #161	S5-252856

Fukuoka, Japan, 19 - 23 May 2025	revision of S5-252367

Source:	Nokia

Title:	Pseudo-CR on CCL-impact assessment, feedback and resolution

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.2.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add use case and requirements for the use on CCL-impact assessment and resolution as was agreed in the CCL study in TR28. 867

Proposed Changes

* * * First Change * * * *

# 5	Management Capabilities

## 5.3		CCL Performance Monitoring - CCLPERF

### 5.3.1		Description

TBD

### 5.3.2	Use Cases

#### 5.3.2.1	Performance Evaluation of a Closed Control Loop – CCLPERF_01

The advanced monitoring functionalities of a CCL can provide real-time insights into the performance and outcomes of a CCL. The monitoring activity for a Closed Control Loop may result in further actions that happen in the operation phase, e.g., evaluate and update, in order to change the closed control loop settings and improve its performance. So, there is a need to evaluate the performance of a Closed Control Loop itself. Such metrics are important to understand and change a CCL's behaviour and to improve its performance to pursue the assigned goal(s).

For example, certain performance aspects of a CCL can be very crucial to know in order to evaluate and decide upon a CCL's performance, such as the number of breached goals, time taken to meet a breached goal, number of conflicts occurred by a CCL etc. With the knowledge of such performance aspects of an existing CCL, a MnS consumer can more effectively update or create a new CCL.

An operator can also compare different CCLs based on these performances and choose the best one for its network deployment.

#### 5.3.2.X	MnS Consumer’s feedback on CCL actions – CCLPERF_02

A CCL should derive its actions without the involvement of any other entity (such as the managed network object) but the actions can have different levels of satisfaction for the different MnS consumers. The MnS consumers should be able to provide feedback to the CCL indicating how satisfied the MnS consumer is with the quality of the CCL actions, which should enable the CCL to fine-tune and optimize its decisions.

EXAMPLE:	The MnS consumer feedback may grade the usefulness of the executed action on a fixed scale say from 0 (indicating a terrible and never to be re-used action) to 10 (indicating a very good action for the interests of the MnS consumer). Other criteria may be added, e.g., to address the case that two consumer experience the same outcomes but may have different grade for feedback,

To be able to gauge the satisfaction, the MnS consumer should be able to receive information about the provisioning operations executed by the CCL. This information includes operation performed, MOIs updated, etc. The CCL does not break its execution when it provides information to the MnS consumer or to wait for feedback from the MnS consumer. The feedback from an MnS consumer does not break the loop.

It may be needed to determine what impact the CCLs’ action(s) had on a given scope that is the responsibility of other CCLs. Based on the CCL actions and the resulting impact on PMs,  it may be determined that new actions are needed to undo the degradation and to avoid it in future.

Based on some local policies or due to degradations observed, the consumer may prefer that a particular NF is not updated as part of the Execution step of CCL. The consumer should be enabled to request the CCL to revoke the changes made to a NF. Consumer may also update the CCL to ensure that a particular NF is never updated in future.

#### 5.3.2.Y	Assessment and resolution of CCL Impact on unknown impact-scope - CCLPERF_03

For some CCLs, the impact-scope affected by the actions of a CCL A may not be known a priori. For example, when a CCL A adjusts transmit power (e.g. to minimize interference), the neighbour cells and related CCLs acting on those cells that would be affected by any transmit power decrease or increase cannot be explicitly enumerated. Any negative effects cannot be easily anticipated, and most may not be easy to resolve by if-then-else rules. Instead, the affected CCLs should report their observed negative or positive impacts to CCL A to determine how to resolve the impact or avoid them in future.

'Related CCLs need to be notified that CCL A has executed an action and the impact-time of the action, i.e. the maximum time within which the action is expected to have impact and at which an observed impacts should be reported. For example, the impact of load balancing is visible in a few seconds while the impact of a handover decision can take several minutes. After the notified impact time, the impacted CCLs need to report  the impact that CCL A had to their performance metrics . The impact may be reported an index say in the range [0,10] where 0 implies an unacceptable action and 10 implies a good action. CCL A can then derive an appropriate remediation, e.g. by reconfiguring the candidate actions of the acting CCL (i.e. CCL A) or by undoing the action.

### 5.3.3		Requirements

Table 5.3.3-1

| Requirement label | Description | Related use case(s)/ Motivation |
| --- | --- | --- |
| REQ-CCLPERF_01-01 | The 3GPP management system should be able to obtain a CCL's performance with respect to the total number of occurrences of a goal breach. | UC-CCLPERF_01 Clause 5.3.2.1 |
| REQ-CCLPERF_01-02 | The 3GPP management system should be able to obtain a CCL's performance with respect to the time taken by CCL to meet a breached goal. | UC-CCLPERF_01 Clause 5.3.2.1 |
| REQ-CCLPERF_01-03 | The 3GPP management system should be able to obtain a CCL's performance with respect to the total number of conflicts occurred by a CCL | UC-CCLPERF_01 Clause 5.3.2.1 |
| REQ-CCLPERF_02-01 | The 3GPP management system should enable MnS consumer to provide its feedback on the action(s) taken by CCL. | UC-CCLPERF_01 Clause 5.B.2.1 |
| REQ- CCLPERF_02-02 | The 3GPP management system should enable MnS consumer to request for revocation of the action(s) taken by the CCL. | UC-CCLPERF_01 Clause 5.B.2.1 |
| REQ-CCLPERF_02-03 | The 3GPP management system should have a capability enabling the MnS consumer to receive information (e.g. operation performed, MOIs updated) about the action(s) taken by a CCL A. | UC-CCLPERF_01 Clause 5.B.2.1 UC-CCLPERF_02 Clause 5.B.2.2 |
| REQ-CCLPERF_03-01 | The 3GPP management system should support a capability enabling an MnS consumer to receive a report containing an executed action and the impact that the action had to a particular impact-scope. | UC-CCLPERF_01 Clause 5.B.2.1 UC-CCLPERF_03 Clause 5.B.2.2 |
| REQ-CCLPERF_03-02 | The 3GPP management system should support a capability enabling an MnS consumer to propose to a CCL a remediation against the noted impact of a CCLs’ actions, e.g. the reconfiguration of the candidate actions of the CCL. | UC-CCLPERF_03 Clause 5.B.2.2 |

* * * End of Changes * * * *