3GPP TSG-SA5 Meeting #161	S5-252878

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0346 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.312 Clarification on Usage of Intent Utility Function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2025-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - First change: S5-251921 Stage 2 for utility function is agreed, in which the IntentUtilityFormula IOC is name contained by the IntentHandlingFunction and assotiatied with the Intent IOC. The assotiation relationship is 1 to *, which indicates 2 potential scenarios, i.e., the IntentUtilityFormula can be a part of the intent, or for mutiple intents. This should be reflected on the stage 1 as well. - Second change: It’s strange that the report includes a request for the consumer. Propose to change it to “an indication”. Besides, there is no conflict between the proposed change and the agreed CR S5-251915 in last meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification on the scenario of IntentUnitilityFUnction |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incosistent definition will be misleading.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.8, 5.3.6, 5.3.9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No Impact on stage 3. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-252878 is the revision of S5-252404 |  |  |  |  |  |  |  |  |

Start of First change

5.3.8	Intent Utility Function

5.3.8.1	Introduction

In some cases, it may not be obvious to the MnS producer how to select from multiple candidate solutions for fulfilling an intent.  For example, an MnS consumer expressing multiple intents may want to provide further preference information to MnS producer to improve overall fulfilment, rather than rely on simple satisfied/unsatisfied.

Extra information provided in an intent can indicate preferences between expectation targets and assist the MnS producer in balancing between acceptable solutions.

This is especially relevant when no solution fully satisfies all intents or meets the expectations of a particular intent. In such situations, balancing trade-offs among different intents becomes necessary.

For example, a network performance function could be used to quantify network performance relative to latency. providing a means for MnS Consumer “to quantify an intent’s relative value as higher when the latency is lower”:

Example 1:  U1 = f(throughput, latency, packet loss);  a single utility function defined for an intent

Example 2:  U2 = f(latency) and U3=f(throughput); multiple utility functions defined for an intent, e.g. for different expectations

An MnS consumer could express this in the form of an Intent Utility Function along with their expected target value.  This assists the MnS producer(s) in fulfilling an MnS consumer’s intents as it provides an indicator to the MnS Producer of the satisfaction level of the intent fulfilment. As the description above, the intent utility function can be for multiple intents or as a part of an intent.

Utility function can be used for intent negotiation.

5.3.8.2	Definition

Intent Utility Function defines a method by which consumers can express the relative value of an intent's expectations to assist the IDMS producer(s) in fulfilling their intents in the most acceptable manner.

Intent utility functions are mathematical expressions that quantify the satisfaction or utility derived from the various levels of fulfilment. The basic components of which include:

-	variables: to quantify specific aspects of the fulfilment, e.g. network performance.

-	weights: to define the relative importance of each variable, e.g. for network performance a variable representing low latency might be assigned higher weight than throughput.

-	function: the mathematical functions to be applied to the variables, e.g. linear, logarithmic, polynomials, scalars.

-	result: the output of the function. The value of which represents the utility level achieved, i.e. the satisfaction of the current fulfilment based on the acceptability of potential solutions defined by the consumer.

MnS producer(s) can use Intent Utility Function information to assess the acceptability of potential solutions, in addition to information such as resource availability and performance targets. Utility functions may be defined by the consumer and provided as part of the intent itself, i.e. the function is defined as part of the intent. A MnS consumer may also specify that an existing (i.e. predefined) utility function is used for one or multiple intent(s). Predefined utility functions may be vendor specified (e.g. system pre-installed) and/or specified by the MnS consumer.

A single intent may contain multiple intent expectations each having its own utility function. In such cases, aggregation of these utility functions can be used to represent the overall utility of the intent. Normalization may also be needed when dealig with Utility Functions expressed in different scales (e.g., latency in milliseconds, throughput in Mbps, energy consumption in Joules).

Figure 5.3.8.2-1 represents a case where MnS Consumer expresses an intent (A) with expectation which should be less than a while expectation of another intent (B) should be greater than b.  There may be no solution to satisfy both expectations. There may however be different solutions available for each intent with differing levels of fulfilment as indicated by the black and red circles. While both solutions would satisfy intent A neither would satisfy intent B and there is limited ability for the MnS producer to decide which would be better from MnS consumer perspective.  From MnS consumer perspective such a case is considered “undifferentiated fulfilment” as the MnS producer has no further guidance available to assist in selecting their preferred solution.

Figure 5.3.8.2-1: Intents with undifferentiated fulfilment

Figure 5.3.8.2-2 represents a case where MnS Consumer defines Utility Functions A and B.  Each provide the MnS producer with a means to calculate utility value based on the deviation between the achieved and target expectation target values for different solutions.  These utility results can be used by MnS producer to select from solutions to improve the overall utility across multiple intents.  For example, the solution with the red circle yields a better outcome since it can fully satisfy A and better satisfies B than the black solution.  From MnS consumer perspective such a case is considered “quantitative fulfilment” as the MnS producer has a means by which to apply a quanitiative value to each solution, and use such information to better select on behalf of the consumer.

Figure 5.3.8.2-2: Intents with quantitative fulfilment (via Utility Functions)

Figure 5.3.8.2-3 represents a case where no solutions can satisfy any of the intents.  Although neither can be fulfilled the MnS producer may choose to prioritize the red circle solution as its utility increases exponentially for Intent B.  This would likely contribute to a better overall utility level across multiple intents.

Figure 5.3.8.2-3: Intents with quanititative fulfilment (via Utility Functions)

5.3.8.3	Requirements

REQ-Intent_Util-01: The intent driven MnS producer should have the capability to advertise its support for allowing MnS Consumers to express utility functions.

REQ-Intent_Util-02: The intent driven MnS producer should have the capability to advertise the mechanisms (e.g. data and format) by which an MnS Consumer can express utility functions.

REQ-Intent_Util-03: The intent driven MnS producer should have the capability to report potential outcomes including results for utility functions when applicable.

REQ-Intent_Util-04: The intent driven MnS producer should allow an authorized MnS consumer to specify utility functions as part of an intent.

REQ-Intent_Util-05: The intent driven MnS producer should allow an authorized MnS consumer to reference utility functions defined external to the intent.

End of First change

Start of Second change

5.3.9.5	MnS consumer advises on preferred alternatives

The MnS consumer wants an intent fulfilled. The intent is feasible, but the MnS producer has multiple alternatives related to fulfilling the intent. The MnS producer wants the MnS consumer to advise on their (the MnS consumer's) preference among these alternatives.

NOTE 1:	An alternative is the combination a set of expectation target values that the MnS producer can achieve together with their (expected) impacts on the network (objects). E.g. for an expectation target on energy consumption, the impact may include coverage area as a result of the energy consumption.

Figure 5.3.9.5-1: MnS consumer advises the MnS producer
on the preferences among alternatives at the MnS producer

After the MnS consumer creates an intent to be fulfilled, the MnS producer determines that there are multiple alternatives, so the MnS producer and provides a report to the MnS consumer so that the MnS consumer may help chose the best alternative.

The report to the MnS consumer may include:

-	The list of available/fulfillable expectation target values that the MnS producer is able to apply/achieve.

-	The expected relative impacts of the different alternatives - on aspects of the submitted intent or their  intents and intent expectations.

-	An information requesting the MnS consumer to select one among the alternatives.

Given the alternatives, the MnS consumer takes any combination of these actions:

-	Chooses and indicates the preferred alternative.

-	Defines the relative importance of their expectation Targets (in the form of a utility function) so that the MnS producer may consider these in deciding upon the solution/ solution approach/ closed loops/ action/ outcome to be applied/deployed/achieved.

-	Provides the relative satisfaction level that each of the alternatives achieves. The satisfaction level is the computation of the utility achieved by each alternative according to the MnS consumer’s utility function. It is an integer in the range  [0,100] . The highest possible value indicates that the solution provided by the MnS producer achieves the best possible outcomes that the MnS consumer expected, e.g., that it achieves the highest range of a target whose desired values were defining as falling in a range. Correspondingly, the values lower than 100 indicate how far from the maximum satisfaction the outcome is.

End of Second change