3GPP TSG-SA5 Meeting #161	S5-252885

Fukuoka, Japan, 17 - 21 May 2025	revision of S5-252378

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0345 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.312 Update intent conflict reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-14 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS28.312 specifies that the conflictType may have one of the following values: INTENT_CONFLICT, EXPECTATION_CONFLICT or TARGET_CONFLICT. The intent report provides information about the observed conflicts with attribute constraints as in table 6.2.1.3.9.3, limiting information on conflciting intent only for INTENT_CONFLICTs and information on conflciting intentExpectations only for EXPECTATIONCONFLICTS With this, if there are two conflicting intents, the report can only refer to the conflicting intent’s Id, but not on what the conflict actually was (ie: which expectation, what target of the conflicting intent caused the conflict). It is proposed that informtion on conflictingExpectation and/or conflictingTarget should be present even if the conflict type is INTENT_CONFLICT. So that the report could contain the conflicting intent’s Id, the expectation within that intent and the target within that expectation, that caused the intent to conflict. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Attribute constraints to allow conflictingExpectation and/or conflictingTarget to be provided for INTENT_CONFLICTs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The intent report may provide inadequate information in case of a conflcit |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

##### 6.2.1.3.9	IntentConflictReport << dataType >>

###### 6.2.1.3.9.1	Definition

IntentConflictReport <<dataType>> represents the conflict information for the detected conflict.

When a conflict is detected, the MnS producer will configure the value of attributes of IntentConflictReport and notify the MnS consumer about the conflict, indicating the intent, intent expectation or expectation target which give rise to the conflict. The value of recommendedSolutions may be configured by MnS producer and notified to MnS consumer.

The IntentConflictReport includes the conflictingIntent for any any type of conflict that is observed. The IntentConflictReport includes the conflictingExpectation if the conflict is of type EXPECTATION_CONFLICT. The IntentConflictReport includes the conflictingExpectation and conflictingTarget if the conflict is of type TARGET_CONFLICT .

###### 6.2.1.3.9.2	Attributes

The IntentConflictReport includes the following attributes.

Table 6.2.1.3.9.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| conflictId | M | T | F | T | T |
| conflictType | M | T | F | F | T |
| conflictingIntent | CM | T | F | F | T |
| conflictingExpectation | CM | T | F | F | T |
| conflictingTarget | CM | T | F | F | T |
| recommendedSolutions | O | T | F | F | T |

###### 6.2.1.3.9.3	Attribute constraints

| Name | Definition |
| --- | --- |
| conflictingIntent | The MnS producer supports reporting on INTENT_CONFLICT or EXPECTATION_CONFLICT or TARGET_CONFLICT |
| conflictingExpectation | The MnS producer supports reporting on conflicts of type EXPECTATION_CONFLICT or TARGET_CONFLICT |
| conflictingTarget | The MnS producer supports reporting on conflicts of type TARGET_CONFLICT |

End of change