3GPP TSG-SA5 Meeting #161	S5-252949

Fukuoka, Japan, 19 - 23 May 2025

Source:	Nokia, Samsung

Title:	Pseudo-CR on Rel-19 pCR TS 28.579 Add use case and requirements for authorization of the external MnS consumers at the CCF

Document for:	Approval

Agenda item:	6.19.21.1

Spec:	3GPP TS 28.579

Version:	0.2.0

Work Item:	MExpo

Comments

This pCR proposes to add a use case “Authorization of the external MnS consumer to access the management service API” and the corresponding requirements to TS 28.579.

Proposed Changes

* * * First Change * * * *

### 5.2.X	Authorization of the external MnS consumer to access service APIs

#### 5.2.X.1 Definition

This usecase describes how the external MnS consumer can be configured at the CCF with the authorization information to enable the consumption of the service APIs at the AEF of MSED.  For the external MnS consumer, the authorization information provided by the CCF contains the following:

- The identity of the external MnS consumer as assigned by the CCF

- The access permissions granted to the external MnS consumers. The permissions will determine, for example, what service API(s), corresponding resources or operations that the external MnS consumer will have access to within the management system.

- The expiry date of the granted authorization.

Traceability: REQ-MEXPO-AUTH-01

* * * Next Change * * * *

## 5.3	Requirements

REQ-MEXPO-FUN-01: 3GPP management system should support the capability to expose management services to external MnS consumers using CAPIF.

REQ-MEXPO-REG-01: The exposure of management services using CAPIF shall have the capability to provide the required mappings between the management service-related information and the CAPIF-defined API provider enrolment related information to enable the registration of MSED to the CCF.

REQ-MEXPO-REG-02: The exposure of management services using CAPIF shall provide the capability to register MSED to the CCF.

REQ-MEXPO-REG-03: The exposure of management services using CAPIF shall provide the capability to deregister MSED from the CCF.

REQ-MEXPO-REG-04: The exposure of management services using CAPIF shall provide the capability to update the registration details of MSED at the CCF.

REQ-MEXPO-PUB-01: The exposure of management services using CAPIF shall have the capability to map the management service-related information into the service API related information to enable the publishing of management services to the CCF.

REQ-MEXPO-PUB-02: The exposure of management services using CAPIF shall have the capability to publish the service APIs to the CCF.

REQ-MEXPO-PUB-03: The exposure of management services using CAPIF shall have the capability to retrieve the published service APIs at the CCF.

REQ-MEXPO-PUB-04: The exposure of management services using CAPIF shall have the capability to update the published service APIs at the CCF.

REQ-MEXPO-PUB-05: The exposure of management services using CAPIF shall have the capability to unpublish the service APIs from the CCF.

REQ-MEXPO-AUTH-01: The exposure of management services using CAPIF shall provide the capability enabling the CCF to authorize the external MnS consumer to access the management service(s).

REQ-MEXPO-LOG-01: The exposure of management services using CAPIF shall support the capability to create logs based on the service API invocations by the external MnS consumers.

REQ-MEXPO-LOG-02: The exposure of management services using CAPIF shall support the capability to log the service API invocations to the CCF.

* * * End of Changes * * * *