3GPP TSG-SA5 Meeting #161 	S5-252975

Fukuoka City, Fukuoka, Japan, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0535 | rev | 1 | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.623 Corrections on MDT PLMN List |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2025-05-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on TS32.422, MDT PLMN List is required for Signalling based Logged/Immediate MDT. For instance, in subclause 4.1.2.17.4 of TS 32.422: “Handling of various scenarios for Signalling based Logged/Immediate MDT are addressed below: 1) Management System initiating MDT activation shall validate that PLMNs specified in the MDT PLMN List are supported by all the cells specified in the area scope If the gNB receives a request where none of the PLMNs in the MDT PLMN List match any PLMN in its list, it shall ignore the request. ” However, the PLMN List is missing in TraceJob for immediate MDT, which is not consistent with TS32.422 specfiications and many RAN3/RAN2 specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding plmnList for both immediate MDT and logged MDT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification, which may lead to incorrect implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forge |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1702 at commit a4a736f16f7d8f1638488d388bf95629df08f112 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE 1 ***

*** yang-models/_3gpp-common-trace.yang ***

<CODE BEGINS>

module _3gpp-common-trace {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-trace";

prefix "trace3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types {prefix types3gpp; }

import _3gpp-common-yang-extensions {prefix yext3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-files { prefix files3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Trace handling

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)" ;

revision 2025-05-06 { reference "CR-0535" ; }

revision 2025-02-06 { reference "CR-0503 CR-0507" ; }

revision 2025-02-05 { reference "CR-0460" ; }

revision 2024-11-05 { reference "CR-0470" ; }

revision 2024-10-06 { reference "CR-0459"; }

revision 2024-08-06 { reference "CR-0376 CR-0378 CR-0420"; }

revision 2024-05-12 { reference "CR-0401"; }

revision 2024-05-06 { reference CR-0359; }

revision 2024-04-06 { reference "CR-0342"; }

revision 2024-01-29 { reference "CR-0316"; }

revision 2023-11-06 { reference "CR-0290 CR-0294"; }

revision 2023-11-03 { reference CR-0302 ; }

revision 2023-09-18 { reference CR-0271 ; }

revision 2023-08-10 { reference CR-0261; }

revision 2023-04-26 { reference CR-0250; }

revision 2023-02-18 { reference "CR-0234"; }

revision 2023-02-16 { reference "CR-0233"; }

revision 2022-09-30 { reference CR-0191 ; }

revision 2022-04-27 { reference "CR-0159"; }

revision 2021-10-18 { reference "CR-0139"; }

revision 2021-07-22 { reference "CR-0137"; }

revision 2021-01-25 { reference "CR-0122"; }

revision 2020-11-16 { reference "CR-0117"; }

revision 2020-08-06 { reference "CR-0102"; }

feature FilesUnderTraceJob {

description "Files shall be contained under TraceJob";

}

grouping FreqInfoGrp {

description "Represents the FreqInfo dataType.

This <<dataType>> defines the RF reference frequency and the frequency

operating bands used in a cell for a given direction (UL or DL) in FDD

or for both UL and DL directions in TDD";

leaf arfcn {

type uint32 {

range 0..3279165;

}

mandatory true;

description "RF Reference Frequency as defined in TS 38.104,

clause 5.4.2.1. The frequency provided identifies the absolute

frequency position of the reference resource block (Common RB 0)

of the carrier. Its lowest subcarrier is also known as Point A.";

}

leaf-list freqBands {

type uint32 {

range 1..1024;

}

min-elements 1;

description "List of NR frequency operating bands. Primary NR

Operating Band as defined in TS 38.104, clause 5.4.2.3.

The value 1 corresponds to n1, value 2 corresponds to NR operating

band n2, etc.";

}

}

grouping AreaConfigGrp {

description "Represents the AreaConfig dataType.

This <<dataType>> defines the area for which measurement logging should

be performed. It is described by a list of cells and a list of

frequencies.";

list freqInfo {

key arfcn;

min-elements 1;

max-elements 32;

description "It specifies the carrier frequency and bands used in

a cell.";

uses FreqInfoGrp ;

}

leaf-list pciList {

type uint32 {

range 0..1007;

}

min-elements 1;

max-elements 32;

description "List of neighbour cells subject for MDT scope.";

}

}

grouping AreaScopeGrp {

description "Represents the AreaScope dataType.

This <<dataType>> defines an area scope";

choice AreaScopeChoice {

leaf-list eutraCellIdList {

type string;

min-elements 1;

max-elements 32;

description "List of E-UTRAN cells identified by E-UTRAN-CGI";

}

leaf-list nrCellIdList {

type string;

min-elements 1;

max-elements 32;

description "List of NR cells identified by NG-RAN CGI";

}

leaf-list tacList {

type types3gpp:Tac;

min-elements 1;

max-elements 8;

description "Tracking Area Code list";

}

list taiList {

description "Tracking Area Identity list";

key idx;

min-elements 1;

max-elements 8;

leaf idx { type string; }

uses types3gpp:TaiGrp;

}

}

list nPNIdentityList {

description "It defines which NPNs that can be served by the NR cell,

and which CAG IDs or NIDs can be supported by the NR cell for

corresponding PNI-NPN or SNPN in case of the cell is NPN-only cell.

(NPN-Identity referring to TS 38.331).";

key idx;

min-elements 1;

uses NpnIdGrp;

leaf idx { type string; }

}

}

grouping NpnIdGrp {

description "Represents the NpnId dataType.";

list plmnId {

key "mcc mnc";

description "It specifies the PLMN Id of the NPN network.";

uses types3gpp:PLMNId;

max-elements 1;

}

choice npnidChoice {

leaf-list cAGIdList {

type string;

max-elements 256;

description "It identifies a CAG list containing up to 256

CAG-identifiers per UE or up to 12 CAG-identifiers per cell,

see TS 38.331.

CAG ID is used to combine with PLMN ID to identify a PNI-NPN.

CAG ID is a hexadecimal range with size 32 bit.";

}

leaf-list nIDList {

type string;

max-elements 16;

description "It identifies a list of NIDs containing up to 16 NIDs,

see TS 38.331.

NID is used to combine with PLMN ID to identify an SNPN.

NID is a hexadecimal range with size 44 bit.";

}

}

}

grouping ExcessPacketDelayThresholdsGrp {

description "Represents the ExcessPacketDelayThresholds dataType.

This <<dataType>> defines a excess packet delay threshold information

to enable the calculation of the PDCP Excess Packet Delay in the

uplink in case of M6 uplink measurements are requested. The excess

packet delay threshold information is specified with the 5QI value

and excess packet delay threshold value.";

leaf fiveQIValue {

type uint8;

mandatory true;

description "It indicates 5QI value.";

}

leaf excessPacketDelayThresholdValue {

type decimal64 {

fraction-digits 2;

range 0.25|0.5|1|2|4|5|10|20|30|40|50|60|70|80|90|100|150|300|500 ;

}

mandatory true;

units milliseconds;

description "Value of excess packet delay threshold

for M6 UL measurement in milliseconds.";

}

}

grouping TraceReferenceGrp {

description "Represents the TraceReference dataType.

This <<dataType>> defines a globally unique identifier, which uniquely

identifies the Trace Session that is created by the TraceJob. It is

composed of the MCC, MNC (resulting in PLMN identifier) and the

trace identifier.";

uses types3gpp:PLMNId;  // mcc+mnc

leaf traceId {

type string;

mandatory true;

description "An identifier, which identifies the Trace

(together with MCC and MNC). This is a 3 byte Octet String.";

}

}

grouping MbsfnAreaGrp {

description "Represents the MbsfnArea dataType.

This <<dataType>> defines a MBSFN area. It is composed of the MBSFN Area

identifier and the carrier frequency (EARFCN).";

leaf mbsfnAreaId {

type uint32 {

range 1..max;

}

mandatory true;

description "MBSFN Area Identifier";

}

leaf earfcn{

type uint32 {

range 1..max;

}

mandatory true;

description "Carrier Frequency";

}

}

grouping TraceConfigGrp {

description "Defines the configuration parameters of TraceJob

which are specific for Trace or combined Trace and Immediate MDT.

The attribute listOfNeTypes specifies the network elements to be

traced. The optional attribute listOfInterfaces allows to specify

the individual interfaces of the network elements to be recorded.

The attribute traceDepth allows to configure the level of detail

of the information which shall be recorded.  For trace the reporting

is event based, where the triggering event is configured with

attribute triggeringEvents. For each triggering event the first and

last message (start/stop triggering event) to record  are specified.";

list listOfInterfaces {

key idx;

description "Specifies the interfaces that need to be traced in the given

ManagedEntityFunction.The attribute is applicable only for Trace. In

case this attribute is not used, it carries a null semantic.";

reference "Clause 5.5 of 3GPP TS 32.422 for additional details on the

allowed values.";

leaf idx { type uint32 ; }

leaf-list MSCServerInterfaces {

type enumeration {

enum A ;

enum Iu-CS ;

enum Mc ;

enum MAP-G ;

enum MAP-B ;

enum MAP-E ;

enum MAP-F ;

enum MAP-D ;

enum MAP-C ;

enum CAP ;

}

}

leaf-list MGWInterfaces {

type enumeration {

enum Mc ;

enum Nb-UP ;

enum Iu-UP ;

}

}

leaf-list RNCInterfaces {

type enumeration {

enum Iu-CS ;

enum Iu-PS ;

enum Iur ;

enum Iub ;

enum Uu ;

}

}

leaf-list SGSNInterfaces {

type enumeration {

enum Gb ;

enum Iu-PS ;

enum Gn ;

enum MAP-Gr ;

enum MAP-Gd ;

enum MAP-Gf ;

enum Ge ;

enum Gs ;

enum S6d ;

enum S4 ;

enum S3 ;

enum S13 ;

}

}

leaf-list GGSNInterfaces {

type enumeration {

enum Gn ;

enum Gi ;

enum Gmb ;

}

}

leaf-list S-CSCFInterfaces {

type enumeration {

enum Mw ;

enum Mg ;

enum Mr ;

enum Mi ;

}

}

leaf-list P-CSCFInterfaces {

type enumeration {

enum Gm ;

enum Mw ;

}

}

leaf-list I-CSCFInterfaces {

type enumeration {

enum Cx ;

enum Dx ;

enum Mg ;

enum Mw ;

}

}

leaf-list MRFCInterfaces {

type enumeration {

enum Mp ;

enum Mr ;

}

}

leaf-list MGCFInterfaces {

type enumeration {

enum Mg ;

enum Mj ;

enum Mn ;

}

}

leaf-list IBCFInterfaces {

type enumeration {

enum Ix ;

enum Mx ;

}

}

leaf-list E-CSCFInterfaces {

type enumeration {

enum Mw ;

enum Ml ;

enum Mm ;

enum Mi-Mg ;

}

}

leaf-list BGCFInterfaces {

type enumeration {

enum Mi ;

enum Mj ;

enum Mk ;

}

}

leaf-list ASInterfaces {

type enumeration {

enum Dh ;

enum Sh ;

enum ISC ;

enum Ut ;

}

}

leaf-list HSSInterfaces {

type enumeration {

enum MAP-C ;

enum MAP-D ;

enum Gc ;

enum Gr ;

enum Cx ;

enum S6d ;

enum S6a ;

enum Sh ;

}

}

leaf-list EIRInterfaces {

type enumeration {

enum MAP-F ;

enum S13 ;

enum MAP-Gf ;

}

}

leaf-list BM-SCInterfaces {

type enumeration {

enum Gmb ;

}

}

leaf-list MMEInterfaces {

type enumeration {

enum S1-MME ;

enum S3 ;

enum S6a ;

enum S10 ;

enum S11 ;

enum S13 ;

}

}

leaf-list SGWInterfaces {

type enumeration {

enum S4 ;

enum S5 ;

enum S8 ;

enum S11 ;

enum Gxc ;

}

}

leaf-list PDN_GWInterfaces {

type enumeration {

enum S2a ;

enum S2b ;

enum S2c ;

enum S5 ;

enum S6b ;

enum Gx ;

enum S8 ;

enum SGi ;

}

}

leaf-list eNBInterfaces {

type enumeration {

enum S1-MME ;

enum X2 ;

}

}

leaf-list en-gNBInterfaces {

type enumeration {

enum S1-MME ;

enum X2 ;

enum Uu ;

enum F1-C ;

enum E1 ;

}

}

leaf-list AMFInterfaces {

type enumeration {

enum N1 ;

enum N2 ;

enum N8 ;

enum N11 ;

enum N12 ;

enum N14 ;

enum N15 ;

enum N20 ;

enum N22 ;

enum N26 ;

}

}

leaf-list AUSFInterfaces {

type enumeration {

enum N12 ;

enum N13 ;

}

}

leaf-list NEFInterfaces {

type enumeration {

enum N29 ;

enum N30 ;

enum N33 ;

}

}

leaf-list NRFInterfaces {

type enumeration {

enum N27 ;

}

}

leaf-list NSSFInterfaces {

type enumeration {

enum N22 ;

enum N31 ;

}

}

leaf-list PCFInterfaces {

type enumeration {

enum N5 ;

enum N7 ;

enum N15 ;

}

}

leaf-list SMFInterfaces {

type enumeration {

enum N4 ;

enum N7 ;

enum N10 ;

enum N11 ;

enum S5-C ;

enum N38 ;

enum N16 ;

enum N16a ;

}

}

leaf-list SMSFInterfaces {

type enumeration {

enum N20 ;

enum N21 ;

}

}

leaf-list UDMInterfaces {

type enumeration {

enum N8 ;

enum N10 ;

enum N13 ;

enum N21 ;

}

}

leaf-list UPFInterfaces {

type enumeration {

enum N4 ;

}

}

leaf-list ng-eNBInterfaces {

type enumeration {

enum NG-C ;

enum Xn-C ;

enum Uu ;

}

}

leaf-list gNB-CU-CPInterfaces {

type enumeration {

enum NG-C ;

enum Xn-C ;

enum Uu ;

enum F1-C ;

enum E1 ;

enum X2-C ;

}

}

leaf-list gNB-CU-UPInterfaces {

type enumeration {

enum E1 ;

}

}

leaf-list gNB-DUInterfaces {

type enumeration {

enum F1-C ;

}

}

}

leaf-list listOfNeTypes {

type enumeration {

enum MSC_SERVER;

enum SGSN;

enum MGW;

enum GGSN;

enum RNC;

enum BM_SC;

enum MME;

enum SGW;

enum PGW;

enum ENB;

enum EN_GNB;

enum GNB_CU_CP;

enum GNB_CU_UP;

enum GNB_DU;

}

description "Specifies the network element types where the trace should

be activated. The attribute is applicable only for Trace with

Signalling Based Trace activation. ";

reference "Clause 5.4 of 3GPP TS 32.422.";

}

leaf traceDepth {

when '../../jobType = "TRACE_ONLY"'

+  ' or ../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration  {

enum MINIMUM;

enum MEDIUM;

enum MAXIMUM;

enum VENDORMINIMUM;

enum VENDORMEDIUM;

enum VENDORMAXIMUM;

}

default MAXIMUM;

description "It specifies the trace depth. The attribute is

applicable only for Trace.";

reference "Clause 5.3 of 3GPP TS 32.422.";

}

list triggeringEvents {

key idx;

description "It specifies the triggering event parameter of the trace

session. The attribute is applicable only for Trace.";

reference "Clause 5.1 of 3GPP TS 32.422";

leaf idx { type uint32; }

max-elements 1;

leaf-list MSC_SERVER {

type enumeration {

enum MO_MT_CALLS;

enum MO_MT_SMS;

enum LU_IMSIattach_IMSIdetach;

enum SS;

}

}

leaf-list SGSN {

type enumeration {

enum PDPcontext;

enum MO_MT_SMS;

enum RAU_GPRSattach_GPRSdetach;

enum MBMScontext;

}

}

leaf-list MGW {

type enumeration {

enum CONTEXT;

}

}

leaf-list GGSN {

type enumeration {

enum PDPcontext;

enum MBMScontext;

}

}

leaf-list IMS {

type enumeration {

enum SIPsession_StandaloenTransaction;

}

}

leaf-list BM_SC {

type enumeration {

enum MBMSactivation;

}

}

leaf-list MME {

type enumeration {

enum UEinitiatedPDNconnectivityRequest;

enum ServiceRequest;

enum InitialAttach_TAU_Detach;

enum UEInitiatedPDNdisconnection;

enum BearerActivationModificationDeletion;

enum Handover;

}

}

leaf-list SGW {

type enumeration {

enum PDNconnectionCreation;

enum PDNconnectionTermination;

enum BearerActivationModificationDeletion;

}

}

leaf-list PGW {

type enumeration {

enum PDNconnectionCreation;

enum PDNconnectionTermination;

enum BearerActivationModificationDeletion;

}

}

leaf-list AMF {

type enumeration {

enum Registration;

enum ServiceRequest;

enum Handover;

enum UEderegistration;

enum NetworkDeregistration;

enum UEMobilityFromEPC;

enum UEMobilityToEPC;

enum CovergedChargingService;

}

}

leaf-list SMF {

type enumeration {

enum PDUsessionEstablishment;

enum PDUsessionModification;

enum PDUsessionRelease;

enum PDUsessionUPactivationDeactivation;

enum MobilityBtw3gppAndN3gppTo5GC;

enum MobilityFromEpc;

enum MobilityBtwISMForVSMF;

enum CovergedChargingService;

enum OfflineOnlyCharging;

}

}

leaf-list PCF {

type enumeration {

enum AMpolicy;

enum SMpolicy;

enum Authorization;

enum BDTpolicy;

enum SpendingLimitControl;

}

}

leaf-list UPF {

type enumeration {

enum N4Session;

}

}

leaf-list AUSF {

type enumeration {

enum UEauthentication;

}

}

leaf-list NEF {

type enumeration {

enum EventExposure;

enum PFDmanagement;

enum ParameterProvisiong;

enum Trigger;

}

}

leaf-list NRF {

type enumeration {

enum NFmanagement;

enum NFdiscovery;

}

}

leaf-list NSSF {

type enumeration {

enum NSSelection;

enum NSSAI;

}

}

leaf-list SMSF {

type enumeration {

enum SMService;

}

}

leaf-list UDM {

type enumeration {

enum UEcontext;

enum SubscriberData;

enum UEauthentication;

enum EventExposure;

}

}

}

}

grouping ImmediateMdtConfigGrp {

description "Represents the ImmediateMdtConfig dataType.

This <<dataType>> defines the configuration parameters of

IOC TraceJob which are specific for Immediate MDT or combine

Trace and Immediate MDT.

The optional attribute positioningMethod allows to specify

the positioning methods to use.

The following attributes are conditional available based on the

measurements configured in listOfMeasurements:

-reportInterval (conditional for M1 in LTE or NR and M1/M2 in UMTS),

-reportAmount (conditional for M1/M2 in UMTS),

-reportAmountM1LTE (conditional for M1 in LTE),

-reportAmountM4LTE (conditional for M4 in LTE),

-reportAmountM5LTE (conditional for M5 in LTE),

-reportAmountM6LTE (conditional for M6 in LTE),

-reportAmountM7LTE (conditional for M7 in LTE),

-reportAmountM1NR (conditional for M1 in NR),

-reportAmountM4NR (conditional for M4 in NR),

-reportAmountM5NR (conditional for M5 in NR),

-reportAmountM6NR (conditional for M6 in NR),

-reportAmountM7NR (conditional for M7 in NR),

-reportingTrigger (conditional for M1 in LTE or NR and M1/M2 in UMTS),

-eventThreshold (conditional for A2 event reporting or A2 event

triggered periodic reporting),

-collectionPeriodRRMNR (conditional for M4 and M5 in NR),

-collectionPeriodM6NR (conditional for M6 in NR),

-collectionPeriodM7NR (conditional for M7 in NR),

-collectionPeriodRRMLTE (conditional for M3 in LTE),

-measurementPeriodLTE (conditional for M4 and M5 in LTE),

-collectionPeriodM6LTE (conditional for M6 in LTE),

-collectionPeriodM7LTE (conditional for M7 in LTE),

-collectionPeriodRRMUMTS (conditional for M4 and M5 in UMTS),

-measurementPeriodUMTS (conditional for M6 and M7 in UMTS),

-measurementQuantity (conditional for 1F event reporting),

-beamLevelMeasurement (conditional for M1 in NR),

-excessPacketDelayThresholds (conditional for M6 UL measurement in NR).

For immediate MDT, the measurement reporting is dependent on the

configured measurements:

- For measurement M1 in LTE or NR, it is possible to select between

periodical, event triggered, event triggered periodic reporting or

reporting according to all configured RRM event triggers. For M1 and M2

measurement in UMTS, it is possible to select between periodical, event

triggered reporting or reporting according to all configured RRM event

triggers. Parameter reportingTrigger determines which of the reporting

methods is selected and in case of event triggered or event-triggered

periodic, which is the decisive event type. For periodical reporting,

parameters reportInterval and reportAmount determine the interval between

two successive reports and the number of reports. This means the

periodical reporting terminates after reportAmount reports have been

sent as long as reportAmount is configured with a value different from

infinity. For event-triggered periodic reporting, these two parameters

apply in addition to parameter eventThreshold which determines the

threshold of the event. In this case up to reportAmount reports are

sent with a periodicity of reportInterval after the entering condition

is fulfilled. The reporting is stopped, if the leaving condition is

fulfulled and is restarted if the configured event reoccurs. For event

based reporting, there is only one report sent after the event occurs.

The parameters to configure are reportingTrigger and eventThreshold.

In case of UMTS and 1f event reporting, additionally parameter

measurementQuantity is necessary in order to determine for which

measurement(s) the event threshold is applicable.  Parameter

beamLevelMeasurement determines whether beam level measurements shall

be included in case of NR.

- For measurement M2 in LTE or NR, reporting is according to RRM

configuration, see TS 38.321, TS 36.321 and TS 38.331, TS 36.331.

- For measurement M4 in UMTS, reporting is either according to RRM

configuration, see TS 25.321 and TS 25.331 or periodic or event

triggered periodic using parameter collectionPeriodRRMUMTS and

eventThresholdUphUMTS.

- For measurement M3 in UMTS, the reporting is done upon

availability, see TS 37.320.

- For measurements M4, M5, M6 and M7 in NR, for measurements

M3, M4, M5, M6 and M7 in LTE and for measurements M5, M6 and M7

in UMTS periodical reporting is applied. The configurable parameter

is the interval between two measurements (collectionPeriodRrmNr,

collectionPeriodM6NR, collectionPeriodM7NR, collectionPeriodRRMLTE,

measurementPeriodLte, collectionPeriodM6LTE, collectionPeriodM7LTE,

collectionPeriodRRMUMTS, measurementPeriodUMTS) and the number of

reports (reportAmountM4NR, reportAmountM5NR, reportAmountM6NR,

reportAmountM7NR, reportAmountM4LTE, reportAmountM5LTE,

reportAmountM6LTE, reportAmountM7LTE).  If no collection

period is configured for M5 in UMTS, all available measurements

ar elogged according to RRM configuration.

- Measurements M8 and M9 in NR or LTE are reported according to

configured M1 and/or M6 related UE measurement reporting.

";

leaf listOfMeasurements {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"';

type enumeration {

enum M1;

enum M2;

enum M3;

enum M4;

enum M5;

enum M6_DL;

enum M6_UL;

enum M7_DL;

enum M7_UL;

enum M1_EVENT_TRIGGERED;

enum M6;

enum M7;

enum M8;

enum M9;

}

description "It specifies the UE measurements that shall be

collected in an Immediate MDT job. The attribute is

applicable only for Immediate MDT.";

reference "Clause 5.10.3 of 3GPP TS 32.422.";

}

leaf reportingTrigger {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"';

type enumeration {

enum PERIODICAL;

enum A2_FOR_LTE;

enum 1F_FOR_UMTS;

enum 1I_FOR_UMTS_MCPS_TDD;

enum A2_TRIGGERED_PERIODIC_FOR_LTE;

enum ALL_CONFIGURED_RRM_FOR_LTE;

enum ALL_CONFIGURED_RRM_FOR_UMTS;

}

description "It specifies whether periodic or event based

measurements should be collected.

The attribute is applicable only for Immediate MDT and when the

listOfMeasurements is configured for M1 (for both UMTS and LTE)

or M2 (only for UMTS).";

reference "Clause 5.10.4 of 3GPP TS 32.422.";

}

leaf reportInterval {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' and ../reportingTrigger = "PERIODICAL"';

type uint32 {

range "120|240|250|480|500|640|1000|1024|2000|2048|3000|4000|"

+"5120|6000|8000|10240|12000|16000|20000|"

+"20480|24000|28000|32000|40960|60000|64000|"

+"360000|720000|1800000|3600000";

}

units milliseconds;

description "It specifies the interval between the periodical measurements

that shall be taken when the UE is in connected mode.

The attribute is applicable only for Immediate MDT and when

reportingTrigger is configured for periodical measurements.";

reference "Clause 5.10.5 of 3GPP TS 32.422.";

}

leaf reportAmount {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' and ../reportingTrigger = "PERIODICAL"';

type union {

type uint32 {

range "1|4|8|16|32|64" ;

}

type enumeration {

enum INFINITY;

}

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected.

The attribute is applicable only for Immediate MDT and when

reportingTrigger is configured for periodical measurements.";

reference "Clause 5.10.6 of 3GPP TS 32.422.";

}

leaf eventThreshold {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"';

type int64;

description "Specifies the threshold which should trigger the reporting

in case A2 event reporting in LTE or 1F/1l event in UMTS. The attribute

is applicable only for Immediate MDT and when reportingTrigger is

configured for A2 event in LTE or 1F event or 1l event in UMTS.";

reference "Clauses 5.10.7 and 5.10.7a of 3GPP TS 32.422.";

}

leaf collectionPeriodRRMNR {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1024|2048|5120|10240|60000";

}

units milliseconds;

description "Specifies the collection period for collecting RRM

configured measurement samples for M4, M5 in NR. The attribute is

applicable only for Immediate MDT.";

reference "Clause 5.10.30 of 3GPP TS 32.422.";

}

leaf collectionPeriodM6NR {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum 120ms;

enum 240ms;

enum 480ms;

enum 640ms;

enum 1024ms;

enum 2048ms;

enum 5120ms;

enum 10240ms;

enum 20480ms;

enum 40960ms;

enum 1min;

enum 6min;

enum 12min;

enum 30min;

}

description "It specifies the collection period for the Packet Delay

measurement (M6) for NR MDT taken by the gNB. The attribute is

applicable only for Immediate MDT.";

reference "clause 5.10.34 of  TS 32.422.";

}

leaf collectionPeriodM7NR {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1..60";

}

description "It specifies the collection period for the Packet Loss Rate

measurement (M7) for NR MDT taken by the gNB. The attribute is

applicable only for Immediate MDT. ";

reference "clause 5.10.35 of  TS 32.422.";

}

leaf collectionPeriodRRMLTE {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum 100ms;

enum 1000ms;

enum 1024ms;

enum 1280ms;

enum 2048ms;

enum 2560ms;

enum 5120ms;

enum 10000ms;

enum 10240ms;

enum 1min;

}

description "Specifies the collection period for collecting RRM configured

measurement samples for M3 in LTE. The attribute is applicable only

for Immediate MDT.";

reference "Clause 5.10.20 of 3GPP TS 32.422.";

}

leaf measurementPeriodLTE {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum 1024ms;

enum 2048ms;

enum 5120ms;

enum 10240ms;

enum 1min;

}

mandatory true;

description "It specifies the measurement period for the

Data Volume (M4) and Scheduled IP throughput measurements

(M5) for LTE MDT taken by the eNB. The attribute is

applicable only for Immediate MDT.";

reference "Clause 5.10.23 of 3GPP TS 32.422.";

}

leaf collectionPeriodM6LTE {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1024|2048|5120|10240";

}

units milliseconds;

description "Specifies the collection period for the Packet Delay

measurement (M6) for MDT taken by the eNB. The attribute is applicable

only for Immediate MDT.";

reference "Clause 5.10.32 of  TS 32.422.";

}

leaf collectionPeriodM7LTE {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint16 {

range 1..60 ;

}

description "It specifies the collection period for the Packet Loss Rate

measurement (M7) for LTE MDT taken by the eNB. The attribute is

applicable only for Immediate MDT.";

reference "Clause 5.10.33 of  TS 32.422.";

}

leaf eventThresholdUphUMTS {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint16 {

range 0..31 ;

}

description "It specifies the threshold which should trigger

the reporting in case of event-triggered periodic reporting for M4

(UE power headroom measurement) in UMTS.";

reference "Clause 5.10.39 of TS 32.422.";

}

leaf collectionPeriodRRMUMTS {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "100|250|500|1000|2000|"

+"3000|4000|6000";

}

units milliseconds;

description "Specifies the collection period for collecting RRM configured

measurement samples for M3, M4, M5 in UMTS. The attribute is applicable

only for Immediate MDT.";

reference "Clause 5.10.21 of 3GPP TS 32.422.";

}

leaf measurementPeriodUMTS {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  'or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1000|2000|3000|4000|6000|8000|12000|16000|20000|"

+"24000|28000|32000|64000";

}

units milliseconds;

description "It specifies the measurement period for the Data Volume and

Throughput measurements for MDT taken by RNC.

The attribute is applicable only for Immediate MDT.";

reference "Clause 5.10.22 of 3GPP TS 32.422.";

}

leaf measurementQuantity {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum CPICH_ECNO;

enum CPICH_RSCP;

enum PATHLOSS;

}

description "It specifies the measurements that are collected in an MDT

job for a UMTS MDT configured for event triggered reporting.";

reference "Clause 5.10.15 of 3GPP TS 32.422.";

}

leaf beamLevelMeasurement {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type boolean;

default false;

description "Indicates whether the NR M1 beam level measurements shall

be included or not.";

reference "Clause 5.10.40 of TS 32.422.";

}

leaf positioningMethod {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum GNSS;

enum E_CELL_ID;

}

description "It specifies what positioning method should be used in the

MDT job.";

reference "Clause 5.10.19 of 3GPP TS 32.422.";

}

list excessPacketDelayThresholds {

description "Excess packet delay thresholds info for M6 UL measurement.";

min-elements 1;

key idx;

leaf idx { type string; }

uses ExcessPacketDelayThresholdsGrp;

}

leaf reportAmountM1LTE {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for LTE.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM4LTE {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for LTE.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM5LTE {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for LTE.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM6LTE {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for LTE.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM7LTE {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for LTE.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM1NR {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for NR.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM4NR {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for NR.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM5NR {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for NR.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM6NR {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for NR.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM7NR {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for NR.";

reference "Clause 5.10.6 of TS 32.422.";

}

}

grouping LoggedMdtConfigGrp {

description "This <<dataType>> defines the configuration parameters of

IOC TraceJob which are specific for Logged MDT or Logged MBSFN MDT.

Based on the value configured for attribute jobType in IOC TraceJob,

different attributes are available. In case of LOGGED_MDT_ONLY, the

attributes reportType, eventListForEventTriggeredMeasurement,

eventThresholdL1, hysteresisL1, timeToTriggerL1,

areaConfigurationForNeighCells

and npnIdentityList are applicable. In case of LOGGED_MBSFN_MDT,

the attribute mbsfnAreaList is applicable.

The optional attribute plmnList allows to specify the PLMNs where

measurement collection, status indication and log reporting is allowed,

the optional attribute areaConfigurationForNeighCell allows to specify

the area for which UE is requested to perform measurements logging for

neighbour cells which have list of frequencies.  For logged MDT in UMTS

and LTE, the reporting is periodical. Parameter loggingInterval

determines  the interval between the reports and parameter

loggingDuration determines how long the configuration is valid

meaning after this duration has passed no further reports are sent.

In NR, the reporting can be periodical or event

based, determined by parameter reportType. For periodical reporting the

same parameters as in LTE and UMTS apply. For event based reporting,

parameter eventListForEventTriggeredMeasurement configures the event

type, namely 'out of coverage' or 'L1 event'. In case 'L1 event' is

selected as event type, the logging is performed according to

parameter loggingInterval at regular intervals only when the

conditions indicated by eventThresholdL1,

hysteresisL1, timeToTriggerL1 (defining the thresholds, hysteresis

and time to trigger) are met and if UE is 'camped normally' state

(TS 38.331, TS 38.304).  In case 'out of coverage' is selected as

event type, the

logging is performed according to parameter loggingInterval at regular

intervals only when the UE is in 'any cell selection' state.

Furthermore, logging is performed immediately upon transition from the

'any cell selection' state to the 'camped normally' state (TS 38.331,

TS 38.304).";

leaf traceCollectionEntityId {

type int64;

description "It specifies the TCE Id which is sent to the UE in

Logged MDT.";

reference "Clause 5.10.11 of 3GPP TS 32.422.";

}

leaf loggingDuration {

when '../../../jobType = "LOGGED_MDT_ONLY" or'

+ ' ../../../jobType = "LOGGED_MBSFN_MDT"';

type uint32 {

range "600|1200|2400|3600|5400|7200";

}

units seconds;

description "Specifies how long the MDT configuration is valid at the

UE in case of Logged MDT. The attribute is applicable only for

Logged MDT and Logged MBSFN MDT.";

reference "Clause 5.10.9 of 3GPP TS 32.422.";

}

leaf loggingInterval {

when '../../../jobType = "LOGGED_MDT_ONLY" or'

+ ' ../../../jobType = "LOGGED_MBSFN_MDT"';

type enumeration {

enum 320ms;

enum 640ms;

enum 1280ms;

enum 2560ms;

enum 5120ms;

enum 10240ms;

enum 20480ms;

enum 30720ms;

enum 40960ms;

enum 61440ms;

enum INFINITY;

}

description "Specifies the periodicty for Logged MDT. The attribute is

applicable only for Logged MDT and Logged MBSFN MDT.";

reference "Clause 5.10.8 of 3GPP TS 32.422.";

}

leaf reportType {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"';

type enumeration {

enum PERIODICAL;

enum EVENT_TRIGGERED;

}

description "It specifies report type for logged NR MDT";

reference "Clause 5.10.27 of 3GPP TS 32.422.";

}

leaf eventListForEventTriggeredMeasurement {

when '../../../jobType = "LOGGED_MDT_ONLY"';

type enumeration {

enum OUT_OF_COVERAGE ;

enum A2_EVENT ;

}

description "Specifies event types for event triggered measurement in the

case of logged NR MDT.  Each trace session may configure at most one

event. The UE shall perform logging of measurements only upon certain

condition being fulfilled:

- Out of coverage.

- A2 event.";

reference "Clause 5.10.28 of 3GPP TS 32.422.";

}

leaf eventThresholdL1 {

when '../../../jobType = "LOGGED_MDT_ONLY" or'

+ ' ../../../jobType = "LOGGED_MBSFN_MDT"';

type uint32 {

range "0..127";

}

description "It specifies the threshold which should trigger

the reporting in case of event based reporting of logged NR MDT.

The attribute is applicable only for Logged MDT and when reportType

is configured for event triggered reporting and when

eventListForEventTriggeredMeasurement is configured for L1 event.";

reference "clause 5.10.36 of TS 32.422.";

}

leaf hysteresisL1 {

when '../../../jobType = "LOGGED_MDT_ONLY" or '

+ '../../../jobType = "LOGGED_MBSFN_MDT"';

type uint32 {

range "0..30";

}

description "It specifies the hysteresis used within the entry and leave

condition of the L1 event based reporting of logged NR MDT.

The attribute is applicable only for Logged MDT, when reportType

is configured for event triggered reporting and when

eventListForEventTriggeredMeasurement is configured for L1 event. ";

reference "clause 5.10.37 of TS 32.422.";

}

leaf timeToTriggerL1 {

when '../../../jobType = "LOGGED_MDT_ONLY" or '

+ '../../../jobType = "LOGGED_MBSFN_MDT"';

type int32 {

range 0|40|64|80|100|128|160|256|320|480|512|640|1024|1280|2560|5120;

}

units milliseconds;

description "It specifies the threshold which should trigger

the reporting in case of event based reporting of logged NR MDT.

The attribute is applicable only for Logged MDT, when reportType

is configured for event triggered reporting and when

eventListForEventTriggeredMeasurement is configured for L1 event.";

reference "clauses 5.10.38 of TS 32.422.";

}

list areaConfigurationForNeighCell {

when '../../../jobType = "LOGGED_MDT_ONLY"';

key "idx";

leaf idx { type uint32 ; }

description "It specifies the area for which UE is requested to perform

measurement logging for neighbour cells which have list of frequencies.

If it is not configured, the UE shall perform measurement logging for

all the neighbour cells.

Applicable only to NR Logged MDT.";

reference "Clause 5.10.26 of 3GPP TS 32.422.";

uses AreaConfigGrp;

}

list mbsfnAreaList {

when '../../../jobType = "LOGGED_MBSFN_MDT"';

key "mbsfnAreaId earfcn";

max-elements 8;

description "The MBSFN Area consists of a MBSFN Area ID and Carrier

Frequency (EARFCN). The target MBSFN area List can have up to 8 entries.

This parameter is applicable only if the job type is Logged MBSFN MDT.";

reference "Clause 5.10.25 of 3GPP TS 32.422.";

uses MbsfnAreaGrp;

}

list nPNIdentityList {

description "It defines which NPNs that can be served by the NR cell,

and which CAG IDs or NIDs can be supported by the NR cell for

corresponding PNI-NPN or SNPN.";

key idx;

max-elements 1;

uses NpnIdGrp;

leaf idx { type string;}

}

}

grouping MdtConfigGrp {

description "Defines the configuration parameters of IOC

TraceJob which are specific for MDT. The attribute

anonymizationOfMdtData specifies the level of anonymization

of MDT data.  The attribute areaScope defines the area scope of MDT,

which is specified in clause 5.10.2 of TS 32.422.  The

optional attribute sensorInformation allows to

specify  the sensor information to include.  Based on the value

configured for attribute jobType in IOC TraceJob, the attributes

immediateMdtConfig or loggedMdtConfig are available: In case of

IMMEDIATE_MDT_ONLY or IMMEDIATE_MDT_AND_TRACE the attribute

immediateMdtConfig is applicable. In case of LOGGED_MDT_ONLY or

LOGGED_MBSFN_MDT the attribute loggedMdtConfig is applicable.";

leaf anonymizationOfMdtData {

when ../areaScope ;

type enumeration {

enum NO_IDENTITY;

enum TAC_OF_IMEI;

}

default NO_IDENTITY;

description "Specifies the level of anonymization of MDT data.

This attribute is only applicable for management based activation";

reference "Clause 5.10.12 of 3GPP TS 32.422.";

}

list areaScope {

key "idx";

leaf idx { type uint32 ; }

description "It specifies the area where data shall be collected.";

max-elements 1;

uses AreaScopeGrp;

}

leaf-list sensorInformation {

type enumeration {

enum BAROMETRIC_PRESSURE;

enum UE_SPEED;

enum UE_ORIENTATION;

}

description "It specifies which sensor information shall be included in

logged NR MDT and immediate NR MDT measurement if they are available.";

reference "Clause 5.10.29 of 3GPP TS 32.422.";

}

list immediateMdtConfig {

description "The set of parameters specific for Immediate MDT

configuration.";

key idx;

max-elements 1;

leaf idx { type string; }

uses ImmediateMdtConfigGrp;

}

list loggedMdtConfig {

description "The set of parameters specific for Logged MDT and Logged

MBSFN MDT configuration.";

key idx;

max-elements 1;

leaf idx { type string; }

uses LoggedMdtConfigGrp;

}

leaf mNOnly {

type boolean;

default false;

description "Specifies whether the MDT configuration is for MN

only or not. The value FALSE means the MDT configuration is for

both MN and SN. The value TRUE means the MDT configuration is

for MN only.";

}

list pLMNList {

key "mcc mnc";

uses types3gpp:PLMNId;

max-elements 16;

description "It indicates the PLMNs where measurement collection.";

reference "Clause 5.10.24 of 3GPP TS 32.422.";

}

}

grouping UECoreMeasConfigGrp {

description "Represents the UECoreMeasConfig dataType.

This <<dataType>> defines the aconfiguration parameters of IOC

TraceJob which are specific for 5GC UE level measurements

collection.";

leaf-list ueCoreMeasurements {

type string;

min-elements 1;

description "List of 5GC UE level measurements identified by name.

The list may include 5GC UE level measurements

defined in TS 28.558, or vendor specific measurements.

For 5GC UE level measurements defined in TS 28.558, the

name is constructed as follows:

'family.measurementName.subcounter' for measurement type

with specified subcounter

'family.measurementName.ALL' for measurement type with all

supported subcounters

'family.measurementName' for measurement type without

subcounters

'family' for measurement family, including all measurement

types and

the associated subcounters under this family.

For non-3GPP specified 5GC UE level measurements the name is

defined elsewhere.";

}

leaf ueCoreMeasGranularityPeriod {

type uint32;

description "Granularity period used to produce 5GC UE

level measurements. The period is defined in milliseconds (ms).";

}

leaf nfTypeToMeasure {

type string;

mandatory true;

description "It indicates the type of NE to produce the 5GC UE level

measurements.

allowedValues: The NF types represented by the measured object

classes as defined by f) of the 5GC UE level measurements specified

in TS 28.558.";

}

leaf-list objectInstances {

type string;

description "List of object instances.

Each object instance is identified by its DN.";

}

leaf-list rootObjectInstances {

type string;

description "List of object instances.

Each object instance is identified by its DN and designates

the root of a substree that contains the root object and all

descendant objects.";

}

}

grouping TraceJobGrp {

leaf jobType {

type enumeration {

enum IMMEDIATE_MDT_ONLY;

enum LOGGED_MDT_ONLY;

enum TRACE_ONLY;

enum IMMEDIATE_MDT_AND_TRACE;

enum RLF_REPORT_ONLY;

enum RCEF_REPORT_ONLY;

enum LOGGED_MBSFN_MDT;

enum 5GC_UE_LEVEL_MEASUREMENTS_ONLY;

enum TRACE_AND_5GC_UE_LEVEL_MEASUREMENTS;

enum IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS;

enum TRACE_AND_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS;

}

default TRACE_ONLY;

description "It specifies whether the

TraceJob represents only MDT, Logged MBSFN MDT, Trace, or

5GC UE level measurements job, or a combined job. It also

defines the MDT mode.";

reference "Clause 5.9a of 3GPP TS 32.422.";

}

list pLMNTarget {

key "mcc mnc";

description "Specifies which PLMN that the subscriber of the session to

be recorded uses as selected PLMN.";

uses types3gpp:PLMNId;

}

leaf-list listOfTraceMetrics {

when '../jobType = "TRACE_ONLY"'

+  ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type string;

description "List of trace metrics identified by name.

Includes trace messages, MDT measurements (Immediate MDT,

Logged MDT, Logged MBSFN MDT), RLF and RCEF reports,

see TS 32.422). Trace messages are identified with their message

identifier. Trace metric identifier is constructed as

defined in clause 10 of TS 32.422.

For non-3GPP specified trace metrics the name is defined elsewhere.";

reference "Clause 10 of 3GPP TS 32.422.";

}

leaf traceCollectionEntityIPAddress {

type inet:ip-address;

description "It specifies the address of the Trace Collection Entity

when the attribute traceReportingFormat is configured for the

file-based reporting. The attribute is applicable for both Trace and

MDT.";

reference "Clause 5.9 of 3GPP TS 32.422.";

}

leaf traceReportingConsumerUri {

when '../traceReportingFormat  = "STREAMING"';

type inet:uri;

description "URI of the Streaming Trace data reporting MnS consumer

(a.k.a. streaming target).";

reference "Clause 5.9 of 3GPP TS 32.422.";

}

list traceReference {

key "idx";

min-elements 1;

max-elements 1;

description "A globally unique identifier, which uniquely identifies the

Trace Session that is created by the TraceJob.

In case of shared network, it is the MCC and MNC of the Participating

Operator that request the trace session that shall be provided.

The attribute is applicable for both Trace and MDT.";

reference "Clause 5.6 of 3GPP TS 32.422.";

leaf idx { type uint32 ; }

uses trace3gpp:TraceReferenceGrp ;

}

leaf jobId {

type string;

yext3gpp:inVariant;

description "Identifier of a TraceJob";

yext3gpp:inVariant;

}

leaf traceReportingFormat {

type enumeration {

enum FILE-BASED;

enum STREAMING;

}

default FILE-BASED;

description "Specifies the trace reporting format - streaming trace

reporting or file-based trace reporting";

reference "3GPP TS 32.422 clause 5.11";

}

list traceTarget {

key "targetIdType targetIdValue";

max-elements 1;

leaf targetIdType {

type enumeration {

enum IMSI;

enum IMEI;

enum IMEISV;

enum PUBLIC_ID;

enum UTRAN_CELL;

enum E-UTRAN_CELL;

enum NG-RAN_CELL;

enum ENB;

enum RNC;

enum GNB;

enum SUPI;

}

}

leaf targetIdValue {

type string;

}

description "Specifies the target object of the Trace and MDT. The

attribute is applicable for both Trace and MDT. This attribute

includes the ID type of the target as an enumeration and the ID value.

The traceTarget shall be 'PUBLIC_ID' in case of a Management Based

Activation is done to an SCSCFFunction (Serving Call Session

Control Function)  or PCSCFFunction (Proxy Call Session Control

Function).  The traceTarget shall be 'UTRAN_CELL' only in case

of the UTRAN cell traffic trace function.

The traceTarget shall be 'E-UTRAN_CELL' only in case of E-UTRAN cell

traffic trace function.

The traceTarget shall be 'NG-RAN_CELL' only in case of NR cell

traffic trace function.

The traceTarget shall be either 'IMSI', 'IMEI' or

'IMEISV' if the Trace Session is activated to any of the following

ManagedEntity(ies):

- HssFunction

- MscServerFunction

- SgsnFunction

- GgsnFunction

- BmscFunction

- RncFunction

- MmeFunction

- ServingGWFunction

- PGWFunction

The traceTarget shall be either 'SUPI' or 'IMEISV' if the Trace Session

is activated to any of the followed ManagedEntity(ies):

- AFFunction

- AMFFunction

- AUSFFunction

- NEFFunction

- NRFFunction

- NSSFFunction

- PCSCFFunction

- SMFFunction

- UPFFunction

- UDMFunction

In case of signalling based MDT, the traceTarget attribute should

be able to carry 'PUBLIC_ID', 'IMSI', 'IMEI', 'IMEISV' or 'SUPI'.

In case of management based 5GC UE level measurements collection,

the traceTarget attribute shall be able to carry the corresponding

Measured UE Identifier as defined by the bullet g) of the 5GC UE

level measurements (see TS 28.558) when the TraceJob is created

at the subject ManagedEntity.";

reference "3GPP TS 32.422.";

}

list traceConfig {

when '../jobType = "TRACE_ONLY"'

+  ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"'

+  ' or ../jobType = "TRACE_AND_5GC_UE_LEVEL_MEASUREMENTS"'

+  ' or ../jobType =

"TRACE_AND_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS"';

key idx;

description "Trace config";

max-elements 1;

uses TraceConfigGrp;

leaf idx { type string; }

}

list mdtConfig {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"'

+ ' or ../jobType = "RLF_REPORT_ONLY"'

+ ' or ../jobType = "RCEF_REPORT_ONLY"'

+ ' or ../jobType = "LOGGED_MBSFN_MDT"'

+ ' or ../jobType = "IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS"'

+ ' or ../jobType =

"TRACE_AND_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS"';

key idx;

description "MDT config";

max-elements 1;

uses MdtConfigGrp;

leaf idx { type string; }

}

list ueCoreMeasConfig {

when '../jobType = "5GC_UE_LEVEL_MEASUREMENTS_ONLY"'

+ ' or ../jobType = "TRACE_AND_5GC_UE_LEVEL_MEASUREMENTS"'

+ ' or ../jobType = "IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS"'

+ ' or ../jobType =

"TRACE_AND_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS"';

key idx;

description "5GC UE level measurements config";

max-elements 1;

uses UECoreMeasConfigGrp;

leaf idx { type string; }

}

list nPNTarget {

description "applicable only for NR and shall be present in case of NPN

either a PNI-NPN or a SNPN) and for management-based activation when

several NPNs are supported in the RAN.";

key idx;

max-elements 1;

uses NpnIdGrp;

leaf idx { type string;}

}

}

grouping TraceSubtree {

description "Contains classes that manage Tracing.

Should be used in all  classes (or classes inheriting from)

- SubNetwork

- ManagedElement

- ManagedFunction

If a YANG module wants to augment these classes/list/groupings they must

augment all user classes!";

list TraceJob {

description "A TraceJob instance represents the Trace Control and

Configuration parameters of a particular Trace Job (see TS 32.421 and

TS 32.422 for details). It can be name-contained by SubNetwork,

ManagedElement, ManagedFunction.

To activate Trace Jobs, a MnS consumer has to create TraceJob object

instances on the MnS producer. A MnS consumer can activate a Trace Job

for another MnS consumer since it is not required the value of

traceCollectionEntityIPAddress or traceReportingConsumerUri to be

his own.

For the details of Trace Job activation see clauses 4.1.1.1.2 and

4.1.2.1.2 of TS 32.422.

When a MnS consumer wishes to deactivate a Trace Job, the MnS consumer

shall delete the corresponding TraceJob instance.

For details of management Trace Job activation/deactivation see clause

4.1.1.1.2 of TS 32.422.

The attribute traceReference specifies a globally unique ID and

identifies a Trace session. One Trace Session may be activated to

multiple Network Elements. The traceReference is populated by the

consumer that makes the request for a Trace Session.

The jobId attribute presents the job identifier of a TraceJob instance.

The jobId can be used to associate  multiple TraceJob instances.

For example, it is possible to configure the same jobId value for

multiple TraceJob instances required to produce the data (e.g. RSRP

values of M1 and RLF reports) for a specific network analysis.

The attribute traceReportingFormat defines the method for reporting

the produced measurements. The selectable options are file-based or

stream-based reporting. In case of file-based reporting the attribute

traceCollectionEntityIPAddress is used to specify the IP address to

which the trace records shall be transferred, while in case of

stream-based reporting the attribute traceReportingConsumerUri

specifies the streaming target.

The mandatory attribute traceTarget determines the target object of

the TraceJob. Dependent on the network element to which the Trace

Session is activated different types of the target object are possible.

The attribute pLMNTarget defines the PLMN for which sessions shall be

selected in the Trace Session in case of management based activation

when several PLMNs are supported in the RAN.

The attribute jobType specifies the kind of data to collect. In case of

Trace only, the configuration parameters of attribute traceConfig shall

be applied. In case of Immediate MDT only, Logged MDT only, RLF reports

only, RCEF reports only and Logged MBSFN MDT, the configuration

parameters of attribute mdtConfig or a subset of these shall be

applied. In case of UE measurements only, the configuration parameters

of attribute ueCoreMeasConfig shall be applied. In case of any

combination of Trace, Immediate MDT, Trace and UE measurements, the

configuration parameters of the corresponding attributes traceConfig,

mdtConfig and ueCoreMeasConfig are applicable.

Creation and deletion of TraceJob instances by MnS consumers is

optional; when not supported, the TraceJob instances may be created

and deleted by the system or be pre-installed.";

key id;

uses top3gpp:Top_Grp ;

container attributes {

uses TraceJobGrp ;

}

uses files3gpp:FilesSubtree {

if-feature FilesUnderTraceJob;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 1 ***