3GPP TSG-SA5 Meeting #161	S5-253002

Fukuoka, Japan, 19 - 23 May 2025	revision of S5-252372

Source:	Nokia, CMCC

Title:	Pseudo-CR on NRM for data generation NDTs

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS28.561

Version:	0.2.0

Work Item:	Management aspects of Network Digital Twins

Comments

This pCR is to add NRM for the NDT supporting data generation following agreements in the NDT study.

Proposed Changes

* * * First Change * * * *

## 6.2	Information model for NDT (MnS component typeB)

#### 6.2.1.3	Class definition

##### 6.2.1.3.2	NDTJob <<InformationObjectClass>>

###### 6.2.1.3.2.1	Definition

This IOC represents the properties of an NDT job demand created by an MnS consumer.

###### 6.2.1.3.2.2	Attributes

The NDTJob IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| networkDataGenTask | M | T | T | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtFunctionRef | M | T | T | F | T |
| ndtReportRefList | M | T | T | F | T |

##### 6.2.1.3.3	NDTReport <<InformationObjectClass>>

###### 6.2.1.3.3.1	Definition

This IOC represents the properties of an NDT report corresponding to an NDT job.

The attribute "PerformanceData" is specified performance metrics and/or alarm types that are collected and reported by NDT after the behaviour is modelled in NDT.

###### 6.2.1.3.3.2	Attributes

The NDTReport IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related roles |  |  |  |  |  |
| ndtJobRef | M | T | T | F | T |
| ndtFunctionRef | M | T | T | F | T |

## 6.3	Attribute definitions

### 6.3.1	Attribute properties

Table 6.3.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| ndtJobRef | It indicates an DN of a NDTJob Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtFunctionRef | It indicates an DN of a NDTFunction Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtReportRefList | It indicates a list of DN for NDTReport Instances. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| networkDataGenTask | It indicates configurations of the network for which data generation is desired. The networkDataGenTask can be applied for: - using NDT to generate ML training data - using NDT to generate user experience data | type: NDTInputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

* * * End of Changes * * * *