3GPP TSG-SA5 Meeting #161	S5-253007

Fukuoka, Japan, 19 - 23 May 2025 	revision of S5-252357

Source:	Nokia

Title:	Pseudo-CR on Control of CCLs

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.2.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add use case and requirements for CCL scope & action-execution conflicts handling as was agreed in the CCL study in TR28. 867

Proposed Changes

* * * First Change * * * *

# 5	Management capabilities

## 5.1	Dynamic control and composition of CCLs - DynCCL

### 5.1.1	Description

CCLs may be dynamically realized. There are two aspects to dynamically realization of CCLs - dynamic instantiation of a CCL from an existing template and dynamically composing the CCL from discrete components based e.g. on the provided goals.

### 5.1.2		Use cases

#### 5.1.2.1	General CCL Control – DynCCL_0y

A CCL contains a set of logic functionalities or steps, each providing a specific functionality and where the steps work together to achieve the stated desired outcomes over a given network scope. The MnS consumer should be able to configure and receive  information about the desired outcomes of the CCL.

Generally, the four CCL steps of Monitoring, Analysis, Decision and Execution are expected with the expectation that each step is accomplished by a single management function or service. However, one management function or service may also accomplish the functionality of more than 1 step. The MnS consumer should be able to receive information about the management functions or services that form  the CCL.

A CCL may have four scopes including a desired outcomes scope, measurement scope, a control scope and an impact scope. The scopes for the different CCLs can be managed by the MnS consumer. The MnS consumer should also be able to receive reports about these different aspects of the CCL, e.g., about the status of the CCLs execution as well as to configure the reporting.

#### 5.1.2.2	Composing a CCL from discrete components – DynCCL_0x

A CCL may be composed from steps provided by different management functions or management services. i.e. the CCLs is assembled on demand by MnS consumers, using capabilities offered by the Management system, e.g. from independent management functions. The CCLs components, as well as the communication and interoperation between components, are based the different 3GPP management services. Accordingly, the MnS consumer should be able to identify and indicate the MnFs or MnS producers that should be used to compose a CCL. Moreover, the MnS consumer may indicate towards the MnS producer the request to compose the CL of a particular type (e.g. for optimizing energy efficiency) without requiring to state the specific components that should be used.

Two approaches are possible:

-	Composition from management Functions: Different management functions may be used to realize the different steps of a closed loop, for example, an MDA function may realize the analytics step of the CCL while another management function may realize the decision step of the CCL.

Figure 5.1.2.2-1: Management functions as steps of a closed control loop

-	Composition from management services: Different management services may be used to realize the different steps of a closed loop, i.e. the management service provides the output expected from a specific step.

EXAMPLE:	A capability of the MDA MnS realizes an analytics step of the CCL while another capability may realize a specific data collection step of the CCL.

b)

Figure 5.1.2.2-2: Management services used as implementations of CCL steps:
a) MDA MnS and PM job the respective implementations of the analysis and data collection steps; and b) MDA MnS as the implementation of the decision step

The MnS consumer should be enabled to control the composition of such a CCL. The MnS consumer could request for and be notified about the composition of a CCL from a set of specific components (i.e. specific management functions or management services). The request could indicate components with specific given capabilities (such as analytics services with specific analytics types) which should be combined to achieve the closed loop. Moreover, the request could be for composition of a CCL required to achieve a specific set of desired outcomes or goals.

### 5.1.3	Requirements

Table 5.1.3-1

| Requirement label | Description | Related use case(s)/ Motivation |
| --- | --- | --- |
| REQ-DynCCL_0y-01 | The CCL MnS Producer should support a capability to provide information to the MnS consumer about the management functions and services that make up the CCL and where applicable the functionality accomplished by the components. | UC-DynCCL_0y Clause 5.1.2.1 |
| REQ-DynCCL_0y-02 | The CCL MnS Producer should support a capability enabling the MnS consumer to configure and receive information on status of execution of the CCL. | UC-DynCCL_0y Clause 5.1.2.1 |
| REQ-DynCCL_0y-03 | The CCL MnS Producer should support a capability enabling the MnS consumer to configure and receive information on the scopes of the CCL | UC-DynCCL_0y Clause 5.1.2.1 |
| REQ-DynCCL_0y-04 | The CCL MnS Producer should support a capability to report to the MnS consumer about the CCL | UC-DynCCL_0y Clause 5.1.2.1 |
| REQ-DynCCL_0x-01 | The CCL MnS Producer should support a capability enabling the MnS consumer to request for a CCL (instance) to be composed from a set of management function types or instances or management services | UC-DynCCL_0x Clause 5.1.2.2 |
| REQ-DynCCL_0x-02 | The MnS producer for CCL management should support a capability enabling the MnS consumer to request that a CCL of a specific type or fulfilling a stated goal should be composed from a set of management function types or instances or services | UC-DynCCL_0x Clause 5.1.2.2 |
| REQ-DynCCL_0x-03 | The MnS producer for CCL management should support a capability enabling the MnS consumer to provide conditions under which a CCL can be dynamically composed or instantiated triggered to execute | UC-DynCCL_0x Clause 5.1.2.2 |
| REQ-DynCCL_0x-04 | The MnS producer for CCL management should support a capability enabling the MnS consumer to be notified when a CCL is dynamically composed or instantiated or triggered to execute | UC-DynCCL_0x Clause 5.1.2.2 |

* * * next Change * * * *

# 6	Model

## 6.1	Imported and associated information entities

TBD

### 6.1.1	Imported information entities and local labels

TBD

### 6.1.2	Associated information entities and local labels

TBD

## 6.2	Class diagram

### 6.2.1	Relationships

Figure 6.2.1-1: Relations for common information models for AI/ML management

Editor’s Note: The handling of Goal, targets or objectives for the general closed control loops is FFS

### 6.2.2	Inheritance

Figure 6.2.2-1: Inheritance Hierarchy for Closed Control Loops

### 6.3	Class definitions

### 6.3.1	ClosedControlLoop

#### 6.3.1.1	Definition

This IOC represents the closed control loop. It represents the information for controlling and monitoring a CCL associated with a stated scope.

The ClosedControlLoop is name-contained by SubNetwork or ManagedElement and is associated with a CCLreport that contains reported  information about the CCL. Accordingly, the report about a CCL can exist even when the CCL is deleted.

The capabilities of the CCL are contained in one or more CCLPurposes that describe what the CCL is capable of doing or can be configured to do - including information the network resources for which the CCL can execute decisions and actions. So, the ClosedControlLoop is associated with one or more CCLPurpose(s) that indicate(s) a list of characteristics that describe what a CCL can/is expected to be able to do. The purpose describes the type of functionality that can be executed including problem recovery and fault management .

The operational information about the CCL is contained in the CCLScope(s), so the ClosedControlLoop is associated with one or more CCLScope(s). The CCLScope defines what the CCL has been configured to read, evaluate, control, etc.

#### 6.3.1.2	Attributes

The CCLControlLoop IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.1.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cCLComponentsInfo | O | T | T | F | T |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| cCLPurposeRefList | M | T | T | T | T |

#### 6.3.1.3	Attribute constraints

None

#### 6.3.1.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

### 6.3.2	CCLScope

#### 6.3.2.1	Definition

It indicates a scope of a CCL. It may be the measurement scope, control scope or impact scope.

The CCLScope includes the attribute scopeType that indicates the type of scope that represented by the particular scope instance.

#### 6.3.2.2	Attributes

The CCLScope IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.2.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| scopeType | O | T | F | F | T |

#### 6.3.2.3	Attribute constraints

None.

#### 6.3.2.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

### 6.3.3	CCLReport

#### 6.3.3.1	Definition

This class represents the reported outcomes on a CCL instance, e.g., the information about the outcomes on one or the executing of the CCL. An CCLReport is contained by the entity containing the CCL, since the CCLReport can exist beyond the life of the CCL on which it is reporting.

There is one CCLReport per CCL for an observation time. The content of the CCLReport may be different for different observation time.

#### 6.3.3.2	Attributes

The CCLReport IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.3.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attributes related to role |  |  |  |  |  |

#### 6.3.3.3	Attribute constraints

None.

#### 6.3.3.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

### 6.3.4	CCLPurpose <<dataType>>

#### 6.3.4.1	Definition

This data type represents a single purpose that describes what a CCL can do. The purpose is alist of characteristics that describe the capabilities of the CCL.

#### 6.3.4.2	Attributes

Table 6.3.4.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attributes related to role |  |  |  |  |  |

#### 6.3.4.3	Attribute constraints

None.

#### 6.3.4.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions..

### 6.3.A	CCLComponentInfo <<dataType>>

#### 6.3.A.1	Definition

This data type represents a single purpose that describes what a CCL can do. The purpose is alist of characteristics that describe the capabilities of the CCL.

#### 6.3.A.2	Attributes

Table 6.3.A.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cCLComponentId | M | T | F | F | T |
| cCLSteps | M | T | F | F | T |

#### 6.3.A.3	Attribute constraints

None.

#### 6.3.A.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

## 6.4	Attribute definitions

### 6.4.1	Attribute properties

Table 6.4.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| scopeType | It indicates the type of scope that represented by the particular scope instance. allowedValues: CCL_MEASUREMENT_SCOPE, CCL_TARGET_SCOPE, CCL_CONTROL_SCOPE, CCL_IMPACT_SCOPE Editor’s Note: The allowed values will be revisited | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| operationalState | It indicates the operational state of the ClosedControlLoop instance. It describes whether the resource is installed and partially or fully operable (Enabled) or the resource is not installed or not operable (Disabled). Allowed values; Enabled/Disabled allowedValues: "ENABLED", "DISABLED". The meaning of these values is as defined in 3GPP TS 28.625 [14] and ITU-T X.731 [15]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Disabled isNullable: False |
| administrativeState | It indicates the administrative state of the ClosedControlLoop instance. It describes the permission to use or the prohibition against using the ClosedControlLoop instance. The administrative state is set by the MnS consumer. Allowed values; Locked/Unlocked allowedValues: "LOCKED", "UNLOCKED". The meaning of these values is as defined in 3GPP TS 28.625 [14] and ITU-T X.731 [15]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Locked isNullable: False |
| cCLComponentsInfo | It indicates information on the constituent components of a CCL. allowedValues: N/A | type: CCLComponentInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLComponentId | It indicates the identifier of a CCL component. It is the DN of a object instantiated to act as a component of the CCL | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLSteps | It indicates the CCL steps or functionality that is accomplished by a CCL component. allowedValues: DATA_COLLECTION, ANALYSIS, DECISION, EXECUTION | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |

* * * End of Changes * * * *