3GPP TSG-SA5 Meeting #161 	S5-252256

Fukuoka City, Fukuoka, Japan, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0538 | rev | - | Current version: | 17.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR TS 28.623 Corrections on Trace Target |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2025-05-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Trace Target is specified in section 5.12 of TS32.422. It consists of the ID type and the ID value. However, in 28.622, traceTarget is defined as a string with allowed value for ID type only, e.g. "NG-RAN_CELL" for NR, or "E-UTRAN_CELL" for E-UTRAN. In YANG, traceTarget is defined as data type with targetIdType as ENUM and targetIdValue as string. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Make traceTarget as a new data type |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification, which may lead to incorrect implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1709 at commit 1c3920a71149f8cd143ef7c7ef008b0aabe31ebb |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE 1 ***

*** yang-models/_3gpp-common-trace.yang ***

<CODE BEGINS>

module _3gpp-common-trace {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-trace";

prefix "trace3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types {prefix types3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-files { prefix files3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Trace handling

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)";

revision 2025-05-05 { reference "CR-0538" ; }

revision 2025-02-05 { reference "CR-0502" ; }

revision 2024-11-05 { reference "CR-0469" ; }

revision 2024-08-09 { reference "CR-0419" ; }

revision 2024-05-13 { reference "CR-0360" ; }

revision 2024-04-05 { reference "CR-0341" ; }

revision 2023-11-05 { reference "CR-0293 CR-0301" ; }

revision 2023-09-17 { reference CR-0270 ; }

revision 2023-07-25 { reference CR00252; }

revision 2023-02-17 { reference "CR-0232"; }

revision 2023-02-15 { reference "CR-0236"; }

revision 2022-04-27 { reference "CR-0159"; }

revision 2021-10-18 { reference "CR-0139"; }

revision 2021-07-22 { reference "CR-0137"; }

revision 2021-01-25 { reference "CR-0122"; }

revision 2020-11-16 { reference "CR-0117"; }

revision 2020-08-06 { reference "CR-0102"; }

feature FilesUnderTraceJob {

description "Files shall be contained under TraceJob";

}

grouping TraceReference {

leaf mcc {

mandatory true;

type types3gpp:Mcc;

}

leaf mnc {

mandatory true;

type types3gpp:Mnc;

}

leaf traceId {

mandatory true;

type int64;

}

}

grouping AreaScopeGrp {

description "Represents the AreaScope dataType.

This <<dataType>> defines the area scope of MDT.

The Area Scope parameter in LTE and NR is either:

- list of Cells, identified by E-UTRAN-CGI or NG-RAN CGI.

Maximum 32 CGI can be defined.

- list of Tracking Area, identified by TAC.

Maximum of 8 TAC can be defined.

- list of Tracking Area Identity, identified by TAC with

associated plmn-Identity perTAC-List containing the

PLMN identity for each TAC. Maximum of 8 TAI can be defined.";

choice AreaScopeChoice {

leaf-list eutraCellIdList {

type string;

min-elements 1;

max-elements 32;

description "List of E-UTRAN cells identified by E-UTRAN-CGI";

}

leaf-list nrCellIdList {

type string;

min-elements 1;

max-elements 32;

description "List of NR cells identified by NG-RAN CGI";

}

leaf-list tacList {

type types3gpp:Tac;

min-elements 1;

max-elements 8;

description "Tracking Area Code list";

}

list taiList {

description "Tracking Area Identity list";

key idx;

min-elements 1;

max-elements 8;

leaf idx { type string; }

uses types3gpp:TaiGrp;

}

}

}

grouping TraceJobGrp {

leaf jobType {

type enumeration {

enum IMMEDIATE_MDT_ONLY;

enum LOGGED_MDT_ONLY;

enum TRACE_ONLY;

enum IMMEDIATE_MDT_AND_TRACE;

enum RLF_REPORT_ONLY;

enum RCEF_REPORT_ONLY;

enum LOGGED_MBSFN_MDT;

}

default TRACE_ONLY;

description "Specifies the MDT mode and it specifies also whether the

TraceJob represents only MDT, Logged MBSFN MDT, Trace or a combined

Trace and MDT job. The attribute is applicable for Trace, MDT, RCEF and

RLF reporting.";

reference "Clause 5.9a of 3GPP TS 32.422 for additional details on the

allowed values.";

}

list listOfInterfaces {

key idx;

must 'count(MSCServerInterfaces)+count(MGWInterfaces)+count(RNCInterfaces)'

+'+count(SGSNInterfaces)+count(GGSNInterfaces)+count(S-CSCFInterfaces)'

+'+count(P-CSCFInterfaces)+count(I-CSCFInterfaces)+count(MRFCInterfaces)'

+'+count(MGCFInterfaces)+count(IBCFInterfaces)+count(E-CSCFInterfaces)'

+'+count(BGCFInterfaces)+count(ASInterfaces)+count(HSSInterfaces)'

+'+count(EIRInterfaces)+count(BM-SCInterfaces)+count(MMEInterfaces)'

+'+count(SGWInterfaces)+count(PDN_GWInterfaces)+count(eNBInterfaces)'

+'+count(en-gNBInterfaces)+count(AMFInterfaces)+count(AUSFInterfaces)'

+'+count(NEFInterfaces)+count(NRFInterfaces)+count(NSSFInterfaces)'

+'+count(PCFInterfaces)+count(SMFInterfaces)+count(SMSFInterfaces)'

+'+count(UDMInterfaces)+count(UPFInterfaces)+count(ng-eNBInterfaces)'

+'+count(gNB-CU-CPInterfaces)+count(gNB-CU-UPInterfaces)'

+'+count(gNB-DUInterfaces)';

description "Specifies the interfaces that need to be traced.

The attribute is applicable only for Trace. In

case this attribute is not used, it carries a null semantic.";

reference "Clause 5.5 of 3GPP TS 32.422.";

leaf idx { type uint32 ; }

leaf-list MSCServerInterfaces {

type enumeration {

enum A ;

enum Iu-CS ;

enum Mc ;

enum MAP-G ;

enum MAP-B ;

enum MAP-E ;

enum MAP-F ;

enum MAP-D ;

enum MAP-C ;

enum CAP ;

}

}

leaf-list MGWInterfaces {

type enumeration {

enum Mc ;

enum Nb-UP ;

enum Iu-UP ;

}

}

leaf-list RNCInterfaces {

type enumeration {

enum Iu-CS ;

enum Iu-PS ;

enum Iur ;

enum Iub ;

enum Uu ;

}

}

leaf-list SGSNInterfaces {

type enumeration {

enum Gb ;

enum Iu-PS ;

enum Gn ;

enum MAP-Gr ;

enum MAP-Gd ;

enum MAP-Gf ;

enum Ge ;

enum Gs ;

enum S6d ;

enum S4 ;

enum S3 ;

enum S13 ;

}

}

leaf-list GGSNInterfaces {

type enumeration {

enum Gn ;

enum Gi ;

enum Gmb ;

}

}

leaf-list S-CSCFInterfaces {

type enumeration {

enum Mw ;

enum Mg ;

enum Mr ;

enum Mi ;

}

}

leaf-list P-CSCFInterfaces {

type enumeration {

enum Gm ;

enum Mw ;

}

}

leaf-list I-CSCFInterfaces {

type enumeration {

enum Cx ;

enum Dx ;

enum Mg ;

enum Mw ;

}

}

leaf-list MRFCInterfaces {

type enumeration {

enum Mp ;

enum Mr ;

}

}

leaf-list MGCFInterfaces {

type enumeration {

enum Mg ;

enum Mj ;

enum Mn ;

}

}

leaf-list IBCFInterfaces {

type enumeration {

enum Ix ;

enum Mx ;

}

}

leaf-list E-CSCFInterfaces {

type enumeration {

enum Mw ;

enum Ml ;

enum Mm ;

enum Mi-Mg ;

}

}

leaf-list BGCFInterfaces {

type enumeration {

enum Mi ;

enum Mj ;

enum Mk ;

}

}

leaf-list ASInterfaces {

type enumeration {

enum Dh ;

enum Sh ;

enum ISC ;

enum Ut ;

}

}

leaf-list HSSInterfaces {

type enumeration {

enum MAP-C ;

enum MAP-D ;

enum Gc ;

enum Gr ;

enum Cx ;

enum S6d ;

enum S6a ;

enum Sh ;

}

}

leaf-list EIRInterfaces {

type enumeration {

enum MAP-F ;

enum S13 ;

enum MAP-Gf ;

}

}

leaf-list BM-SCInterfaces {

type enumeration {

enum Gmb ;

}

}

leaf-list MMEInterfaces {

type enumeration {

enum S1-MME ;

enum S3 ;

enum S6a ;

enum S10 ;

enum S11 ;

enum S13 ;

}

}

leaf-list SGWInterfaces {

type enumeration {

enum S4 ;

enum S5 ;

enum S8 ;

enum S11 ;

enum Gxc ;

}

}

leaf-list PDN_GWInterfaces {

type enumeration {

enum S2a ;

enum S2b ;

enum S2c ;

enum S5 ;

enum S6b ;

enum Gx ;

enum S8 ;

enum SGi ;

}

}

leaf-list eNBInterfaces {

type enumeration {

enum S1-MME ;

enum X2 ;

}

}

leaf-list en-gNBInterfaces {

type enumeration {

enum S1-MME ;

enum X2 ;

enum Uu ;

enum F1-C ;

enum E1 ;

}

}

leaf-list AMFInterfaces {

type enumeration {

enum N1 ;

enum N2 ;

enum N8 ;

enum N11 ;

enum N12 ;

enum N14 ;

enum N15 ;

enum N20 ;

enum N22 ;

enum N26 ;

}

}

leaf-list AUSFInterfaces {

type enumeration {

enum N12 ;

enum N13 ;

}

}

leaf-list NEFInterfaces {

type enumeration {

enum N29 ;

enum N30 ;

enum N33 ;

}

}

leaf-list NRFInterfaces {

type enumeration {

enum N27 ;

}

}

leaf-list NSSFInterfaces {

type enumeration {

enum N22 ;

enum N31 ;

}

}

leaf-list PCFInterfaces {

type enumeration {

enum N5 ;

enum N7 ;

enum N15 ;

}

}

leaf-list SMFInterfaces {

type enumeration {

enum N4 ;

enum N7 ;

enum N10 ;

enum N11 ;

enum S5-C ;

enum N38 ;

enum N16 ;

enum N16a ;

}

}

leaf-list SMSFInterfaces {

type enumeration {

enum N20 ;

enum N21 ;

}

}

leaf-list UDMInterfaces {

type enumeration {

enum N8 ;

enum N10 ;

enum N13 ;

enum N21 ;

}

}

leaf-list UPFInterfaces {

type enumeration {

enum N4 ;

}

}

leaf-list ng-eNBInterfaces {

type enumeration {

enum NG-C ;

enum Xn-C ;

enum Uu ;

}

}

leaf-list gNB-CU-CPInterfaces {

type enumeration {

enum NG-C ;

enum Xn-C ;

enum Uu ;

enum F1-C ;

enum E1 ;

enum X2-C ;

}

}

leaf-list gNB-CU-UPInterfaces {

type enumeration {

enum E1 ;

}

}

leaf-list gNB-DUInterfaces {

type enumeration {

enum F1-C ;

}

}

}

leaf-list listOfNETypes {

type enumeration {

enum MSC_SERVER;

enum SGSN;

enum MGW;

enum GGSN;

enum RNC;

enum BM_SC;

enum MME;

enum SGW;

enum PGW;

enum ENB;

enum EN_GNB;

enum GNB_CU_CP;

enum GNB_CU_UP;

enum GNB_DU;

}

description "Specifies the network element types where the trace should

be activated. The attribute is applicable only for Trace with

Signalling Based Trace activation. In case this attribute is not used,

it carries a null semantic";

reference "Clause 5.4 of 3GPP TS 32.422.";

}

list pLMNTarget {

key "mcc mnc";

description "Specifies which PLMN that the subscriber of the session to

be recorded uses as selected PLMN.";

reference "Clause 5.9b of 3GPP TS 32.422.";

uses types3gpp:PLMNId;

}

leaf-list listOfTraceMetrics {

when '../jobType = "TRACE_ONLY"'

+  ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type string;

description "Specifies the metrics to be reported.";

reference "Clause 10 of 3GPP TS 32.422";

}

leaf traceReportingConsumerUri {

when '../traceReportingFormat  = "STREAMING"';

type inet:uri;

description "It specifies the URI of the Streaming Trace data reporting

MnS consumer (a.k.a. streaming target).";

reference "Clause 5.9c of 3GPP TS 32.422";

}

leaf traceCollectionEntityIPAddress {

when '../traceReportingFormat  = "FILE_BASED" or '

+'../jobType = "LOGGED_MDT_ONLY" or ../jobType = "LOGGED_MBSFN_MDT"';

type union {

type inet:uri;

type inet:ip-address;

}

mandatory true;

description "It specifies the address of the Trace Collection Entity when

the attribute traceReportingFormat is configured for the file-based

reporting. The attribute is applicable for both Trace and MDT.";

reference "Clause 5.9 of 3GPP TS 32.422";

}

leaf traceDepth {

when '../jobType = "TRACE_ONLY"'

+  ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration  {

enum MINIMUM;

enum MEDIUM;

enum MAXIMUM;

enum VENDORMINIMUM;

enum VENDORMEDIUM;

enum VENDORMAXIMUM;

}

default MAXIMUM;

description "It specifies the trace depth.The attribute is applicable only

for Trace, otherwise it carries a null semantic.";

reference "Clause 5.3 of 3GPP TS 32.422.";

}

list traceReference {

uses TraceReference;

key "mcc mnc traceId";

max-elements 1;

description "A globally unique identifier, which uniquely identifies the

Trace Session that is created by the TraceJob.

In case of shared network, it is the MCC and MNC of the Participating

Operator that request the trace session that shall be provided.

The attribute is applicable for both Trace and MDT.";

}

reference "Clause 5.6 of 3GPP TS 32.422.";

leaf jobId {

type string;

description "Identifier of a TraceJob";

yext3gpp:inVariant;

}

leaf traceReportingFormat {

type enumeration {

enum FILE-BASED;

enum STREAMING;

}

default FILE-BASED;

description "Specifies the trace reporting format - streaming trace

reporting or file-based trace reporting";

reference "3GPP TS 32.422 clause 5.11";

}

list traceTarget {

key "targetIdType";

max-elements 1;

description "It specifies the target object of the Trace and MDT.

The attribute is applicable for both Trace and MDT. This attribute

consists the TraceTargetType and traceTargetValueList

In case of management based Immediate MDT, RLF reporting, or RCEF

reporting, the traceTarget attribute shall be null value.";

leaf targetIdType {

type enumeration {

enum IMSI;

enum IMEI;

enum IMEISV;

enum PUBLIC_ID;

enum UTRAN_CELL;

enum E-UTRAN_CELL;

enum NG-RAN_CELL;

enum ENB;

enum RNC;

enum GNB;

enum SUPI;

}

description "It specifies the target object type of the Trace and MDT.

The attribute is applicable for both Trace and MDT.

The targetIdType shall be PUBLIC_ID in case of a Management Based

Activation is done to an SCSCFFunction (Serving Call Session Control

Function) or PCSCFFunction (Proxy Call Session Control Function)

The targetIdType shall be UTRAN_CELL only in case of

UTRAN cell traffic trace function.

The targetIdType shall be E-UTRAN_CELL only in case of E-UTRAN cell

traffic trace function.

The targetIdType shall be NG-RAN_CELL only in case of NR cell

traffic trace function.

The targetIdType shall be either IMSI, IMEI or

IMEISV if the Trace Session is activated to any of the following

ManagedEntity(ies):

- HssFunction

- MscServerFunction

- SgsnFunction

- GgsnFunction

- BmscFunction

- RncFunction

- MmeFunction

- ServingGWFunction

- PGWFunction

The targetIdType shall be either SUPI or IMEISV if the Trace Session

is activated to any of the following ManagedEntity(ies):

- AFFunction

- AMFFunction

- AUSFunction

- NEFFunction

- NRFFunction

- NSSFFunction

- PCFFunction

- SMFFunction

- UPFFunction

- UDMFunction

In case of signalling based MDT, the targetIdType attribute shall be

able to carry PUBLIC_ID, IMSI, IMEI,  IMEISV or SUPI.

In case of management based Logged MDT, the targetIdType attribute

shall carry an eNB or a gNB or an RNC.

The Logged MDT should be initiated on the specified eNB/gNB/RNC in

targetIdValueList.";

}

leaf-list targetIdValueList {

type string;

description "It specifies the ID value(s) of the target object defined

by TraceTargetType";

}

reference "3GPP TS 32.422";

}

list triggeringEvents {

when '../jobType = "TRACE_ONLY" or ' +

'../jobType = "IMMEDIATE_MDT_AND_TRACE"';

key idx;

description "It specifies the triggering event parameter of the trace

session. The attribute is applicable only for Trace.";

reference "Clause 5.1 of 3GPP TS 32.422";

leaf idx { type uint32; }

max-elements 1;

leaf-list MSC_SERVER {

type enumeration {

enum MO_MT_CALLS;

enum MO_MT_SMS;

enum LU_IMSIattach_IMSIdetach;

enum SS;

}

}

leaf-list SGSN {

type enumeration {

enum PDPcontext;

enum MO_MT_SMS;

enum RAU_GPRSattach_GPRSdetach;

enum MBMScontext;

}

}

leaf-list MGW {

type enumeration {

enum CONTEXT;

}

}

leaf-list GGSN {

type enumeration {

enum PDPcontext;

enum MBMScontext;

}

}

leaf-list IMS {

type enumeration {

enum SIPsession_StandaloenTransaction;

}

}

leaf-list BM_SC {

type enumeration {

enum MBMSactivation;

}

}

leaf-list MME {

type enumeration {

enum UEinitiatedPDNconnectivityRequest;

enum ServiceRequest;

enum InitialAttach_TAU_Detach;

enum UEInitiatedPDNdisconnection;

enum BearerActivationModificationDeletion;

enum Handover;

}

}

leaf-list SGW {

type enumeration {

enum PDNconnectionCreation;

enum PDNconnectionTermination;

enum BearerActivationModificationDeletion;

}

}

leaf-list PGW {

type enumeration {

enum PDNconnectionCreation;

enum PDNconnectionTermination;

enum BearerActivationModificationDeletion;

}

}

leaf-list AMF {

type enumeration {

enum Registration;

enum ServiceRequest;

enum Handover;

enum UEderegistration;

enum NetworkDeregistration;

enum UEMobilityFromEPC;

enum UEMobilityToEPC;

enum CovergedChargingService;

}

}

leaf-list SMF {

type enumeration {

enum PDUsessionEstablishment;

enum PDUsessionModification;

enum PDUsessionRelease;

enum PDUsessionUPactivationDeactivation;

enum MobilityBtw3gppAndN3gppTo5GC;

enum MobilityFromEpc;

enum MobilityBtwISMForVSMF;

enum CovergedChargingService;

enum OfflineOnlyCharging;

}

}

leaf-list PCF {

type enumeration {

enum AMpolicy;

enum SMpolicy;

enum Authorization;

enum BDTpolicy;

enum SpendingLimitControl;

}

}

leaf-list UPF {

type enumeration {

enum N4Session;

}

}

leaf-list AUSF {

type enumeration {

enum UEauthentication;

}

}

leaf-list NEF {

type enumeration {

enum EventExposure;

enum PFDmanagement;

enum ParameterProvisiong;

enum Trigger;

}

}

leaf-list NRF {

type enumeration {

enum NFmanagement;

enum NFdiscovery;

}

}

leaf-list NSSF {

type enumeration {

enum NSSelection;

enum NSSAI;

}

}

leaf-list SMSF {

type enumeration {

enum SMService;

}

}

leaf-list UDM {

type enumeration {

enum UEcontext;

enum SubscriberData;

enum UEauthentication;

enum EventExposure;

}

}

}

leaf anonymizationOfMDTData {

when ../areaScope ;

type enumeration {

enum NO_IDENTITY;

enum TAC_OF_IMEI;

}

default NO_IDENTITY;

description "Specifies the level of anonymization for

management based MDT.";

reference "Clause 5.10.12 of 3GPP TS 32.422";

}

list areaConfigurationForNeighCell {

when '../jobType = "LOGGED_MDT_ONLY"';

key "idx";

min-elements 1;

leaf idx { type uint32 ; }

description "It specifies the area for which UE is requested to perform

measurement logging for neighbour cells which have list of frequencies.

If it is not configured, the UE shall perform measurement logging for

all the neighbour cells.

Applicable only to NR Logged MDT.";

reference "Clause 5.10.263GPP TS 32.422.";

uses AreaConfigGrp;

}

grouping AreaConfigGrp {

description "Represents the AreaConfig dataType.

This <<dataType>> defines the area for which measurement logging should

be performed. It is described by a list of cells and a list of

frequencies.";

list freqInfo {

key arfcn;

min-elements 1;

max-elements 32;

description "It specifies the carrier frequency and bands used in

a cell.";

uses FreqInfoGrp ;

}

leaf-list pciList {

type uint32 {

range 0..1007;

}

min-elements 1;

max-elements 32;

description "List of neighbour cells subject for MDT scope.";

}

}

grouping FreqInfoGrp {

description "Represents the FreqInfo dataType.

This <<dataType>> defines the RF reference frequency and the frequency

operating bands used in a cell for a given direction (UL or DL) in FDD

or for both UL and DL directions in TDD";

leaf arfcn {

type uint32 {

range 0..3279165;

}

mandatory true;

description "RF Reference Frequency as defined in TS 38.104,

clause 5.4.2.1. The frequency provided identifies the absolute

frequency position of the reference resource block (Common RB 0)

of the carrier. Its lowest subcarrier is also known as Point A.";

}

leaf-list freqBands {

type uint32 {

range 1..1024;

}

min-elements 1;

description "List of NR frequency operating bands. Primary NR

Operating Band as defined in TS 38.104, clause 5.4.2.3.

The value 1 corresponds to n1, value 2 corresponds to NR operating

band n2, etc.";

}

}

list areaScope {

description "it specifies the area where data shall be collected.";

key idx;

min-elements 0;

max-elements 1;

leaf idx {type string; }

uses AreaScopeGrp;

}

leaf collectionPeriodRRMLTE {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum 100ms;

enum 1000ms;

enum 1024ms;

enum 1280ms;

enum 2048ms;

enum 2560ms;

enum 5120ms;

enum 10000ms;

enum 10240ms;

enum 1min;

}

description "Specifies the collection period for collecting RRM configured

measurement samples for M3 in LTE. The attribute is applicable only

for Immediate MDT. In case this attribute is not used, it carries a

null semantic.";

reference "Clause 5.10.20 of 3GPP TS 32.422";

}

leaf CollectionPeriodM6LTE {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1024|2048|5120|10240";

}

units milliseconds;

description "Specifies the collection period for the Packet Delay

measurement (M6) for MDT taken by the eNB. The attribute is applicable

only for Immediate MDT. In case this attribute is not used,

it carries a null semantic.";

reference "Clause 5.10.32 of  TS 32.422 ";

}

leaf collectionPeriodM7LTE {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint16 {

range 1..60 ;

}

description "It specifies the collection period for the Data Volume (M6)

and Throughput measurements (M7) for UMTS MDT taken by RNC. The

attribute is applicable only for Immediate MDT. In case this attribute

is not used, it carries a null semantic.";

reference "Clause 5.10.22 of  TS 32.422 .";

}

leaf collectionPeriodRRMUMTS {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "100|250|500|1000|2000|"

+"3000|4000|6000";

}

units milliseconds;

description "Specifies the collection period for collecting RRM configured

measurement samples for M3, M4, M5 in UMTS. The attribute is applicable

only for Immediate MDT. In case this attribute is not used, it carries

a null semantic";

reference "Clause 5.10.21 of 3GPP TS 32.422";

}

leaf collectionPeriodRRMNR {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1024|2048|5120|10240|60000";

}

units milliseconds;

description "Specifies the collection period for collecting RRM

configured measurement samples for M4, M5 in NR. The attribute is

applicable only for Immediate MDT. In case this attribute is not

used, it carries a null semantic.";

reference "Clause 5.10.30 of 3GPP TS 32.422";

}

leaf collectionPeriodM6NR {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum 120ms;

enum 240ms;

enum 480ms;

enum 640ms;

enum 1024ms;

enum 2048ms;

enum 5120ms;

enum 10240ms;

enum 20480ms;

enum 40960ms;

enum 1min;

enum 6min;

enum 12min;

enum 30min;

}

description "It specifies the collection period for the Packet Delay

measurement (M6) for NR MDT taken by the gNB. The attribute is

applicable only for Immediate MDT. In case this attribute is not used,

it carries a null semantic.";

reference "clause 5.10.34 of  TS 32.422";

}

leaf collectionPeriodM7NR {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1..60";

}

description "It specifies the collection period for the Packet Loss Rate

measurement (M7) for NR MDT taken by the gNB. The attribute is

applicable only for Immediate MDT. In case this attribute is not used,

it carries a null semantic.";

reference "clause 5.10.35 of  TS 32.422";

}

leaf eventListForTriggeredMeasurement {

when '../jobType = "LOGGED_MDT_ONLY"';

type enumeration {

enum OUT_OF_COVERAGE ;

enum A2_EVENT ;

}

mandatory true;

description "Specifies event types for event triggered measurement in the

case of logged NR MDT.  Each trace session may configure at most one

event. The UE shall perform logging of measurements only upon certain

condition being fulfilled:

- Out of coverage.

- A2 event.";

reference "Clause 5.10.28 of 3GPP TS 32.422";

}

leaf eventThreshold {

type int64;

description "Specifies the threshold which should trigger the reporting

in case A2 event reporting in LTE or 1F/1l event in UMTS. The attribute

is applicable only for Immediate MDT and when reportingTrigger is

configured for A2 event in LTE or 1F event or 1l event in UMTS. In

case this attribute is not used, it carries a null semantic.";

reference "Clauses 5.10.7 and 5.10.7a of 3GPP TS 32.422";

}

leaf listOfMeasurements {

when '../jobType = "IMMEDIATE_MDT_ONLY"';

type int64;

mandatory true;

description "It specifies the UE measurements that shall be collected in

an Immediate MDT job. The attribute is applicable only for Immediate MDT.

In case this attribute is not used, it carries a null semantic.";

reference "Clause 5.10.3 of 3GPP TS 32.422.";

}

leaf loggingDuration {

when '../jobType = "LOGGED_MDT_ONLY" or'

+ ' ../jobType = "LOGGED_MBSFN_MDT"';

type uint32 {

range "600|1200|2400|3600|5400|7200";

}

units seconds;

mandatory true;

description "Specifies how long the MDT configuration is valid at the

UE in case of Logged MDT. The attribute is applicable only for

Logged MDT and Logged MBSFN MDT. In case this attribute is not used, it

carries a null semantic.";

reference "Clause 5.10.9 of 3GPP TS 32.422";

}

leaf loggingInterval {

when '../jobType = "LOGGED_MDT_ONLY" or'

+ ' ../jobType = "LOGGED_MBSFN_MDT"';

type enumeration {

enum 320ms;

enum 640ms;

enum 1280ms;

enum 2560ms;

enum 5120ms;

enum 10240ms;

enum 20480ms;

enum 30720ms;

enum 40960ms;

enum 61440ms;

enum INFINITY;

}

mandatory true;

description "Specifies the periodicity for Logged MDT. The attribute is

applicable only for Logged MDT and Logged MBSFN MDT. In case this

attribute is not used, it carries a null semantic";

reference "Clause 5.10.8 of 3GPP TS 32.422";

}

leaf eventThresholdL1 {

when '../jobType = "LOGGED_MDT_ONLY" or'

+ ' ../jobType = "LOGGED_MBSFN_MDT"';

type uint32 {

range "0..127";

}

description "It specifies the threshold which should trigger

the reporting in case of event based reporting of logged NR MDT.

The attribute is applicable only for Logged MDT and when reportType

is configured for event triggered reporting and when

eventListForTriggeredMeasurement is configured for L1 event.

In case this attribute is not used, it carries a null semantic.";

reference "clause 5.10.36 of TS 32.422";

}

leaf hysteresisL1 {

when '../jobType = "LOGGED_MDT_ONLY" or '

+ '../jobType = "LOGGED_MBSFN_MDT"';

type uint32 {

range "0..30";

}

description "It specifies the hysteresis used within the entry and leave

condition of the L1 event based reporting of logged NR MDT.

The attribute is applicable only for Logged MDT, when reportType

is configured for event triggered reporting and when

eventListForTriggeredMeasurement is configured for L1 event.

In case this attribute is not used, it carries a null semantic.";

reference "clause 5.10.37 of TS 32.422";

}

leaf timeToTriggerL1 {

when '../jobType = "LOGGED_MDT_ONLY" or '

+ '../jobType = "LOGGED_MBSFN_MDT"';

type int32 {

range 0|40|64|80|100|128|160|256|320|480|512|640|1024|1280|2560|5120;

}

units milliseconds;

description "It specifies the threshold which should trigger

the reporting in case of event based reporting of logged NR MDT.

The attribute is applicable only for Logged MDT, when reportType

is configured for event triggered reporting and when

eventListForTriggeredMeasurement is configured for L1 event.

In case this attribute is not used, it carries a null semantic.";

reference "Clause 5.10.38 of TS 32.422";

}

grouping MbsfnAreaGrp {

description "Represents the MbsfnArea dataType.

This <<dataType>> defines a MBSFN area. It is composed of the MBSFN Area

identifier and the carrier frequency (EARFCN).";

leaf mbsfnAreaId {

type uint32 {

range 1..max;

}

mandatory true;

description "MBSFN Area Identifier";

}

leaf earfcn{

type uint32 {

range 1..max;

}

mandatory true;

description "Carrier Frequency";

}

}

list mbsfnAreaList {

when '../jobType = "LOGGED_MBSFN_MDT"';

key "mbsfnAreaId earfcn";

max-elements 8;

description "The MBSFN Area consists of a MBSFN Area ID and Carrier

Frequency (EARFCN). The target MBSFN area List can have up to 8 entries.

This parameter is applicable only if the job type is Logged MBSFN MDT.";

reference "Clause 5.10.25 of 3GPP TS 32.422";

uses MbsfnAreaGrp;

}

leaf measurementPeriodLTE {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum 1024ms;

enum 2048ms;

enum 5120ms;

enum 10240ms;

enum 1min;

}

mandatory true;

description "It specifies the measurement period for the Data Volume (M4) and

Scheduled IP throughput measurements (M5) for LTE MDT taken by the eNB.

The attribute is applicable only for Immediate MDT. In case this

attribute is not used, it carries a null semantic.";

reference "Clause 5.10.23 of 3GPP TS 32.422";

}

leaf measurementPeriodUMTS {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+  'or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1000|2000|3000|4000|6000|8000|12000|16000|20000|"

+"24000|28000|32000|64000";

}

units milliseconds;

mandatory true;

description "It specifies the measurement period for the Data Volume (M6)

for MDT taken by RNC. The attribute is applicable only for Immediate MDT.

In case this attribute is not used, it carries a null semantic.";

reference "Clause 5.10.22 of 3GPP TS 32.422";

}

leaf measurementQuantity {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum CPICH_ECNO;

enum CPICH_RSCP;

enum PATHLOSS;

}

description "It specifies the measurements that are collected in an MDT

job for a UMTS MDT configured for event triggered reporting.";

reference "Clause 5.10.15 of 3GPP TS 32.422";

}

leaf eventThresholdUphUMTS {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint16 {

range 0..31 ;

}

description "It specifies the threshold which should trigger

the reporting in case of event-triggered periodic reporting for M4

(UE power headroom measurement) in UMTS. In case this attribute is

not used, it carries a null semantic.";

reference "Clause 5.10.39 of 3GPP TS 32.422.";

}

list pLMNList {

when '../jobType = "LOGGED_MDT_ONLY"';

key "mcc mnc";

uses types3gpp:PLMNId;

min-elements 1;

max-elements 16;

description "It indicates the PLMNs where measurement collection, status

indication and log reporting is allowed.";

reference "Clause 5.10.24 of 3GPP TS 32.422";

}

leaf positioningMethod {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum GNSS;

enum E-CELL_ID;

}

mandatory true;

description "It specifies what positioning method should be used in the

MDT job.";

reference "Clause 5.10.19 of 3GPP TS 32.422";

}

leaf reportAmount {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' and ../reportingTrigger = "PERIODICAL"';

type enumeration {

enum 1ms;

enum 4ms;

enum 8ms;

enum 16ms;

enum 32ms;

enum 64ms;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected.

The attribute is applicable only for Immediate MDT and when

MDTReportingTrigger is configured for periodical measurements. In

case this attribute is not used, it carries a null semantic.";

reference "Clause 5.10.6 of 3GPP TS 32.422";

}

leaf reportingTrigger {

when '../jobType = "IMMEDIATE_MDT_ONLY"';

type enumeration {

enum PERIODICAL;

enum A2_FOR_LTE;

enum 1F_FOR_UMTS;

enum 1I_FOR_UMTS_MCPS_TDD;

enum A2_TRIGGERED_PERIODIC_FOR_LTE;

enum ALL_CONFIGURED_RRM_FOR_LTE;

enum ALL_CONFIGURED_RRM_FOR_UMTS;

}

description "It specifies whether periodic or event based measurements

should be collected.

The attribute is applicable only for Immediate MDT and when the

MDTListOfMeasurements is configured for M1 (for both UMTS and LTE)

or M2 (only for UMTS). In case this attribute is not used, it carries

a null semantic.";

reference "Clause 5.10.4 of 3GPP TS 32.422";

}

leaf reportInterval {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' and ../reportingTrigger = "PERIODICAL"';

type uint32 {

range "120|240|250|480|500|640|1000|1024|2000|2048|3000|4000|"

+"5120|6000|8000|10240|12000|16000|20000|"

+"24000|28000|32000|60000|64000|"

+"360000|720000|1800000|3600000";

}

units milliseconds;

mandatory true;

description "It specifies the interval between the periodical measurements

that shall be taken when the UE is in connected mode.

The attribute is applicable only for Immediate MDT and when

reportingTrigger is configured for periodical measurements. In case

this attribute is not used, it carries a null semantic.";

reference "Clause 5.10.5 of 3GPP TS 32.422";

}

leaf reportType {

when '../jobType = "LOGGED_MDT_ONLY"';

type enumeration {

enum PERIODICAL;

enum EVENT_TRIGGERED;

}

mandatory true;

description "It specifies report type for logged NR MDT";

reference "Clause 5.10.27 of 3GPP TS 32.422";

}

leaf sensorInformation {

type enumeration{

enum BAROMETRIC_PRESSURE;

enum UE_SPEED;

enum UE_ORIENTATION;

}

description "It specifies which sensor information shall be included in

logged NR MDT and immediate NR MDT measurement if they are available.

The following sensor measurement can be included or excluded for

the UE.";

reference "Clause 5.10.29 of 3GPP TS 32.422";

}

leaf traceCollectionEntityId {

when '../jobType = "LOGGED_MDT_ONLY" or '

+ '../jobType = "LOGGED_MBSFN_MDT"';

type uint8;

description "It specifies the TCE Id which is sent to the UE in

Logged MDT.";

reference "Clause 5.10.11 of 3GPP TS 32.422";

}

}

grouping TraceSubtree {

description "Contains classes that manage Tracing.

Should be used in all  classes (or classes inheriting from)

- SubNetwork

- ManagedElement

- ManagedFunction

If a YANG module wants to augment these classes/list/groupings they must

augment all user classes!";

list TraceJob {

description "Represents the Trace Control and Configuration parameters of a

particular Trace Job (see TS 32.421 and TS 32.422 for details).

It can be name-contained by SubNetwork, ManagedElement, ManagedFunction

or NetworkSliceSubnet.

To activate Trace Jobs, a MnS consumer has to create TraceJob object

instances on the MnS producer. A MnS consumer can activate a Trace Job

for another MnS consumer since it is not required the value of

traceCollectionEntityAddress or streamingTraceConsumerUri to be his

own.

When a MnS consumer wishes to deactivate a Trace Job, the MnS consumer

shall delete the corresponding TraceJob instance.

For details of management Trace Job activation/deactivation see clause

4.1.1.1.2 of TS 32.422.

The attribute traceReference specifies a globally unique ID and

identifies a Trace session. One Trace Session may be activated to

multiple Network Elements. The traceReference is populated by the

consumer that makes the request for a Trace Session.

The attribute tjJobType specifies the kind of data to collect.

Dependent on the selected type various parameters shall be available.

The attributes tjJobType, tjTraceReference,

tjTraceCollectionEntityAddress and tjTraceReportingFormat are

mandatory for all job types. If streaming reporting is selected

for tjTraceReportingFormat, tjStreamingTraceConsumerURI shall be

present additionally. The attribute tjPLMNTarget shall be present

if trace activation method is management based.

For the different job types the attributes are differentiated as follows:

- In case of TRACE_ONLY additionally the following attributes shall be

available: listOfNeTypes, traceDepth, traceTarget and

triggeringEvent.

For this case the optional attribute listOfInterfaces allows to

specify the interfaces to be recorded.

- In case of IMMEDIATE_MDT_ONLY additionally the following attributes

shall be available:

- traceTarget

-	anonymizationOfMDTData,

-	listOfMeasurements,

-	collectionPeriodRRMUMTS (conditional for M4 and M5 in UMTS),

-	measurementPeriodUMTS (conditional for M6 and M7 in UMTS),

-	collectionPeriodRRMLTE (conditional for M3 in LTE),

-	measurementPeriodLTE (conditional for M4 and M5 in LTE),

-	collectionPeriodM6LTE (conditional for M6 in LTE),

-	collectionPeriodM7LTE (conditional for M7 in LTE),

-	collectionPeriodRRMNR (conditional for M4 and M5 in NR),

-	collectionPeriodM6NR (conditional for M6 in NR),

-	collectionPeriodM7NR (conditional for M7 in NR),

-	beamLevelMeasurement (conditional for M1 in NR),

-	reportInterval (conditional for M1 in LTE or NR and M1/M2 in UMTS),

-	reportAmount (conditional for M1 in LTE or NR and M1/M2 in UMTS),

-	reportingTrigger (conditional for M1 in LTE or NR and M1/M2 in UMTS),

-	eventThreshold (conditional for A2 event reporting or A2 event triggered periodic reporting),

-	measurementQuantity (conditional for 1F event reporting).

-	excessPacketDelayThresholds (conditional for M6 UL measurement in NR).

For this case the attribute areaScope defines the area scope of MDT,

which is specified in clause 5.10.2 of TS 32.422. Additionally, the optional

attributes positioningMethod, sensorInformation allow to

specify the positioning methods to use or the sensor information to

include.

- In case of IMMEDIATE_MDT_AND_TRACE both additional attributes of

TRACE_ONLY and IMMEDIATE_MDT_ONLY shall apply.

- In case of LOGGED_MDT_ONLY additionally the following attributes

shall be available: traceTarget, MDTAnonymizationOfData,

MDTTraceCollectionEntityID, MDTLoggingInterval,

MDTLoggingDuration, MDTReportType,

MDTEventListForTriggeredMeasurements.

For this case the optional attribute areaScope defines the area scope of MDT,

which is specified in clause 5.10.2 of TS 32.422. Additionally, the optional

attribute plmnList allows to specify the PLMNs where measurement collection,

status indication and log reporting is allowed, the optional attribute

MDTAreaConfigurationForNeighCell allows to specify the area for

which UE is requested to perform measurements logging for neighbour

cells which have list of frequencies and the optional attribute

MDTSensorInformation allows to specify the sensor information to

include.

- In case of LOGGED_MBSFN_MDT additionally the following attributes

shall be available: MDTAnonymizationOfData, MDTLoggingInterval,

MDTLoggingDuration, MDTMBSFNAreaList.

Creation and deletion of TraceJob instances by MnS consumers is optional;

when not supported, the TraceJob instances may be created and deleted by

the system or be pre-installed.";

key id;

uses top3gpp:Top_Grp ;

container attributes {

uses TraceJobGrp ;

}

uses files3gpp:FilesSubtree {

if-feature FilesUnderTraceJob;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 1 ***