3GPP TSG-SA5 Meeting #161 	S5-252265

Fukuoka City, Fukuoka, Japan, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.560 | CR | 0009 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.560 Corrections on STM NRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Monstra-OAM |  |  |  |  |  | Date: |  |  | 2025-05-01 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In SA5 meeting 160, a few agreed-upon CRs on TS 28.560 updated the STM NRM. Those CRs are S5-252085, S5-252087, and S5-252089. The above CRs update or define a few IOCs and data types. However, there are many overlapped attributes has been specified. This may create issues at STM configurations. This CR proposes to optimize the NRM definition to avoid too many overlapped attributes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Optimize the NRM definition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Duplicated NRM definition, risk for implementation problems would occur |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4, 6.2, 6.2.x (new), 6.3.1, 7.x(new), A.1 Forge MR link for YANG: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1698 at commit a51a08f827920588f06b2aa2bf428020d4658472 Forge MR link for YAML: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1699 at commit 3882bd3ad53b2fbfaed770b4b511267df470f186 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The agreed-upon CRs (S5-252085, S5-252087, S5-252089) are merged into this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.532: "Generic management services".

[3]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[4]	3GPP TS 33.501: " Security architecture and procedures for 5G System ".

[5]	3GPP TS 28.622: "Generic Network Resource Model (NRM); Integration Reference Point (IRP); Information Service (IS)".

[6]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[7]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[8]	IETF RFC8086: GRE-in-UDP Encapsulation.

[9]	IETF draft-ietf-opsawg-pcapng-04: PCAP Next Generation (pcapng) Capture File Format.

[10]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

[11]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[x]	3GPP TS 28.541: “Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3”

[y]	3GPP TS 23.273: “5G System (5GS) Location Services (LCS); Stage 2”

[z]		3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2"

*** START OF NEXT CHANGE ***

# 4	Signalling traffic monitoring management capabilities (stage 1)

The 3GPP management system shall have the capabilities as listed in the following table.

Table 4-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-SM-FUN-1 | The 3GPP system shall have the capability to send copies of the 5G Core control plane signalling messages, of selected interface(s) of selected NF(s), to an external entity that collects these signalling messages. | Signalling Traffic Monitoring Streaming |
| REQ-SM-FUN-2 | Authorised consumers shall have the capability to control (to configure, to enable, and to disable) sending copies of the 5G Core control plane signalling messages to the external collecting entity. | Signalling Monitoring Activation Signalling Monitoring Termination |
| REQ-SM-FUN-3 | The 3GPP system shall have the capability to send the copies of the 5G Core control plane signalling messages in a reliable or unreliable transport manner. | Signalling Traffic Monitoring Streaming |
| REQ-SM-FUN-4 | Authorised consumers shall have the capability to indicate the network interface type, and optionally the network interface instance and service operations within the specified network interface type, of which the signalling messages are to be monitored. | Signalling Traffic Monitoring Activation |

*** START OF NEXT CHANGE ***

### 6.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 3GPP TS 28.622 [5], IOC, SubNetwork | SubNetwork |
| 3GPP TS 28.622 [5], IOC, ManagedElement | ManagedElement |
| 3GPP TS 28.622 [5], IOC, ManagedFunction | ManagedFunction |

*** START OF NEXT CHANGE ***

## 6.2	Class definitions

### 6.2.1	StmCtrl <<IOC>>

#### 6.2.1.1	Definition

This IOC represents STM jobs. It can be name-contained by SubNetwork, ManagedElement, or ManagedFunction.

reportingNFList specifies the network function whose signalling traffic is to be monitored. If this parameter is not present or it is empty, then all Network Functions within the SubNetwork or ManagedElement shall be monitored. This parameter shall be omitted if the STM control object is specified under a ManagedFunction.

networkInterfaceList specifies the target network interface to be monitored. If this parameter is not present or it is empty, then all applicable interfaces from the target NF shall be monitored.

stmTargetUri is the URI of the STM consumer that shall receive the monitored signalling message copies.

administrativeState is used by the STM consumer to lock or unlock the StmCtrl instance in order to stop or start the signalling traffic monitoring.

operationalState is used by STM consumer to report its working state.

#### 6.2.1.2	Attributes

The StmCtrl IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [5] subclause 4.3.29) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| reportingNFList | CM | T | T | F | T |
| networkInterfaceList | M | T | T | F | T |
| stmTargetUri | M | T | T | T | T |
| administrativeState | M | T | T | F | T |
| operationalState | M | T | F | F | T |

#### 6.2.1.3	Attribute constraints

None.

#### 6.2.1.4	Notifications

The common notifications defined in 3GPP TS 28.622 [5] subclause 4.5 are valid for this IOC, without exceptions or additions.

*** START OF NEXT CHANGE ***

### 6.2.x	NetworkInterface <<Data Type>>

#### 6.2.1.1	Definition

This defines the network interface for which signalling traffic messages are to be copied and sent to the external entity.

The attribute networkInterfaceType specifies the network interface type to be monitored. The optional attribute networkInterfaceInstanceList specifies the network interface instances to be monitored. It is a list of applicable network interface instances of the network interface type specified by networkInterfaceType. The optional attribute serviceOperationList specifies the service operations to be monitored. It is a list of applicable service operations exchanged over the network interface instances specified by networkInterfaceInstanceList, or, if networkInterfaceInstanceList is not present, it is list of applicable service operations exchanged over the network interface type specified by networkInterfaceType.

#### 6.2.x.2	Attributes

The NetworkInterface data type contains the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| networkInterfaceType | M | T | T | F | T |
| networkInterfaceInstanceList | O | T | T | F | T |
| serviceOperationList | O | T | T | F | T |

#### 6.2.x.3	Attribute constraints

None.

#### 6.2.x.4	Notifications

The common notifications defined in 3GPP TS 28.622 [5] subclause 4.5 are valid for this IOC, without exceptions or additions.

*** START OF NEXT CHANGE ***

### 6.3.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| reportingNFList | List of Network Function Distinguished Name. allowedValues: N/A | Type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| networkInterfaceList | List of network interface to be monitored. | Type: NetworkInterface multiplicity: * isOrdered: False isUnique: True defaultValue: N/A isNullable: False |
| networkInterfaceType | The network interface type to be monitored. The applicable network interface type names are specified based on subclause 4.2.3 of 3GPP TS 23.501 [7] and clause 4.2.1 of 3GPP TS 23.273 [y]. The value "ALL" is specified for the case if all the applicable interface type of the network function shall be monitored. allowedValues: ALL, N2, N4, N5, N7, N8, N10, N11, N12, N13, N14, N15, N16, N17, N18, N20, N21, N22, N23, N26, N28, N29, N30, N33, N34, N35, N36, N37, N40, N41, N42, N51, N52, N58, N59, N60, N61, N62, N63, N80, N81, N82, N83, N84, N85, N86, N87, N88, N89, N96, NL1, NL2, NL5, NL6, NL8, NL9. | Type: ENUM multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: ALL isNullable: False |
| networkInterfaceInstanceList | The list of applicable network interface instances of the network interface type specified by networkInterfaceType. allowedValues: DN of MOIs of the following IOCs as specified in subclause 5.2.1 of 3GPP TS 28.541 [x]: EP_N2, EP_N4, EP_N5, EP_N7, EP_N8, EP_N10, EP_N11, EP_N12, EP_N13, EP_N14, EP_N15, EP_N16, EP_N17, EP_N18, EP_N20, EP_N21, EP_N22, EP_N23, EP_N26, EP_N28, EP_N29, EP_N30, EP_N33, EP_N34, EP_N35, EP_N36, EP_N37, EP_N40, EP_N41, EP_N42, EP_N51, EP_N52, EP_N58, EP_N59, EP_N60, EP_N61, EP_N62, EP_N63, EP_N80, EP_N81, EP_N82, EP_N83, EP_N84, EP_N85, EP_N86, EP_N87, EP_N88, EP_N89, EP_N96, EP_NL1, EP_NL2, EP_NL5, EP_NL6, EP_NL8, EP_NL9. | Type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| serviceOperationList | The list of applicable service operations exchanged over the network interface instances specified by networkInterfaceInstanceList. Or, If networkInterfaceInstanceList is not present, the list of applicable service operations exchanged over the network interface type specified by networkInterfaceType. allowedValues: refer to subclause 5.2 of 3GPP TS 23.502 [z]. | Type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| stmTargetUri | It specifies the Uniform Resource Identifier (URI) of the streaming target where the signalling traffic shall be sent. The detailed URI structure is defined in clause 4.4 of 3GPP TS 32.158 [6]. | type: Uri multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| administrativeState | Administrative state of a managed object instance. The administrative state describes the permission to use or prohibition against using the object instance. The administrative state is set by the STM consumer. allowedValues: LOCKED, UNLOCKED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |
| operationalState | Operational state of manged object instance. The operational state describes if an object instance is operable ("ENABLED") or inoperable ("DISABLED"). This state is set by the object instance or the STM producer and is hence READ-ONLY. allowedValues: ENABLED, DISABLED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLED isNullable: False |

*** START OF NEXT CHANGE ***

## 7.x	OpenAPI document for StmCtrl NRM

The OpenAPI/YAML definitions for StmCtrl NRM are specified in 3GPP Forge, refer to clause 4.3 of TS 28.623 [11] for the Forge location.

Directory: OpenAPI

File: TS28560_StmCtrl.yaml

*** START OF NEXT CHANGE ***

Annex A (informative):
Use Cases

# A.1 	Signalling Monitoring Activation

External monitoring systems are often used by mobile network operators (MNOs) to track network activity for analysis and troubleshooting purposes, and subsequently to perform diagnosis and analysis of their system. Such monitoring system(s) are fully under the control of the MNOs, and the monitoring is performed at signalling level.

The goal of this use case is to enable the authorized consumer (signalling monitor consumer) to activate the streaming of packets from a specific NF or entity acting on behalf of the NF (signalling monitor producer) for a list of network interface type and applicable interface instances and service operations. In this activation it will be requested the desired reliability for the streaming of the signalling traffic, along with the targeted interface(s).

*** START OF NEXT CHANGE ***

*** yang-models/_3gpp-5gc-nrm-stmfunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-stmfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-stmfunction;

prefix stm3gpp;

import _3gpp-common-top { prefix top3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the STM function defined in 3GPP TS 28.560.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.abc

Signalling traffic monitoring management";

revision 2025-05-07 { reference "CR-0009"; }

revision 2024-11-07 { reference "S5-247077"; }

grouping NetworkInterfaceGrp {

description "Represents the network interface for which signalling traffic

messages are to be copied and sent to the external entity.

The attribute networkInterfaceType specifies the network interface type to

be monitored. The optional attribute networkInterfaceInstanceList

specifies the network interface instances to be monitored. It is a list of

applicable network interface instances of the network interface type

specified by networkInterfaceType. The optional attribute

serviceOperationList specifies the service operations to be monitored.

It is a list of applicable service operations exchanged over the network

interface instances specified by networkInterfaceInstanceList, or, if

networkInterfaceInstanceList is not present, it is list of applicable

service operations exchanged over the network interface type specified

by networkInterfaceType.";

leaf networkInterfaceType {

type enumeration {

enum ALL;

enum N2;

enum N4;

enum N5;

enum N7;

enum N8;

enum N10;

enum N11;

enum N12;

enum N13;

enum N14;

enum N15;

enum N16;

enum N17;

enum N18;

enum N20;

enum N21;

enum N22;

enum N23;

enum N26;

enum N28;

enum N29;

enum N30;

enum N33;

enum N34;

enum N35;

enum N36;

enum N37;

enum N40;

enum N41;

enum N42;

enum N51;

enum N52;

enum N58;

enum N59;

enum N60;

enum N61;

enum N62;

enum N63;

enum N80;

enum N81;

enum N82;

enum N83;

enum N84;

enum N85;

enum N86;

enum N87;

enum N88;

enum N89;

enum N96;

enum NL1;

enum NL2;

enum NL5;

enum NL6;

enum NL8;

enum NL9;

}

default ALL;

description "List of network interface type to be monitored. The

applicable network interface type names are specified based on

subclause 4.2.3 of 3GPP TS 23.501 and clause 4.2.1 of 3GPP

TS 23.273.

The value ALL is specified for the case if all the applicable

interface type of the network function shall be monitored.";

reference "Clause 4.2.3 of 3GPP TS 23.501 and clause 4.2.1 of

3GPP TS 23.273 for details on the allowed values.";

}

leaf-list networkInterfaceInstanceList {

type types3gpp:DistinguishedName;

description "The list of applicable network interface instances

of the network interface type specified by

networkInterfaceType.";

reference "allowedValues: DN of the following MOIs as specified

in subclause 5.2.1 of 3GPP TS 28.541";

}

leaf-list serviceOperationList {

type string;

description "The list of applicable service operations exchanged

over the network interface instances specified by

networkInterfaceInstanceList. Or,

If networkInterfaceInstanceList is not present, the list of

applicable service operations exchanged over the network

interface type specified by networkInterfaceType.";

reference "allowedValues: refer to subclause 5.2 of 3GPP

TS 23.502";

}

}

grouping StmCtrlGrp {

description "Represents the StmCtrl IOC.";

leaf-list reportingNFList {

type types3gpp:DistinguishedName;

description "List of Network Function Distinguished Name, which

specifies the target network interface type to be monitored. If

this parameter is not present or it is empty, then all applicable

interface types from the target NF shall be monitored";

}

list networkInterfaceList {

uses NetworkInterfaceGrp;

key "networkInterfaceType";

description "List of network interface to be monitored. If this

parameter is not present or it is empty, then all Network Functions

within the SubNetwork or ManagedElement shall be monitored. This

parameter shall be omitted if the STM control object is specified

under a ManagedFunction. ";

}

leaf stmTargetUri {

type inet:uri;

mandatory true;

description "It specifies the Uniform Resource Identifier (URI) of the STM

consumer that shall receive the monitored signalling message copies ";

reference "Clause 4.4 of 3GPP TS 32.158";

}

leaf administrativeState {

default LOCKED;

type types3gpp:BasicAdministrativeState ;

description "It is used by the STM consumer to lock or unlock the

StmCtrl instance in order to stop or start the signalling traffic

monitoring";

}

leaf operationalState {

config false;

mandatory true;

type types3gpp:OperationalState ;

description "It is used by STM consumer to report its working state";

}

}

grouping StmCtrlSubTree {

description "Contains classes that manage Signalling traffic monitoring

management";

list StmCtrl  {

description "This IOC represents the STM Control and Configuration

parameters of a particular STM controlling. It can be

name-contained by SubNetwork, ManagedElement, or ManagedFunction.";

key id;

uses top3gpp:Top_Grp ;

container attributes {

uses StmCtrlGrp ;

}

}

}

}

<CODE ENDS>

*** START OF NEXT CHANGE ***

*** OpenAPI/TS28560_StmCtrl.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Signalling traffic monitoring management

version: 19.1.0

description: >-

OAS 3.0.1 specification of the StmCtrl NRM

Â© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.560; StmCtrl NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.560/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

NetworkInterface:

type: object

properties:

networkInterfaceType:

$ref: '#/components/schemas/NetworkInterfaceType'

serviceOperationList:

$ref: '#/components/schemas/ServiceOperationList'

networkInterfaceInstanceList:

$ref: '#/components/schemas/NetworkInterfaceInstanceList'

NetworkInterfaceType:

type: string

enum:

- ALL;

- N2;

- N4;

- N5;

- N7;

- N8;

- N10;

- N11;

- N12;

- N13;

- N14;

- N15;

- N16;

- N17;

- N18;

- N20;

- N21;

- N22;

- N23;

- N26;

- N28;

- N29;

- N30;

- N33;

- N34;

- N35;

- N36;

- N37;

- N40;

- N41;

- N42;

- N51;

- N52;

- N58;

- N59;

- N60;

- N61;

- N62;

- N63;

- N80;

- N81;

- N82;

- N83;

- N84;

- N85;

- N86;

- N87;

- N88;

- N89;

- N96;

- NL1;

- NL2;

- NL5;

- NL6;

- NL8;

- NL9;

NetworkInterfaceInstanceList:

type: array

uniqueItems: true

description: List of network interface instance

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

ServiceOperationList:

type: array

uniqueItems: true

description: List of service operations

items:

type: string

#-------- Definition of concrete IOCs --------------------------------------------

StmCtrl-Single:

type: object

properties:

reportingNFList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

networkInterfaceList:

$ref: '#/components/schemas/NetworkInterface'

stmTargetUri:

type: string

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/OperationalState'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

StmCtrl-Multiple:

type: array

items:

$ref: '#/components/schemas/StmCtrl-Single'

#--------------------------------- Definition ------------------------------------

resources-StmCtrl:

oneOf:

- $ref: '#/components/schemas/StmCtrl-Single'

<CODE ENDS>

*** END OF CHANGE ***