3GPP TSG-SA5 Meeting #161	S5-252270

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.404 | CR | 0022 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 32.404 PM Counter Metadata |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-04-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As discussed in SA5#159, the CR proposes a subbullet in the PM template with additional machine-readable information, such as Measurement Method |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of a subbullet in the PM template with additional machine-readable information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lacking of machine-readable information for a performance measurement |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

## 3.3	Measurement definition template

Following is the template used to describe the measurements contained in this subclause.

C.x.y. Measurement Name (clause header)

This is a descriptive name of the measurement type that is specified as clause C.x.y of the present document.

The measurement name shall be written in lower-case characters except abbreviations (e.g. RNC).

A measurement name can apply to one or more measurements. If the measurement name applies to several measurements then all fields of the template will take this into account.

Measurement names shall not exceed 64 characters in length and should be constrained to 32 characters maximum. Exceptions greater than 32 characters are allowed but should be kept to a minimum and only made where necessary.

a)	Description

This subclause contains an explanation of the measurement operation.

b)	Collection Method

This n contains the form in which this measurement data is obtained:

-	CC (Cumulative Counter);

-	GAUGE (dynamic variable), used when data being measured can vary up or down during the period of measurement;

-	DER (Discrete Event Registration), when data related to a particular event are captured every nth event is registered, where n can be 1 or larger;

-	SI (Status Inspection);

-	TF (Transparent Forwarding) ;

-	OM (Object Mapping).

b.1	)	Measurement Method

This subclause contains details on the methodology used for each counter to generate the measurements. The measurement methods used by the counters are:

- Sum: It represents the cumulative value of a specific metric (counter) over a given measurement period

- Average/Mean: It represents a value calculated over time or across multiple instances.

- Minimum: It represents the smallest or lowest values recorded over a given measurement period

- Maximum/Peak value: It represents the largest or highest value recorded over a given measurement period. Peak value may sometimes refer to a local maximum value over a given measurement period.

c)	Condition

This subclause contains the condition which causes the measurement result data to be updated;
This will be defined by identifying protocol related trigger events for starting and stopping measurement processes, or updating the current measurement result value. Where it is not possible to give a precise condition, then the conditional circumstances leading to the update are stated.

If a measurement is not available for FDD or TDD, then the measurement description shall contain a statement.

If a measurement is related to "external" technologies, this subclause shall give a brief reference to other standard bodies.

d)	Measurement Result (measured value(s), Units)

This subclause contains a description of expected result value(s) (e.g. a single integer value). If a measurement is related to "external" technologies, this subclause shall also give a brief reference to other standard bodies.

The definition applies for each measurement result.

e)	Measurement Type

This subclause contains a short form of the measurement name specified in the header, which is used to identify the measurement type in the result files.

The measurement names are dotted sequences of items. The sequence of elements identifying a measurement is organised from the general to the particular.

-	The first item identifies the measurement family (e.g. HHO, RAB, SMS). Note that this family may also be used for measurement administration purpose.

-	The second item identifies the name of the measurement itself, for which the following rules shall apply:

-	The second item of the measurement name can be divided into <Operation>, <Reason/Result> and <Direction> (and in that order)

-	Examples of Operation can be Establishment, Release, and Modification

-	Examples of Reason/Result can be Attempt, Failure, Success, Throughput and Volume

-	Examples of Direction can be Incoming, Outgoing, Uplink and Downlink

-	Depending on the measurement type, additional items may be present to specify subcounters (failure causes, traffic classes, min, max, avg, G, U ...). In case of multiple additional items, they are also represented as a dotted sequence of items. When available, the template will describe to which standard it is referring to for these additional items (e.g. cause, traffic class). Otherwise, the additional item semantics must be described in details in the present document.

Standardised causes will be a number.  This number shall be derived according to which of the following rules applies:

-	For the standardised causes with numeric values explicitly specified in the reference specification, the subcounter name will be the number assigned to this cause in this reference specification.

-	For the standardised causes without numeric values explicitly specified in the reference specification, but where the causes are identified, the subcounter name shall be a number from 1 to n mapped in an incremental sequence to each of the specified causes following top down sequence in the order they are identified in the reference specification (e.g. RRC.ConnEstab.1, 1 identifies the establishment cause " Originating Conversational Call", see TS 25.331 [30]);

-	For the standardised causes without numeric values explicitly specified in the reference specification and the causes identified and the causes have been divided into different cause groups, the subcounter should be defined as the form of ‘Cause group’.Cause, where:

-	the subcounter name of ‘Cause group’ shall be an incremental number from 1 to n to identify each cause group specified in a top down sequence following the order they are identified in the reference specification,

-	the subcounter name of cause within this cause group shall be an incremental number from 1 to n to identify each cause within the group specified in a top down sequence following the order they are identified within the cause group in the reference specification (e.g. RLM.FailRLSetupIub.2.1, 2.1 indenfies the cause " Transport resource unavailable" in the cause group " Transport Layer Cause", see TS 25.433 [29]). Subcounter ‘Cause group’.sum is permitted to identify the aggregate of measurement values of all the causes within the cause group (e.g. RLM.FailRLSetupIub.1.sum, 1.sum identifies the aggregate of all the causes within the cause group "Radio Network Layer Cause", see TS 25.433 [29]).

The non standardised causes should be a string (e.g. RRC.ConnEstab.NoReply).

The set of values issued for a measurement does not depend on the associated collection method (CC, SI, Gauge, DER, TF). For instance, a gauge collected counter does not necessarily provide min, max, average values.

The vendor-specific measurement names will all begin with the VS prefix.

In addition, it is recommended that a prefix is added for non-UMTS measurements:

-	Q3 for Q3 measurements;

-	MIB for IETF measurements (ATM, IP);

-	OS for other standards measurements.

The 3GPP standardised measurements name must not commence with the above prefixes.

Examples of valid measurement names are:

-	HO.PrepAtt

-	SAEB.EstabSucc

-	RRC.ConnEstabAtt.Cause
where Cause identifies the failure cause.

Examples of valid measurement names for measurements defined before version 8.3.0 of the present document are:

-	VS.HO.InterSGSNReject.NoResource;

-	ATM. ATML.IngressCells;

-	HHO.SuccOutIntraCell;

-	MM.AttachedSubs.Max;

-	RAB.EstabAttCS.Conversational;

-	RRC.ConnEstab.Cause
where Cause identifies the failure cause.

Abbreviations to be used within measurement types can be found in subclause 3.2 of the present document.

f)	Measurement Object Class

This subclause describes the measured object class (e.g. UtranCell, RncFunction, SgsnFunction). The object class used for this purpose shall be in accordance with NRMs, such as those defined in TS 28.622 [24], TS 28.702 [26] ,TS 28.652 [25] and TS 28.541 [24].

For object classes currently not defined according to the above, the present document defines its own nomenclature (e.g. RA, LAC).

It is possible to use the same measurement name for a standardized measurement type implemented at a different object class level than the one defined in the Standard. The same measurement type can apply to one or more measurements for which all fields of the measurement template are the same except the clause f) "Measurement Object Class". For instance, a measurement which uses the same template as a given measurement type but relates to another or different object classes (e.g. UtranCell instead of UtranRelation, or RncFunction and UtranCell) shall have the same name.

g)	Switching Technology

This subclause contains the Switching domain(s) this measurement is applicable to i.e. Circuit Switched and/or Packet Switched.

h)	Generation

The generation determines if it concerns a GSM, UMTS, EPS, 5GS, combined (GSM+UMTS+EPS+5GS) or IMS measurement.

-	GSM: pure GSM measurement; it only counts GSM events. In a combined (GSM+UMTS+EPS) NE the count would be exactly the same as in a pure GSM NE. In a pure UMTS, pure EPS or combined UMTS and EPS NE this counter does not exist;

-	UMTS: pure UMTS measurement; it only counts UMTS events. In a combined (GSM+UMTS+EPS) NE the count would be exactly the same as in a pure UMTS NE. In a pure GSM, pure EPS or combined GSM and EPS NE this counter does not exist;

-	EPS: pure EPS measurement; it only counts EPS events. In a combined (GSM+UMTS+EPS) NE the count would be exactly the same as in a pure EPS NE. In a pure GSM, pure UTMS or combined GSM and UTMS NE this counter does not exist;

- 	5GS: pure 5GS measurement; it only counts 5GS events. In a combined (GSM+UMTS+EPS+5GS) NE the count would be exactly the same as in a pure 5GS NF. In a pure GSM, pure UTMS, pure EPS or combined GSM, UTMS and EPS NE this counter does not exist;

-	GSM/UMTS: measurement applicable to both GSM and UMTS systems; in a combined (GSM+UMTS) NE separate subcounts for GSM and/or UMTS events can be obtained;

-	GSM/UMTS/EPS: measurement applicable to GSM, UMTS and EPS systems; in a combined (GSM+UMTS+EPS) NE separate subcounts for GSM, and/or UMTS, and/or EPS events can be obtained;

- 	EPS/5GS: measurement applicable to EPS and 5GS systems; in a combined (EPS+5GS) NE separate subcounts for EPS, and/or 5GS events can be obtained;

-	Combined: measurement applicable to combined GSM, UMTS, EPS and 5GS systems, but regardless of whether the measured event occurred on the GSM, UMTS, EPS or 5GS part of the system. This means that in a combined NE only one total (i.e. GSM+UMTS+EPS+5GS) count is obtained for the measured event.

The above aspects are also reflected in the measurement type name in template item E by adding a "G" to the GSM measurements, "U" to the UMTS measurements, "E" to the EPS measurements, "5G" for 5GS measurements.

-	IMS: measurement applicable to IMS.

NOTE:	The GSM component of combined GSM/UMTS equipment may actually choose to implement GSM measurements according to the present document or TS 52.402 [2], based on GSM specifications.

i)	Purpose

This optional clause aims at describing who will be using the measurement. It is proposed to indicate in this clause the targeted categories of users based on the measurement user communities described in Annex B.

When available, this clause provides additional information on the interest of the measurement but is however purely indicative.

*** END OF CHANGE ***