3GPP TSG-SA5 Meeting #161	S5-252315

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.310 | CR | 0079 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.310 Add solution on Energy saving optimization for multi-carrier RAN scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR proposes a solution for the use case and requirements on Energy saving optimization for multi-carrier RAN scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A procedure for energy savingoptimization for multi-carrier RAN scenarios is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no soluton for the agreed requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.1.X (New), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

##### 6.2.2.1.X	Energy saving optimization for multi-carrier RAN scenarios with several partially or fully overlaid capacity booster cells

Figure 6.2.2.1.X-1 depicts a procedure that describes how MnS producer of Domain Centralized ES management configures the energy saving function with characteristics that enable the optimization of energy saving.

Figure 6.2.2.1.X-1: Energy saving optimization for multi-carrier RAN scenarios

The MnS consumer for Domain-centralized ES configures the MnS producer with the area for which energy saving optimizations shall be computed.

The MnS consumer for Domain-centralized ES configures the cell-overlap grouping threshold, which is the threshold at which cell that overlap by more than that threshold are considered to be within the same ES group.

The MnS producer for Domain-centralized ES collects the traffic load performance measurements from all cells in the configured area – via the performance assurance MnS producer.

The MnS producer for Domain-centralized ES collects configuration data for all cells in the configured area – e.g., from the provisioning MnS producer.

The MnS producer for Domain-centralized ES computes degree of neigbourliness among cells, which  is the degree to which the cells overlap with one another.

The MnS producer for Domain-centralized ES computes minimum set of cells that provide full coverage for the area. These are marked as coverage cells and the rest marked as capacity booster cells.

The MnS producer for Domain-centralized ES consumes the provisioning MnS with modifyMOIAttributes operation to configure the computed capacity booster cells on to the ES function, indicating to the ES function the set of cells that can be deactivated for ES.

The MnS producer for Domain-centralized ES apply the configured cell-overlap grouping threshold to compute the cells whose degree of neigbourliness is higher than the cell-overlap grouping threshold and so belong to the same ES group. The ES group contains at least 1 coverage cell and 1 capacity booster cell. The ES group provides a matching of candidate cells to capacity booster cells, i.e., the best set of coverage cells to compensate for the coverage of the respective capacity booster cells.

The MnS producer for Domain-centralized ES consumes the provisioning MnS with modifyMOIAttributes operation to configure the computed ES group (i.e., the matching of candidate cells to capacity booster cells) on to the ES function.

The ES group may contain more than one capacity booster cell, so the ES function needs to be configured with the order/sequence in which the capacity booster cells are deactivated (i.e., sequence of entering energy saving state). The MnS producer for Domain-centralized ES computes the order of deactivating capacity booster cells for each ES group.

The MnS producer for Domain-centralized ES consumes the provisioning MnS with modifyMOIAttributes operation to configure the computed order of deactivating capacity booster cells for each ES group on to the ES function.

Next Change

# A.X	Energy saving optimization for multi-carrier RAN scenarios

@startuml

title Centralized Energy saving optimization for multi-carrier RAN

scenarios

skinparam Shadowing false

autonumber

skinparam monochrome true

participant "ES MnS Consumer,\n e.g. operator" as ESCons

participant "MnS Producer of\n Centralized ES" as ESProd

participant "provisioning\n MnS Producer" as ProvMnS

participant "Performance\n Assurance\n MnS Producer" as PerfMnS

participant "Centralized\n ES Function" as ESFun

Note over ESCons, ESFun: Relevant MOIs are created.

ESCons -> ESProd: configure area for\n ES saving optimizations

ESCons -> ESProd: configure cell-oveelap\n grouping threshold

ESProd <- PerfMnS: collects traffic load\n performance measurements

ESProd <- ProvMnS: collects the cell configuration data

ESProd <- ESProd: compute set of coverage cells,\n mark rest as capacity booster cells.

ESProd -> ProvMnS: configure capacity booster cells

ProvMnS -> ESFun: configure capacity booster cells

ESProd <- ESProd: compute degree of neigborliness among cells

ESProd <- ESProd: compute cells ES groups based on\n onfigured cell-oveelap grouping threshold

ESProd -> ProvMnS: configure ES groups

ProvMnS -> ESFun: configure ES groups

ESProd <- ESProd: compute matching of candidate cells\n to capacity booster cells

ESProd -> ProvMnS: configure matching of candidate cells\n to capacity booster cells

ProvMnS -> ESFun: configure matching of candidate cells\n to capacity booster cells

ESProd <- ESProd: compute capacity-booster cells\n deactivation order for each ES group

ESProd -> ProvMnS: configure capacity-booster cells\n deactivation order for each ES group

ProvMnS -> ESFun: configure capacity-booster cells\n deactivation order for each ES group

@enduml

End of change