3GPP TSG-SA5 Meeting #161	S5-252370

Fukuoka, Japan, 19 - 23 May 2025

Source:	Nokia

Title:	Pseudo-CR on General NRM for NDTs

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS28.561

Version:	0.2.0

Work Item:	Management aspects of Network Digital Twins

Comments

This pCR is to add NRM for the NDT  following agreements in the NDT study.

Proposed Changes

* * * First Change * * * *

## 6.2	Information model for NDT (MnS component typeB)

### 6.2.1	Information model definition

#### 6.2.1.1	Imported information entities and local labels

| 3GPP TS 28.622 [7], DataType, GeoArea | GeoArea |
| --- | --- |
| 3GPP TS 28.622 [7], DataType, TimeWindow | TimeWindow |

#### 6.2.1.2	Class diagram

##### 6.2.1.2.1	Relationship

Figure 6.2.1.2.1-1: Relationship UML diagram for NDT management

##### 6.2.1.2.2	Inheritance

Figure 6.2.1.2.2-1: Inheritance UML diagram for NDT management

#### 6.2.1.3	Class definition

Editor’s Note: The attributes of IOCs below are FFS.

##### 6.2.1.3.1	NDTFunction <<InformationObjectClass>>

###### 6.2.1.3.1.1	Definition

This IOC represents the properties of an NDT Function.

###### 6.2.1.3.1.2	Attributes

The NDTFunction IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.1.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedSimulationTypes | M | T | F | T | T |
| Attribute related to role |  |  |  |  |  |

###### 6.2.1.3.1.3	Attribute constraints

None.

###### 6.2.1.3.1.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC.

##### 6.2.1.3.2	NDTJob <<InformationObjectClass>>

###### 6.2.1.3.2.1	Definition

This IOC represents the properties of an NDT job demand created by an MnS consumer.

###### 6.2.1.3.2.2	Attributes

The NDTJob IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| simulationType | M | T | T | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtFunctionRef | M | T | T | F | T |
| ndtReportRefList | M | T | T | F | T |
| simulationScopeRef | M | T | T | F | T |

###### 6.2.1.3.2.3	Attribute constraints

None.

###### 6.2.1.3.2.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC.

##### 6.2.1.3.X	SimulationScope

###### 6.2.1.3.X.1	Definition

This IOC represents a single SimulationScope on an NDT .

A SimulationScope is name-contained on a NetworkDigitalTwin which runs simulation over that SimulationScope. A SimulationScope may be associated with zero, one or more Simulation(s).

###### 6.2.1.3.X.2	Attributes

The SimulationScope IOC includes attributes inherited from Top IOC (defined TS 28.622[7]) and the following attributes:

Table 6.3.3.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| scopeType | O | T | F | F | T |
| scopeList | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| NDTJobRef | M | T | T | F | T |

###### 6.2.1.3.X.3	Attribute constraints

None

###### 6.2.1.3.X.4	Notifications

None.

### 6.2.1.3.Y			ScopeDefinition <<choice>>

###### 6.2.1.3.Y.1	Definition

This <<choice>> represents a definition of a scope.

The managedEntitiesScope attribute identifies the scope by the DNs of the managed entities.

The areaScope attribute identify the scope by the geographical area information.

The managedEntitiesScope attribute and areaScope attribute shall not be present at the same time.

###### 6.2.1.3.Y.2	Attributes

Table 9.4.3.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Choice_1 managedEntitiesScope | CM | T | T | F | T |
| Choice_2 areaScope | CM | T | T | F | T |

###### 6.2.1.3.Y.3	Attribute constraints

Table 9.4.3.3-1

| Name | Definition |
| --- | --- |
| Choice_1 managedEntitiesScope | Condition: the MnS producer supports identifying the scope by managed entities. |
| Choice_2 areaScope | Condition: the MnS producer supports identifying the scope by geographical area information. |

###### 6.2.1.3.Y.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC.

##### 6.2.1.3.3	NDTReport <<InformationObjectClass>>

###### 6.2.1.3.3.1	Definition

This IOC represents the properties of an NDT report corresponding to an NDT job.

The attribute "PerformanceData" is specified performance metrics and/or alarm types that are collected and reported by NDT after the behaviour is modelled in NDT.

###### 6.2.1.3.3.2	Attributes

The NDTReport IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTOutputs | M | T | T | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtJobRef | M | T | T | F | T |
| ndtFunctionRef | M | T | T | F | T |

###### 6.2.1.3.3.3	Attribute constraints

None.

###### 6.2.1.3.3.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC.

##### 6.2.1.3.Z	NDTOutputPerTask <<dataType>>

###### 6.2.1.3.Z.1	Definition

This dataType represents the set of output data for a given input task in an nDTJob. It is a set of output

###### 6.2.1.3.Z.2	Attributes

The nDTOutputPerTask <<datatype>> includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.Z.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTTask | M | T | T | F | T |
| nDTOutputDataList | M | T | T | F | T |

###### 6.2.1.3.Z.3	Attribute constraints

None.

###### 6.2.1.3.Z.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC.

##### 6.2.1.3.Z	NDTOutputDataPoint <<dataType>>

###### 6.2.1.3.Z.1	Definition

This dataType represents a single output of the NDT.

###### 6.2.1.3.Z.2	Attributes

The nDTOutputDataPoint <<datatype>> includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.Z.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| networkState | M | T | T | F | T |
| networkConfiguration | M | T | T | F | T |
| networkImpact |  |  |  |  |  |

###### 6.2.1.3.Z.3	Attribute constraints

None.

###### 6.2.1.3.Z.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC.

## 6.3	Attribute definitions

### 6.3.1	Attribute properties

Table 6.3.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| ndtJobRef | It indicates an DN of a NDTJob Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtFunctionRef | It indicates an DN of a NDTFunction Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtReportRefList | It indicates a list of DN for NDTReport Instances. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedSimulationTypes | It indicates the different types of simulations which the NDT is capable of undertaking. | type: String multiplicity:1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| simulationType | It indicates the type of simulation that is desired to be executed. allowedValues: N/A | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| scopeList | It indicates the scope of the network that should be modelled by the NDT. | type: ScopeDefinition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTTask | It indicates a task triggered within the NDT for which an output is to be provided by the NDT Function instance. A specific NDT output is provided per nDTTask. | type: String multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTOutputDataList | It indicates the list of NDTOutputDataPoint(s) that are provided byte NDT function as the output for a given NDT Task | type: NDTOutputDataPoint multiplicity: 1 ..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTOutputs | It indicates the list of NDTOutput(s)corresponding to the different nDTTask(s) within an instantiated NDT job. | type: NDTOutputPerTask multiplicity: 1 ..* isOrdered: False isUnique: True defaultValue: None isNullable: False |

* * * Next Change * * * *

Annex A (informative):
PlantUML source code

## A.1	Information model definition for NDT

### A.1.1	Relationship UML diagram for NDT management

@startuml

hide empty members

skinparam ClassStereotypeFontStyle normal

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

'skinparam linetype ortho

'skinparam BoxPadding 40

skinparam nodesep 2

class ManagedEntity <<ProxyClass>>

class NDTFunction <<InformationObjectClass>>

class NDTJob <<InformationObjectClass>>

class SimulationScope <<InformationObjectClass>>

class NDTReport <<InformationObjectClass>>

note right of ManagedEntity

Represents SubNetwork

end note

ManagedEntity "1" *-- "*" NDTFunction: <<names>>

NDTFunction "1" --> "*" NDTFunction

NDTFunction "1" *-- "*" NDTJob: <<names>>

NDTFunction "1" *-- "*" SimulationScope: <<names>>

NDTFunction "1" *-- "*" NDTReport: <<names>>

NDTJob "1" -r-> "*" NDTReport

@enduml

### A.1.2	Inheritance UML diagram for NDT management

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class Top << InformationObjectClass >>

class ManagedFunction << InformationObjectClass >>

class NDTFunction <<InformationObjectClass>>

class NDTJob <<InformationObjectClass>>

class SimulationScope <<InformationObjectClass>>

class NDTReport <<InformationObjectClass>>

ManagedFunction <|-- NDTFunction

Top <|-- NDTJob

Top <|-- NDTReport

Top <|-- SimulationScope

@enduml

* * * End of Changes * * * *