3GPP TSG-SA5 Meeting #161	S5-252447

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.622 | CR | Input to DraftCR | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 Input to DraftCR TS 28.622 Miscellaneous Issues on Management Data Collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MADCOL_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Usage of attribute “jobId” is unclear. - Addressing Editors’ Notes. - Attribute value meaning is stated in attribute constraints subclause instead of in attribute definition subclause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove sentence “The attribute jobId is used to identify a management data collection request.” because we have already the DN as identifier for each “ManagementDataCollection” MOI. - Clarify meaning or usage of attribute “jobId”. - Remove Editor’s Note on one time reporting. The three reporting methods specified in attribute “ReportingCtrl” are equally supported for one time reporting. - Move sentence about relationship between historical management data and start time from constraints (4.3.48.3) to definition clause (4.3.48.2). - Editiorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Usage of attribute “jobId” is ambiguous. - Differences of one time reporting and “legacy” reporting is unclear. - Uncertainty of meaning of time value in the past in attributes “startTime” and “endTime”. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.47.1, 4.3.47.2, 4.3.48.1, 4.3.48.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 4.3.47	ManagementDataCollection

#### 4.3.47.1	Definition

This IOC represents a management data collection request job. The requested data is of kind Trace, MDT (Minimization of Drive Test), RLF (Radio Link Failure) report, RCEF (RRC Connection Establishment Failure) report, RRC report, PM (performance measurements), KPI (end-to-end key performance indicators) or a combination of these.

The attribute managementData defines the management data which shall be reported. This may either include a list of data categories or a list of management data identified with their name. For further details see clause 4.3.50.

The targetNodeFilter attribute can be used to target object instance(s) producing the required management data. It is assumed that the consumer may not have detailed knowledge of the network and hence may not identify the exact object instance producing the required management data. In this case consumer can request management data, specified by 3GPP, produced by certain object instance (s) based on a particular location, the domain (CN or RAN) of the object instances, and the handled traffic (CP or UP) of the object instances.

To activate the production of the requested data, a MnS consumer has to create a ManagementDataCollection object instance on the MnS producer.

The production and reporting of the management data can be constrained by conditions such that only when the conditions are satisfied shall management data collection be enabled. For example, a MnS consumer can request to create two ManagementDataCollection instances. One can be configured with high data producing and reporting period on a set of conditions (e.g, to reduce transmission cost when network performance metric is in normal range). Another can be configured with low data producing and reporting period on another set of conditions (e.g. to enable network optimization when network performance metric is in abnormal range).

Editor’s Note: It is currently not possible to construct conditions based on performance metrics. This needs to be enabled before the text in the paragraph above can be approved and published. Furthermore, it needs to be investigated if the ConditionMonitor shall be used or if the conditions should be added to this IOC directly using an attribute.

The MnS producer may derive multiple jobs (PerfMetricJob, TraceJob) from a single ManagementDataCollection job for collecting the required management data. The attribute jobId is used to correlate the derived jobs which will have the same jobId value as the parent ManagementDataCollection job.

If the MnS producer receives the collected data from multiple sources, it will consolidate the data into a set of management data for reporting based on the value of the attribute consolidateOutput.

For consolidation of file-based management data the attribute consolidateOutput controls:

- 	True: the MnS Producer will combine the file output from jobs used to collect the required management data into a single output file as follows:

- 	File is in compressed format, i.e. zip.

-	File shall contain individual output files from each configured job

- 	Each file retains its original filename

-	Each file retains it original content

-	Consolidated filename uses naming convention defined in [27], clause 11.3.2.1.4 with the following:

-	<Type> is a combination of the management data types included in the consolidated output

-	<UniqueIdList> is omitted

-	False: the MnS Producer will not combine the output from jobs used to collect the required management data.  The MnS Consumer will receive separate output from the derived jobs.

Subject to the reporting method, the MnS Consumer may receive file related notifications.  When consolidated output is selected the MnS Producer will create file notifications for the consolidated files.  When consolidated output is not selected, the MnS Producer will create the notification subscriptions on behalf of the MnS Consumer and the MnS Consumer will receive notifications directly from the derived jobs.

The attribute collectionTimeWindow specifies the time window for which the management data should be reported. The attributes startTime and the endTime can be in the past, present or in the future.

The attribute reportingCtrl specifies the method and associated control parameters for reporting the produced management data to MnS consumers. Three methods are available: file-based reporting with selection of the file location by the MnS producer, file-based reporting with selection of the file location by the MnS consumer and stream-based reporting.

The attribute dataScope configures, whether the management data should be reported per S-NSSAI or per 5QI or per PLMN, if applicable.

The attribute processMonitor allows the MnS consumer to monitor the status of the management data collection represented by the object ManagementDataCollection.

The MnS producer indicates in the attribute progressStateInfo the state of the management data collection:

-	NOT_STARTED

- 	RUNNING

- 	CANCELING

and indicates in the attribute resultStateInfo:

-	FINISHED

-	FAILED

-	PARTIALLY_FAILED

-	CANCELLED

#### 4.3.47.2	Attributes

The ManagementDataCollection IOC includes the attributes inherited from Top IOC (defined in clause 4.3.29) and the following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| managementData | M | T | T | T | N/A |
| targetNodeFilter | M | T | T | T | N/A |
| collectionTimeWindow | M | T | T | T | N/A |
| reportingCtrl | M | T | T | T | N/A |
| dataScope | O | T | T | T | N/A |
| condition | O | T | T | T | N/A |
| processMonitor | O | T | F | F | T |
| consolidateOutput | M | T | T | T | T |
| jobId | M | T | T | T | T |

#### 4.3.47.3	Attribute constraints

None.

#### 4.3.47.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC. In addition, the following set of notifications is also valid.

| Name | S | Notes |
| --- | --- | --- |
| notifyFileReady | M | -- |
| notifyFilePreparationError | M | -- |

### 4.3.48	TimeWindow <<choice>>

#### 4.3.48.1	Definition

This choice defines a time window.

It is a choice between the control parameters required to define the time window as follows:

When startTime and endTime is present (CHOICE_1), the time window starts when startTime is reached and ends when endTime is reached.

When only the startTime attribute is present (CHOICE_2), the time window starts when startTime is reached and runs until deletion of the managed object instance including this timeWindow.

When only the endTime attribute is present (CHOICE_3), the time window starts when the managed object instance including this TimeWindow is created and ends when endTime is reached.

A startTime value in the past and/or a endTime value in the past indicate that historical management data is included.

#### 4.3.48.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| CHOICE_1.1 startTime | CM | T | T | T | T |
| CHOICE_1.2 endTime | CM | T | T | T | T |
| CHOICE_2.1 startTime | CM | T | T | T | T |
| CHOICE_3.1 endTime | CM | T | T | T | T |

#### 4.3.48.3	Attribute constraints

| Name | Definition |
| --- | --- |
| CHOICE_1.1 startTime CHOICE_1.2 endTime | These attributes shall be supported, when the MnS consumer configures the start and end time of the time window. These attributes are supported for ManagementDataCollection IOC. |
| CHOICE_2.1 startTime | This attribute shall be supported, if the MnS consumer indicates only the start time of a time window and the end time is defined by the deletion of the managed object instance. This attribute is not supported for ManagementDataCollection IOC. |
| CHOICE_3.1 endTime | This attribute shall be supported, if the MnS consumer indicates only the end time of a time window and the start time is defined by the creation of the managed object instance. This attribute is not supported for ManagementDataCollection IOC. |

#### 4.3.48.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

End of Changes