# 3GPP TSG-SA5 Meeting #161	S5-252506

# Fukuoka, Japan, 19 - 23 May 2025

# Source:	Huawei

# Title:	Discussion on performance measurements for MOCN charging

# Document for:	Discussion

Agenda Item:	7.4.12

1	Decision/action requested

To analyse the performance measurements for MOCN charging.

2	References

[1]	3GPP TS 32.130 Telecommunication management; Network sharing; Concepts and requirements

[2]	3GPP TS 36.314 Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2 – Measurements

[3]		3GPP TS 28.552 Management and orchestration; 5G performance measurements

[4]	3GPP TS 28.201 Charging management; Network slice performance and analytics charging in the 5G System (5GS); Stage 2

3	Rationale

The charging requirement and use cases for network sharing charging based on OA&M performance measurement is specified in clauses 5.1.3, 5.4.4, and 6.4.1 of TS 32.130 [1], with the objective “to support the cross-operator accounting based on data volume and QoS”.

The requirement, per clauses 5.1.3 of TS 32.130 [1], indicates that “The MOP shall be able to charge the POPs for the data volume used by POP’s users per selected QoS profile criteria via measurements defined for shared networks. The QoS profile criteria may include QCI Indicator, GBR Indicator, ARP Indicator. It shall be possible to differentiate between DL and UL.”

According to the clause 6.4.1 of TS 32.130 [1], the performance measurements for cross-operator accounting based on data volume and QoS, which is referenced to the clause 4.1.9 of TS 36.314 [2], includes the following:

-	Data volume for shared networks in DL: “Data Volume for shared networks in DL. Amount of PDCP SDU bits in downlink delivered from PDCP layer to RLC layer in a measurement period. The measurement is performed per configured PLMN ID per configured DL QoS profile criteria. The unit is Mbit.”

-	Data volume for shared networks in UL: “Data Volume for shared networks in UL. Amount of PDCP SDU bits successfully received by the eNB in uplink in a measurement period. The measurement is performed per configured PLMN ID per configured UL QoS profile criteria. The unit is Mbit.”

However, the performance measurements defined in TS 36.314 [2] is “to measure the data volume transmitted or received by the eNB in a configured measurement period, for the different PLMN identities in a shared network.”

This is a misalignment with the objective of the Rel-19 charging WI on CH_MOCN_NetShare.

Nevertheless, the 5G performance measurements, as defined in clause 5.1.2.1 of TS 28.552 [3], could possibly being used as the MOCN performance measurement for gNB, with the following:

-	DL Cell PDCP SDU Data Volume: “This measurement provides the Data Volume (amount of PDCP SDU bits) in the downlink delivered to PDCP layer. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI. The unit is Mbit.”

-	UL Cell PDCP SDU Data Volume: “This measurement provides the Data Volume (amount of PDCP SDU bits) in the uplink delivered from PDCP layer to higher layers. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI. The unit is Mbit.”

Since the performance measurement provided by MnS Producer are all based on per S-NSSAI level as well, it is recommended to reuse the charging architecture, requirement, procedure and information that has specified in TS 28.201 [4], with some adaptation. For example, the MOCN performance measurement can be extended to the NSPA container information in Table 6.2.1.3-1 of TS 28.201 [4] as following:

Table 6.2.1.3-1: Structure of NSPA Container Information

| Information Element | Category | Description |
| --- | --- | --- |
| Uplink Latency | OC | This field holds uplink latency as described in TS 28.541 [252] clause 6.4 uLLatency attribute (see NOTE 1). |
| Downlink Latency | OC | This field holds downlink latency as described in TS 28.541 [252] clause 6.4 dLLatency attribute. |
| Uplink Throughput | OC | This field holds uplink throughput of one single network slice as described in TS 28.541 [252] clause 6.4 uLThptPerSlice attribute (see NOTE 2). |
| Downlink Throughput | OC | This field holds downlink throughput of one single network slice as described in TS 28.541 [252] clause 6.4 dLThptPerSlice attribute |
| Maximum packet loss rate UL | OC | This field holds maximum packet loss rate uplink as described in TS 28.541 [252] clause 5.4 maxPacketLossRateUl attribute (see NOTE 3). |
| Maximum packet loss rate DL | OC | This field holds maximum packet loss rate downlink as described in TS 28.541 [252] clause 5.4 maxPacketLossRateDl attribute. |
| Service Experience statistics data | OC | This field holds service experience statistics data as described in TS 23.288 [150] |
| Number of PDU sessions | OC | This field holds the number of PDU sessions as described in TS 28.554 [271]. |
| Number of registered Subscribers | OC | This field holds the number of registered subscribers as described in TS 28.554 [271]. |
| Load level | OC | This field holds the load level as described in TS 23.288 [150]. |
| Estimated Energy Consumption | OC | This field holds the KPI that describe the estimated energy consumption of one single network slice during the measured period, as described in TS 28.554 [271] clause 6.7.3.3. |
| MOCN Info | OC | This field holds the MOCN performance measurements for cross-operator accounting based on data volume and QoS, as described in TS 32.130 [x] clause 6.4.1. Each measurement is calculated per PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI, as described in TS 28.552 [y] clause 5.1.2.1. |
| QoS Level | M | This field holds the QoS Level mapped to 5QI. |
| DL Data Volume | OC | This field holds the data volume (amount of PDCP SDU bits) in the downlink delivered to PDCP layer. The unit is Mbit. |
| UL Data Volume | OC | This field holds the data volume (amount of PDCP SDU bits) in the uplink delivered from PDCP layer to higher layers. The unit is Mbit. |
| Note 1: For the back compatible, by default, the Latency holds the uplink latency. Note 2: For the back compatible, by default, the Throughput holds the uplink throughput. Note 3: For the back compatible, by default, the Maximum packet loss rate UL holds Maximum packet loss rate uplink. |  |  |

4	Detailed proposal

Proposal #1: A set of CR is proposed to specify the charging architecture and requirement in the TS 28.201 [4].

Proposal #2: Whether the above measurements in TS 28.552 [3] can be used as the MOCN performance charging information is to be discussed. The CR will be proposed in the next meeting.

Proposal #3: If the above proposal can be agreed, the affected TS in the WID proposal should also be updated, replacing TS 32.255 with TS 28.201.