3GPP TSG-SA5 Meeting #161	S5-252516

Fukuoka, Japan, 19 - 23 May 2025

Source:	ZTE Corporation

Title:	Pseudo-CR on TS 28.561 Add attributes on NDT NRM

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS 28.561

Version:	0.2.0

Work Item:	NDT

Comments

Define a generic NDT NRM to support NDT Lifecycle management.

Proposed Changes

Start of First Changes

6.2	Information model for NDT (MnS component typeB)

6.2.1	Information model definition

6.2.1.1	Imported information entities and local labels

| 3GPP TS 28.622 [7], DataType, GeoArea | GeoArea |
| --- | --- |
| 3GPP TS 28.622 [7], DataType, TimeWindow | TimeWindow |

6.2.1.2	Class diagram

6.2.1.2.1	Relationship

Figure 6.2.1.2.1-1: Relationship UML diagram for NDT management

6.2.1.2.2	Inheritance

Figure 6.2.1.2.2-1: Inheritance UML diagram for NDT management

6.2.1.3	Class definition

Editor’s Note: The attributes of IOCs below are FFS.

6.2.1.3.1	NDTFunction <<InformationObjectClass>>

6.2.1.3.1.1	Definition

This IOC represents the properties of an NDT Function.

6.2.1.3.1.2	Attributes

The NDTFunction IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedNDTCapabilities | M | T | T | F | F |

6.2.1.2.4.3	Attribute constraints

None.

6.2.1.2.4.4	Notifications

The common notifications defined in clause 6.2.1.5 are valid for this IOC.

6.2.1.3.2	NDTJob <<InformationObjectClass>>

6.2.1.3.2.1	Definition

This IOC represents the properties of an NDT job demand created by an MnS consumer.

6.2.1.3.2.2	Attributes

The NDTJob IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTCreationRequest | M | T | T | F | F |
| nDTConfigurationRequest | M | T | T | F | F |
| nDTRunningRequest | M | T | T | F | F |
| Attribute related roles |  |  |  |  |  |
| ndtFunctionRef | M | T | T | F | T |
| ndtReportRefList | M | T | T | F | T |

6.2.1.3.3	NDTReport <<InformationObjectClass>>

6.2.1.3.3.1	Definition

This IOC represents the properties of an NDT report corresponding to an NDT job.

The attribute "PerformanceData" is specified performance metrics and/or alarm types that are collected and reported by NDT after the behaviour is modelled in NDT.

6.2.1.3.3.2	Attributes

The NDTReport IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related roles |  |  |  |  |  |
| ndtJobRef | M | T | T | F | T |
| ndtFunctionRef | M | T | T | F | T |

6.2.1.4	DataType definition

6.2.1.4.1	NDTCreationRequest <<dataType>>

6.2.1.4.1.1	Definition

NDTCreationRequest <<dataType>> represents the request from the NDT MnS Consumer in the NDTJob instance creation, which may include orchestration and updating of the NDTJob instance.

nDTCapability indicates the capability that should be supported by the NDTJob instance to be created from the consumer’s perspective.

nDTCreationOp indicates the method to generate a NDTJob instance.

- When the value of nDTCreationOp is “Orchestration”, it means consumer requests the NDT MnS producer to create a NestedNDT based on existing NDTs.

- When the value of nDTCreationOp is “Updating”, it means consumer requests the NDT MnS producer to update the existing NestedNDT by replacing, adding, or removing the NDT componen.

nDTCreationOpInfo represents specific information for each operation in the NDTJob instance creation, e.g., the identifier of the managed NDT being operated on.

6.2.1.4.1.2	Attributes

The NDTCreationRequest <<dataType>> includes the following attributes.

Table 6.2.1.3.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTCapability | M | T | T | F | T |
| nDTCreationOp | M | T | T | F | T |
| nDTCreationOpInfo | M | T | T | F | T |

6.2.1.4.1.3	Attribute constraints

None.

6.2.1.4.2	NDTConfigurationRequest <<dataType>>

6.2.1.4.2.1	Definition

NDTConfigurationRequest <<dataType>> represents the request from the NDT MnS Consumer in the NDT configuration phase, which may include direct configuration for non-real time NDT and data collection & synchronization for real-time NDT.

6.2.1.4.2.2	Attributes

The NDTConfigurationRequest includes the following attributes.

Table 6.2.1.3.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| configurationObject | M | T | T | F | T |
| configurationValue | M | T | T | F | T |

6.2.1.4.2.3	Attribute constraints

None.

6.2.1.4.3	NDTRunningRequest <<dataType>>

6.2.1.4.3.1	Definition

NDTRunningRequest <<dataType>> represents the request from the NDT MnS Consumer in the NDT running phase.

6.2.1.4.3.2	Attributes

The NDTRunningRequest includes the following attributes.

Table 6.2.1.3.3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| startTime | M | T | T | F | T |
| stopTime | M | T | T | F | T |

6.2.1.4.3.3	Attribute constraints

None.

6.2.1.4	Attribute definitions

6.2.1.4.1	Attribute properties

Table 6.3.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| ndtJobRef | It indicates an DN of a NDTJob Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtFunctionRef | It indicates an DN of a NDTFunction Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtReportRefList | It indicates a list of DN for NDTReport Instances. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedNDTCapabilities | It indicates the specific NDT capabilities supported by the NDT function. AllowedValues: TBD. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTCreationRequest | It represents the NDT MnS Consumer’s requirements in the NDT Creation. allowedValues: N/A. | type: NDTCreationRequest multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTConfigurationRequest | It represents the NDT MnS Consumer’s requirements in the NDT Configuration. AllowedValues: N/A | type: NDTConfigurationRequest multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NDTCreationRequest.nDTRunningRequest | It represents the NDT MnS Consumer’s requirements in the NDT Running. AllowedValues: N/A. | type: NDTRunningRequest multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NDTCreationRequest.nDTCreationOp | It indicates the requested operation from the consumer in the NDT Creation. AllowedValues: Creation, Orchestration, Updating and Deletion. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NDTCreationRequest.nDTCreationOpInfo | It represents specific information for each operation in the NDTJob instance creation, i.e., the identifier of the managed object being operated on. When the nDTCreationOp is “Orchestration”, it includes a list of DN of NDT that need to be used to generate the NestedNDT. When the nDTCreationOp is “Updating”, it includes a list of DNs of NDT component that need to be updated. AllowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTCreationOp | It indicates the requested operation from the consumer in the NDT Creation phase. AllowedValues: Orchestration and Updating. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NDTConfigurationRequest.configurationObject | It indicates the DN of the managed object to be configured. AllowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NDTConfigurationRequest.configurationValue | It indicates the configuration value of the managed object to be configured. AllowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NDTRunningRequest.startTime | It indicates the start time of the NDT running requested by the MnS consumer. AllowedValues: N/A | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NDTRunningRequest.endTime | It indicates the start time of the NDT running requested by the MnS consumer. AllowedValues: N/A | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |