3GPP TSG-SA5 Meeting #161	S5-252631

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0385 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.532 Implement HTTP POST method for createMOI operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Regarding the RESTful HTTP-based solution set for generic provisioning management service, the createMOI operation is only implemented by HTTP method “PUT”, which means the id of the resource needs to assiged by consumer. However, in several scenarios, the id of the resource may be assigned by producer, especially for following scenaros: - Multiple MnFs register the MnSInfo to a centralized MnSRegistry. In this case, MnSRegistry is responbile for assigning id for the MnSInfo instance from different MnFs. - Multiple MnS consumer expresses intents to the same intent handling function. In this case, intent handling function can assign id for the Intent instance from different MnS consumers. Also in TS 32.158, clause 5.1.1 describes the detailed information for creating a resource with identifier creation by the MnS Producer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add HTTP POST method for createMOI operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.1.1.1, 12.1.1.1.2, 12.1.1.1.2.1(new), 12.1.1.1.2.2(new), 12.1.1.3.2.X(new) Following YAML files normatively defined in the forge are updated: - OpenAPI/TS28532_ProvMnS.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1739 at commit cefc5bcb44d52a5e172a65c99109789dcf0e91fc |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

12.1.1.1	Mapping of operations

12.1.1.1.1	Introduction

The IS operations are mapped to SS equivalents according to table 12.1.1.1.1-1.

Table 12.1.1.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | S |
| --- | --- | --- | --- |
| createMOI | PUT | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{className}={id} | M |
|  | POST | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part} | M |
| getMOIAttributes | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{className}={id} | M |
| modifyMOIAttributes | PUT PATCH | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{className}={id} | M |
| deleteMOI | DELETE | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{className}={id} | M |
| changeMOIs | PATCH | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{className}={id} | M |

12.1.1.1.2	Operation createMOI

12.1.1.1.2.1	Mapping to HTTP PUT

This operation creates a single resource representing a managed object instance if the identifier of the new resource is assigned by the MnS consumer.

Table 12.1.1.1.2.1-1: Mapping of IS operation input parameters to SS equivalents (HTTP PUT)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| managedObjectClass managedObjectInstance | path | …/{className}={id} | className: string id: string | M |
| attributeListIn | request body | n/a | Resource | M |

Note 1: Void.

Table 12.1.1.1.2.1-2: Mapping of IS operation output parameters to SS equivalents (HTTP PUT)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| attributeListOut | response body | n/a | Resource | M |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponseDefault | O |

Further details on creating a resource with HTTP PUT are provided in TS 32.158 [15], clause 5.1.2.

12.1.1.1.2.2	Mapping to HTTP POST

This operation creates a single resource representing a managed object instance if the identifier of the new resource needs to be assigned by the MnS producer.

Table 12.1.1.1.2.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| managedObjectClass managedObjectInstance | n/a | n/a | n/a | n/a |
| attributeListIn | request body | n/a | Resource | M |

Table 12.1.1.1.2.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP POST)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| attributeListOut | response body | n/a | Resource | M |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponseDefault | O |

Further details on creating a resource with HTTP POST are provided in TS 32.158 [15], clause 5.1.1.

Next change

12.1.1.3.2.X	Resource "…/{URI-LDN-first-part}"

12.1.1.3.2.X.1	Description

This resource represents a managed object instance if the identifier of the new resource is assigned by the MnS producer.

12.1.1.3.2.X.2	URI

Resource URI: {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}

The resource URI variables are defined in table 12.1.1.3.2.X.2-1.

Table 12.1.1.3.2.1.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.2 of TS 32.158 [15] |
| URI-LDN-first-part | See clause 4.4.2 of TS 32.158 [15] |

12.1.1.3.2.X.3	HTTP methods

12.1.1.3.2.X.3.1	HTTP POST

This method shall support the URI query parameters specified in the following table.

Table 12.1.1.3.2.X.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.1.1.3.2.X.3.1-2: Data structures supported by the POST request body on this resource

| Data type | Description | S |
| --- | --- | --- |
| Resource | Resource representation of the resource to be created | M |

Table 12.1.1.3.2.X.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| Resource | 201 Created | Status code returned when the resource is created. The representation of the created resource is returned in the response message body. | M |
| ErrorResponse | 4xx/5xx | Returned in case of an error | O |

Next change

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1739 at commit cefc5bcb44d52a5e172a65c99109789dcf0e91fc

*** START OF CHANGE 1 ***

*** OpenAPI/TS28532_ProvMnS.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Provisioning MnS

version: 19.0.0

description: >-

OAS 3.0.1 definition of the Provisioning MnS

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.532; Generic management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.532/

servers:

- url: '{MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}'

variables:

MnSRoot:

description: See clause 4.4.2 of TS 32.158

default: http://example.com/3GPPManagement

MnSVersion:

description: Version number of the OpenAPI definition

default: XXX

URI-LDN-first-part:

description: See clause 4.4.2 of TS 32.158

default: ''

paths:

'/{className}={id}':

parameters:

- name: className

in: path

required: true

schema:

type: string

- name: id

in: path

required: true

schema:

type: string

put:

summary: Replaces a complete single resource or creates it if it does not exist

description: >-

With HTTP PUT a complete resource is replaced or created if it does not

exist. The target resource is identified by the target URI.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

responses:

'200':

description: >-

Success case ("200 OK").

This status code shall be returned when the resource is replaced, and

when the replaced resource representation is not identical to the resource

representation in the request.

This status code may be returned when the resource is updated and when the

updated resource representation is identical to the resource representation

in the request.

The representation of the updated resource is returned in the response

message body.

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

'201':

description: >-

Success case ("201 Created").

This status code shall be returned when the resource is created.

The representation of the created resource is returned in the response

message body.

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

'204':

description: >-

Success case ("204 No Content").

This status code may be returned only when the replaced resource

representation is identical to the representation in the request.

The response has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponseDefault'

callbacks:

notifyMOICreation:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiCreation'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyMOIDeletion:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiDeletion'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyMOIAttributeValueChanges:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiAttributeValueChanges'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyEvent:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyEvent'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyMOIChanges:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiChanges'

application/yang-data+json:

schema:

$ref: '#/components/schemas/NotifyMoiChanges'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

get:

summary: Reads one or multiple resources

description: >-

With HTTP GET resources are read. The resources to be retrieved are

identified with the target URI. The attributes and fields parameter

of the query components allow to select the resource properties to be returned.

parameters:

- name: scope

in: query

description: >-

This parameter extends the set of targeted resources beyond the base

resource identified with the path component of the URI.

required: false

content:

application/json:

schema:

$ref: '#/components/schemas/Scope'

- name: filter

in: query

description: >-

This parameter reduces the targeted set of resources by applying a

filter to the scoped set of resource representations. Only resource

representations for which the filter construct evaluates to "true"

are targeted.

required: false

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/JexConditions'

- name: attributes

in: query

description: >-

This parameter specifies the attributes of the scoped resources that

are returned.

required: false

schema:

type: array

items:

type: string

style: form

explode: false

- name: fields

in: query

description: >-

This parameter specifies the attribute field of the scoped resources

that are returned.

required: false

schema:

type: array

items:

type: string

style: form

explode: false

- name: dataNodeSelector

in: query

description: >-

This parameter contains an expression allowing to conditionally

select data nodes.

required: false

schema:

oneOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/JexNodeSelectionBasic'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/JexNodeSelectionAdvanced'

responses:

'200':

description: >-

Success case ("200 OK").

The resources identified in the request for retrieval are returned

in the response message body. In case the attributes or fields query

parameters are used, only the selected attributes or sub-attributes are

returned. The response message body is constructed according to the

hierarchical response construction method (TS 32.158 [15]).

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

application/vnd.3gpp.object-tree-hierarchical+json:

schema:

$ref: '#/components/schemas/Resource'

application/vnd.3gpp.object-tree-flat+json:

schema:

type: array

items:

$ref: '#/components/schemas/Resource'

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponseGet'

patch:

summary: Patches one or multiple resources

description: >-

With HTTP PATCH resources are created, updated or deleted. The resources

to be modified are identified with the target URI (base resource) and

the patch document included in the request message body.

requestBody:

description: >-

The request body describes changes to be made to the target resources.

The following patch media types are available

- "application/merge-patch+json" (RFC 7396)

- "application/3gpp-merge-patch+json" (TS 32.158)

- "application/json-patch+json" (RFC 6902)

- "application/3gpp-json-patch+json" (TS 32.158)

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/Resource'

application/3gpp-merge-patch+json:

schema:

$ref: '#/components/schemas/Resource'

application/json-patch+json:

schema:

type: array

items:

$ref: '#/components/schemas/PatchItem'

application/3gpp-json-patch+json:

schema:

type: array

items:

$ref: '#/components/schemas/PatchItem'

responses:

'200':

description: >-

Success case ("200 OK").

This status code is returned when the updated the resource representations

shall be returned for some reason.

The resource representations are returned in the response message body. The

response message body is constructed according to the hierarchical response

construction method (TS 32.158 [15])

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

'204':

description: >-

Success case ("204 No Content").

This status code is returned when there is no need to return the updated

resource representations.

The response message body is empty.

default:

description: Error case.

content:

application/json:

schema:

oneOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponseDefault'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponsePatch'

delete:

summary: Deletes one resource

description: >-

With HTTP DELETE one resource is deleted. The resources to be deleted is

identified with the target URI.

responses:

'200':

description: >-

Success case ("200 OK").

This status code is returned, when the resource has been successfully deleted.

The response body is empty.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponseDefault'

'/{URI-LDN-first-part}':

parameters:

- name: URI-LDN-first-part

in: path

required: true

schema:

type: string

post:

summary: Creates a complete single resource

description: >-

With HTTP POST a complete single resource is created. The identifier of the new

resource is assigned by the MnS producer.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

responses:

'201':

description: >-

Success case ("201 Created").

This status code shall be returned when the resource is created.

The representation of the created resource is returned in the response

message body.

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponseDefault'

components:

schemas:

CmNotificationTypes:

type: string

enum:

- notifyMOICreation

- notifyMOIDeletion

- notifyMOIAttributeValueChanges

- notifyEvent

- notifyMOIChanges

SourceIndicator:

type: string

enum:

- RESOURCE_OPERATION

- MANAGEMENT_OPERATION

- SON_OPERATION

- UNKNOWN

ScopeType:

type: string

enum:

- BASE_ONLY

- BASE_NTH_LEVEL

- BASE_SUBTREE

- BASE_ALL

Operation:

type: string

enum:

- add

- remove

- replace

Insert:

type: string

enum:

- before

- after

PatchOperation:

type: string

enum:

- add

- replace

- remove

- copy

- move

- test

Resource:

oneOf:

- type: object

properties:

id:

type: string

objectClass:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

attributes:

type: object

additionalProperties:

type: array

items:

type: object

required:

- id

- $ref: 'TS28623_FeatureNrm.yaml#/components/schemas/resources-feature'

Scope:

type: object

properties:

scopeType:

$ref: '#/components/schemas/ScopeType'

scopeLevel:

type: integer

CorrelatedNotification:

type: object

properties:

source:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

notificationIds:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

required:

- source

- notificationIds

MoiChange:

type: object

properties:

notificationId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

op:

$ref: '#/components/schemas/Operation'

path:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

insert:

$ref: '#/components/schemas/Insert'

value: {}

oldValue: {}

required:

- notificationId

- op

- path

NotifyMoiCreation:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

attributeList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyMoiDeletion:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

attributeList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyMoiAttributeValueChanges:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

attributeListValueChanges:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeValueChangeSet'

required:

- attributeListValueChanges

NotifyEvent:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- specificProblem

properties:

specificProblem:

$ref: 'TS28111_FaultNrm.yaml#/components/schemas/SpecificProblem'

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyMoiChanges:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

moiChanges:

type: array

items:

$ref: '#/components/schemas/MoiChange'

required:

- moiChanges

PatchItem:

type: object

properties:

op:

$ref: '#/components/schemas/PatchOperation'

from:

type: string

path:

type: string

value:

nullable: true

required:

- op

- path

<CODE ENDS>

*** END OF CHANGE 1 ***

End of Changes