3GPP TSG-SA5 Meeting #161	S5-252722

Fukuoka, Japan, 19 - 23 May 2025

# Source:	Nokia

# Title:	Discussion on AI-ML UE side data collection

# Document for:	Endorsement

Agenda Item:	6.1

# 1	Decision/action requested

The group is requested to discuss and endorse detailed proposals.

# 2	References

[1]	R2-2503168 LS on AI/ML UE sided data collection

[2]	S5-250828 Reply LS on AI/ML UE sided data collection

[3]	RP-242389/S5-245373 LS on AIML data collection

# 3	Rationale

SA5 received LS response from RAN2 [1], which provides clarification to the AI/ML UE sided data collection based on the LS response from SA5 [2]. In the following sections, two options provided in LS received from RAN [3] are analysed according to the response received in [1].

# 4	Discussion

4.1			Option 3 UP tunnel

In option 3 UP tunnel there is a UP tunnel established between the UE and a collection entity / server. OAM is responsible to configure, control, monitor and implement the methods for the UE sided data collection. Meaning OAM provides inter alias the parameters regarding the server e.g. IP address or URI where to put the data on the server.

OAM being responsible to configure, control, monitor and implement methods for the UE sided data collection is the main difference to Option 1a (AI/ML-specific Data Transfer Path: UE to OTT server via either 3GPP or non-3GPP network) where the UP tunnel and the UE selection is triggered by the OTT server itself.

Observation 1:			There is a difference between Option 3 UP tunnel and Option 1a from SA5 point of view.

The below flow diagram provides an overview of the procedure to collect UE data according to option 3 UP tunnel.

1)	MnS Consumer sends a data collection request to Management system.

2)	Management system configures data collection to gNB(s) via MDT procedures.

3) 	The parameters for UP tunnel setup are sent to the UE for a network triggered UP connection setup. From SA5 perspective, this should be specified according to the trace activation procedure(details are ffs).

4)	UP tunnel is established between UE and a collection entity/server (similar to TCE).

5)	UE performs data collection based on information received from Management System/RAN (MDT like solution). This may require measurement configuration from RAN.

6)	UE sends the collected data to a collection entity/server directly via UP tunnel.

7)	Management System informs MnS Consumer where to fetch the collected data.

8) MnS consumer may fetch the collected data from the collection entity/server.

Proposal 1:  	From SA5 perspective, it is feasible to develop methods to configure, control, monitor and implement for the UE sided data collection for option 3 UP tunnel.

As stated above SA5 can provide the configuration parameters to set up the UP tunnel between a UE and a server. However, there might be security issues because the UE reports directly to the server. This might be a non-trusted network. Remember in Logged MDT, there is a TCE ID provided to the UE such that the security can be guaranteed. Therefore, we propose to include SA3 in the development of this solution

Proposal 2:	SA3 should be included in the discussion regarding security aspects for the UP tunnel between UE and server.

3.2			Option 2 UP tunnel

In option 2 the UP tunnel is established between the UE and a server within CN.

However, it is worth to mention, that in this option OAM is responsible to configure, control and monitor methods for the UE sided data collection including providing the parameters of the server (e.g IP address or URI). 5GC is responsible to implement the methods for the UE sided data collection.

The below flow diagram provides an overview of the procedure to collect UE data according to option 2 UP tunnel.

1)	MnS Consumer sends a data collection request to Management System.

2)	Management System sends data collection request to a function in 5GC.

3)	Function entity/entities in 5GC configure and set up UP tunnel between UE and server in 5GC (including selection of UEs and gNBs) Details are specified by SA2.

4)	UE performs data collection based on information received from 5GC. This may require measurement configuration from RAN.

5)	UE sends the collected data to the server in 5GC over established UP tunnel.

6)	Function entity in 5GC forwards the data to the MnS Consumer or provides information about where to fetch data.

Proposal 3:	From SA5 perspective, it is feasible to develop methods to configure, control and monitor for the UE sided data collection for option 2 UP tunnel.

Proposal 4:	Respond to RAN2 according to Proposal 1, Proposal 2 and Proposal 3.

# 4	Detailed proposal

Proposal 1:	FromSA5 perspective, it is feasible to develop methods to configure, control, monitor and implement for the UE sided data collection for option 3 over UP tunnel.

Proposal 2:	SA3 should be included in the discussion regarding security aspects for the UP tunnel between UE and server.

Proposal 3: 	From SA5 perspective, it is feasible to develop methods to configure, control, and monitor for the UE sided data collection for option 2 over UP tunnel.

Proposal 4:	Respond to RAN2 according to Proposal 1, Proposal 2 and Proposal 3.