3GPP TSG-SA5 Meeting #161	S5-252777

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.201 | CR | 0023 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.201 Addition of charging information for MOCN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH_MOCN_NetShare |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of charging information for MOCN network sharing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding charging information for MOCN network sharing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The charging information for MOCN network sharing will be missing in TS 28.201. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.1.1.2	Charging Data Request message

Table 6.1.1.2-1 illustrates the basic structure of a Charging Data Request message from the CEF as used for performance and analytics based charging.

Table 6.1.1.2-1: Charging Data Request message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Tenant Identifier | OM | This field if present is the identifier of subscriber of network slice and may in the case of 5G MOCN network sharing represent the participating operator, e.g. PLMN Id. |
| NF Consumer Identification | M | Described in TS 32.290 [57]. |
| NF Functionality | M | Described in TS 32.290 [57]. |
| NF Name | OC | Described in TS 32.290 [57]. |
| NF Address | OC | Described in TS 32.290 [57]. |
| NF PLMN ID | OC | Described in TS 32.290 [57]. |
| Invocation Timestamp | M | Described in TS 32.290 [57]. |
| Invocation Sequence Number | M | Described in TS 32.290 [57]. |
| One-time Event | OC | Described in TS 32.290 [57]. |
| One-time Event Type | OC | Described in TS 32.290 [57]. |
| Service Specification Information | OC | Described in TS 32.290 [57]. |
| Multiple Unit Usage | OC | Described in TS 32.290 [57], with the exception that quota management is not applicable. |
| Rating Group | M | Described in TS 32.290 [57] |
| Used Unit Container | OC | Described in TS 32.290 [57] |
| Local Sequence Number | OM | Described in TS 32.290 [57] |
| NSPA Container Information | OC | This field holds the network slice performance and analytics container specific information described in clause 6.2.1.3. |
| Network Sharing Container Information | OC | This field holds 5G MOCN network sharing container specific information described in clause 6.2.1.x. |
| NSPA Charging Information | OC | This field holds the network slice information, which is reported to the CHF described in clause 6.2.1.2. |
| Network Sharing Charging Information | OC | This field holds 5G MOCN network sharing specific information described in clause 6.2.1.y. |

* * * Next Change * * * *

#### 6.2.1.x	Definition of Network Sharing Charging Information

Specific charging information used for 5G MOCN network sharing charging is provided within the Network Sharing Charging Information.

The detailed structure of the Network Sharing Charging Information can be found in table 6.2.1.x.1.

Table 6.2.1.x.1: Structure of Network Sharing Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| PLMN Identifier | M | This field holds PLMN that the performance information is related to. |
| Single NSSAI | OC | This field holds network slice information the performance analytics information is related to. |

* * * Next Change * * * *

#### 6.2.1.y	Definition of Network Sharing Container Information

Specific charging information used for 5G MOCN network sharing charging is provided within the Network Sharing Container Information.

The detailed structure of the Network Sharing Container Information can be found in table 6.2.1.y.1.

Table 6.2.1.y.1: Structure of Network Sharing Container Information

| Information Element | Category | Description |
| --- | --- | --- |
| Downlink Data Volume | OC | This field holds downlink cell PDCP SDU Data Volume as described in clause 5.1.3.6.2 of TS 28.552 [270]. |
| Uplink Data Volume | OC | This field holds uplink cell PDCP SDU Data Volume as described in clause 5.1.3.6.2 of TS 28.552 [270]. |
| Number of PDU Sessions requested | OC | This field holds the number of PDU sessions requested by the gNB as described in clause 5.1.1.5.1 of TS 28.552 [270]. |
| Number of PDU Sessions successful | OC | This field holds the number of PDU sessions successfully setup by the gNB from AMF as described in clause 5.1.1.5.2 of TS 28.552 [270]. |

Editor’s Note:	Handing of QoS is FFS.

* * * End of Changes * * * *