3GPP TSG-SA5 Meeting #159	S5-251019

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025            revision of S5-250103

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.620 | CR | 0034 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | R19 CR 28.620 Change attribute properties |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The dnPrefix is often set by the consumer (management system). It should be isWritable=True The dnPrefix should also be isInvariant=True. The NRM contains many attributes that have the type DN. According to 32.300 the DN contains the dnPrefix. Changing the dnPrefix should result in changing all these attributes. Changing many dozens of attribute’s values as a side-effect when the dnPrefix is changed would not be feasible. Make attribute id isWritable=True. The id is set by the consumer, so it is writable. The description of locationName states it may contain no information, however its multiplicity is strictly 1. Making it mandatory to but alowing it to be an empty strings is bad practice. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Make the dnPrefix isWritable=True and isInvariant=True Make id isWritable=True. Change multiplicity of locationName |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interoperability problems due to dnPrefix and misunderstanding for id and locationName. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.2, 4.3.2.1, 4.3.2.2, 4.3.8.2, 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 4.3.1	Domain_

#### 4.3.1.1	Definition

This class groups managed entities:

-	Such that the group represents a topological structure which describes the potential for connectivity;

-	Subject to common administration;

-	With common characteristics.

A domain is a partition of instances of managed entities.

#### 4.3.1.2	Attributes

The Domain_ IOC includes attributes inherited from Top_ IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dnPrefix | M | M | - | - | M |
|  |  | T | T | T | T |
| userLabel | M | M | M | - | M |
|  |  | T | T | F | T |
| userDefinedNetworkType | M | M | M | - | M |
|  |  | T | T | F | T |

### 4.3.2	ManagedElement_

#### 4.3.2.1	Definition

This (and its contained Function_(s)) represents telecommunications resources (e.g. equipment) within the telecommunications network. This group performs Managed Element (ME) functions, e.g., provides support and/or service to the subscriber.

This can also provide access to a grouping of equipment for management purposes.

An ME communicates with a manager (directly or indirectly) for the purpose of being monitored and/or controlled. MEs may or may not additionally perform element management functionality.

An ME (and its contained Function_(s)) may or may not be geographically distributed. An ME (and its contained Function_(s)) is often referred to as a "Network Element".

#### 4.3.2.2	Attributes

The ManagedElement_ IOC includes attributes inherited from Top_ IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dnPrefix | M | M | - | - | M |
|  |  | T | T | T | T |
| managedElementTypeList | O | M | - | - | M |
|  |  | T | F | F | T |
| userLabel | M | M | M | - | M |
|  |  | T | T | F | T |
| locationName | M | M | - | - | M |
|  |  | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| managedBy | O | M | - | - | M |
|  |  | T | F | F | T |

Next change

### 4.3.8	Top_

#### 4.3.8.1	Definition

The Top_ is a logical construct representing the origin of definitions for all classes defined for the converged network management purposes. All other classes specified in this document and in other FNIM specifications must inherit from Top_ directly or indirectly.

#### 4.3.8.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | M | - | M | - |
|  |  | T | T | T | F |

Next change

## 6.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| direction | Represents the flow of traffic within the LT. allowedValues: The allowed values are:  Client-Server: Signal flows down the LT, e.g. traffic is taken from a number of low rate clients and multiplexed into a higher rate server.  Server-Client: Signal flows up the LT.  Bidirectional; Signal flow is both Client-Server and Server-Client. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dnPrefix | It carries the DN Prefix information or no information. See Annex C of 32.300 [2] for one usage of this attribute. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| id | An attribute whose class name and value can be used as an RDN when naming an instance of the object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. allowedValues: format of allowed values to be conformant with TS 32.300 [3]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| index | Provides any relevant indexing of the LT (channel number, e.g. ‘3’) allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| layerProtocolNameList | Name(s) and additional descriptive information such as version number for the protocol(s)/layer(s) used for the associated communication link. Syntax and semantic are not specified. allowedValues: allowed value examples: “X2AP”, “LR Optical Channel” | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| locationName | The physical location (e.g. an address) of an entity represented by a (derivative of) ManagedElement_. It may contain no information to support the case where the derivative of ManagedElement_ needs to represent a distributed multi-location NE. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ltType | The name of the specification that describes the internal construction of the LT, indicating for example that it possesses a G.805 CP but no G.805 TCP (see [11]). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| managedElementTypeList | It is a multi-valued attribute with one or more unique elements. Thus, it may represent one ME functionality or a combination of more than one functionality. The actual syntax and encoding of this attribute is Solution Set specific. allowedValues: 1) The allowed values of this attribute are the names of the IOC(s) that are (a) derived/subclassed from ManagedFunction and (b) directly name-contained by ManagedElement IOC (on the first level below ManagedElement), but with the string “Function” excluded. 2) If a ManagedElement contains multiple instances of a ManagedFunction this attribute will not contain repeated values. 3) The capitalisation (usage of upper/lower case) of characters in this attribute is insignificant. Thus, the NodeB should be case insensitive when reading these values. 4) Two examples of allowed values are:  NodeB;  HLR, VLR. | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| tpeType | The name of the specification that describes the construction of the TPE emphasising for example the access to the TPE and whether it is associated with a physical port directly or not (see [11]). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| userDefinedNetworkType | Textual information indicating network type, e.g. “UTRAN”. It may contain no information if there is no appropriate network type can be used. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| userLabel | A user-friendly (and user assignable) name of this object. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Attribute related to role |  |  |
| aEnd | The value of this attribute shall be a list of Distinguished Name of the alphabetically first instance in the Link subclass name to which this link/relation is associated (i.e., pointing to the instance of <X> as described in the definition of Link IOC in the present document). As an example, with Link_As_Slf, aEnd would contain the Distinguished Name of the AsFunction instance, and the zEnd would contain the Distinguished Name of SlfFunction instance. allowedValues: 1) For the instance whose class is defined by 3GPP, the format of the allowed values would be in conformant with that defined in TS 32.300 [3]. 2) See Note1. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False passedById: True |
| managedBy | This relates to the role played by ManagementSystem_ in the relation between ManagedSystem_ and ManagedElement_. This attribute contains a list of the DN(s) of the related subclasses of ManagementSystem_ instance(s). allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False passedById: True |
| managedElements | This relates to the role played by ManagedElement_ in the relation between ManagedSystem_ and ManagedElement_. This attribute contains a list of the DN(s) of the related subclasses of ManagedElement_ instance(s). allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False passedById: True |
| zEnd | The value of this attribute shall be a list of Distinguished Name of the alphabetically second instance in the Link subclass name to which this link/relation is associated (i.e., pointing to the instance of <Y> as described in the definition of Link IOC in the present document). As an example, with Link_As_Slf, aEnd would contain the Distinguished Name of the AsFunction instance, and the zEnd would contain the Distinguished Name of SlfFunction instance. allowedValues: 1) For the instance whose class is defined by 3GPP, the format of the allowed values would be in conformant with that defined in TS 32.300 [3]. 2)See Note1. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False passedById: True |
| Note 1: For the instance whose class is defined by TM Forum, the format of the allowed values would be in conformant with that defined in TM Forum MTOSI SD1-25_objectNaming [16]. |  |  |

End of  changes