3GPP TSG-SA5 Meeting #161	S5-252869

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0218 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.104 add stage3 solution for the specific use case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Samsung, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According the agreement discussion paper S5‑250822 in last meeting, we propose to add stage 3 solution for MDA. In this proposal, the contribution propose to add the solution for stage 3. In addition, for coverage problem analysis, the type definition of attribute “cellConfigurations” defined in clause 8.4.1.1.3 is not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the stage3 solution for the MDA output of mobility performance analysis, coverage problem analysis, Training Data Analysis,NF Scaling Dimensioning Data Analysis, PM Data Analysis, Failure Prediction Analysis. - Correct the definition of attribute “cellConfigurations”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The implementation of mobility performance anlaysis and coverage problem analysis will not be clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.1.3, OpenAPI/TS28104_MdaReport.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1721 at commit b4608cca39047b95ecaef6f4a0a864b9fa561ea6 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of next change

#### 8.4.1.1	Coverage problem analysis

##### 8.4.1.1.1	MDA type

The MDA type for coverage problem analysis is: CoverageAnalytics.CoverageProblemAnalysis.

##### 8.4.1.1.2	Enabling data

The enabling data for CoverageAnalytics.CoverageProblemAnalysis MDA type are provided in table 8.4.1.1.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.1.1.2-1: Enabling data for coverage problem analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | SS-RSRP distribution per SSB (beam) of serving NR cell | SS-RSRP distribution per SSB (clause 5.1.1.22.1 of TS 28.552 [4]). |
|  | SS-RSRP distribution per SSB (beam) of neighbor NR cell | SS-RSRP distribution per SSB of neighbor NR cell (clause 5.1.1.22.2 of TS 28.552 [4]) |
|  | RSRP distribution of neighbor E-UTRA cell for an NR cell | RSRP distribution per neighbor E-UTRAN cell (clause 5.1.1.22.3 of TS 28.552 [4]) |
|  | Power headroom distribution for NR cell | Type 1 power headroom distribution (clause 5.1.1.26.1 of TS 28.552 [4]). |
|  | Wideband CQI distribution for NR cell | Wideband CQI distribution (clause 5.1.1.11.1 of TS 28.552 [4]). |
|  | Timing Advance distribution for NR cell | Timing Advance distribution for NR Cell (clause 5.1.1.33.1 of TS 28.552 [4]) |
|  | Number of UE Context Release Request (gNB-DU initiated) | Number of UE Context Release Request (gNB-DU initiated) (clause 5.1.3.5.1 of TS 28.552 [4]). |
|  | Number of UE Context Release Request per SSB (gNB-DU initiated) | Number of UE Context Release Request (gNB-DU initiated) (clause 5.1.3.5.1 of TS 28.552 [4]). |
|  | Number of UE Context Release Requests (gNB-CU initiated) | Number of UE Context Release Request (gNB-CU initiated) (clause 5.1.3.5.2 of TS 28.552 [4]). |
|  | Number of UE Context Release Requests per SSB (gNB-CU initiated) | Number of UE Context Release Request (gNB-CU initiated) (clause 5.1.3.5.2 of TS 28.552 [4]). |
|  | RSRP related measurements for ng-eNB | RSRP related measurements (clause 6.1 of TS 32.425 [12]). |
|  | UE power headroom related measurements for ng-eNB | UE power headroom related measurements (clause 6.3 of TS 32.425 [12]). |
|  | Wideband CQI distribution for ng-eNB | Wideband CQI distribution (clause 4.10.1.1 of TS 32.425 [12]). |
|  | Average sub-band CQI for ng-eNB | Average sub-band CQI (clause 4.10.1.2 of TS 32.425 [12]). |
|  | UE Rx - Tx time difference related measurements for ng-eNB | UE Rx - Tx time difference related measurements (clause 6.4 of TS 32.425 [12]). |
|  | AOA related measurements for ng-eNB | AOA related measurements (clause 6.5 of TS 32.425 [12]). |
|  | Timing Advance distribution for ng-eNB | Timing Advance Distribution (clause 4.10.2 of TS 32.425 [12]). |
|  | Number of UE CONTEXT Release Request initiated by ng-eNodeB | Number of UE CONTEXT Release Request initiated by eNodeB/RN (clause 4.1.5.1 of TS 32.425 [12]). |
| MDT reports | MDT reports containing RSRPs of the serving cell and neighbour cells, and UE location. | RSRPs and UE location of M1 measurements for NR in TS 32.422 [6] and TS 32.423 [7]. |
| RLF reports | RLF reports containing RSRPs of the last serving cell and neighbour cells, and UE location. | RLF data collection and RLF reporting in TS 32.422 [6], and rlf-Report-r16 in TS 38.331 [13]. |
| RCEF reports | RCEF reports containing RSRPs of NR cell where the RRC connection establishment failed and neighbour cells, and UE location. | RCEF data collection and RCEF reporting in TS 32.422 [6], and ConnEstFailReport-r16 in TS 38.331 [13]. |
| UE location reports | UE location information provided by the LMF services which can be used to correlate with the MDT reports. | The UE location information provided by LMF via service-based interface (see TS 23.273 [14]). |
| Geographical data | The geographical information (longitude, latitude, altitude) of the deployed RAN (NG-RAN and E-UTRAN). | The geographical information (longitude, latitude, altitude) information (see the peeParametersList attribute of the ManagedFunction IOC in TS 28.622 [19]). |
| Configuration data | The NRMs containing the attributes affecting the coverage for (NG-RAN and E-UTRAN). | NRCellDU IOC, NRSectorCarrier IOC, BWP IOC, CommonBeamformingFunction IOC, and Beam IOC in TS 28.541 [15]; EUtranGenericCell IOC in TS 28.658 [16]; SectorEquipmentFunction IOC, AntennaFunction IOC, and TMAFunction IOC in TS 28.662 [17]. |

##### 8.4.1.1.3	Analytics output

The specific information elements of the analytics output for coverage problem analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.1.1.3-1.

Table 8.4.1.1.3-1: Analytics output for coverage problem analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| coverageProblemId | The identifier of the coverage problem. | M | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| coverageProblemType | Indication of type of the coverage Problem. allowedValues: WEAK_COVERAGE, COVERAGE_HOLE, PILOT_POLLUTION, OVERSHOOT_COVERAGE, DL_ULCHANNEL_COVERAGE_MISMATCH, OTHER. | M | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| coverageProblemAreas | Geographical location areas where the coverage problem occurred. | O | type: GeoArea (see TS 28.622 [19]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| problematicCells | The CGIs of cells where the coverage problem occurred. | M | type: Integer multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| recommendedActions | The recommended actions to solve the coverage problem. The recommended action may be (but not limited to): - creation of new beam(s), or cell(s); - change the transmission power of the NR sector carrier; - delete some unwanted beam(s) or cell(s). | M | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| radioEnvironmentMap | The graphical description of the observed radio coverage characteristics. The graphic may be for the RSRP or SINR of the selected cluster of cells mapped against the physical geographical information (longitude, latitude, altitude) of the area where the RAN (NG-RAN and E-UTRAN) cells are deployed. | O | type: RadioEnvironmentMap multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cellConfigurations | The cell configurations for a new cell or reconfigurations of existing cells derived based on the characteristics in the radioEnvironmentMap. The cell configurations are the changes to the NRMs attributes affecting the cell coverage (NG-RAN and E-UTRAN). Allowed values: attribute name and values as defined in one of the following: NRCellDU IOC, NRSectorCarrier IOC, BWP IOC, CommonBeamformingFunction IOC, and Beam IOC in TS 28.541 [15]; EUtranGenericCell IOC in TS 28.658 [16]; SectorEquipmentFunction IOC, AntennaFunction IOC, and TMAFunction IOC in TS 28.662 [17]. | O | type: AttributeValuePair multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

Next change

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1721 at commit b4608cca39047b95ecaef6f4a0a864b9fa561ea6

*** START OF CHANGE 1 ***

*** OpenAPI/TS28104_MdaReport.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: MDA Report

version: 19.1.0

description: >-

OAS 3.0.1 specification of the MDA Report

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.104; MDA Report

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.104/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

MDAOutputs:

type: object

properties:

mDAType:

$ref: 'TS28104_MdaNrm.yaml#/components/schemas/MDAType'

mDAOutputList:

oneOf:

- $ref: '#/components/schemas/CoverageProblemAnalysisoutput'

- $ref: '#/components/schemas/MobilityPerformanceAnalysisoutput'

- $ref: '#/components/schemas/TrainingDataAnalysisOutput'

- $ref: '#/components/schemas/NFScalingDimensioningDataAnalysisOutput'

- $ref: '#/components/schemas/PMDataOutput'

- $ref: '#/components/schemas/FailurePredictionOutput'

- type: array

uniqueItems: true

items:

$ref: '#/components/schemas/MDAOutputEntry'

analyticsWindow:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindowRo'

confidenceDegree:

type: number

format: float

MDAOutputEntry:

type: object

properties:

mDAOutputIEName:

type: string

readOnly: true

mDAOutputIEValue: {}

#-------- Definition of MDA Report --------------------------------------------

MDAReport:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

mDAReportID:

type: string

mDAOutputs:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/MDAOutputs'

mDARequestRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

#-------- Definition of recommended action --------------------------------------------

Recommended3GPPAction:

type: object

properties:

mOInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

path:

type: string

op:

type: string

enum:

- ADD

- REMOVE

- REPLACE

value:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

additionalText:

type: array

items:

type: string

RecommendedAction:

type: object

properties:

recommended3GPPActions:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Recommended3GPPAction'

recommendedNon3GPPActions:

type: array

uniqueItems: true

items:

type: string

recommendedHumanReadableActions:

type: array

uniqueItems: true

items:

type: string

actionInterval:

type: integer

timeWindow:

type: array

uniqueItems: false

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

#-------- Definition of specific MobilityPerformanceAnalysis MDAoutput -----------------------------

MobilityPerformanceAnalysisoutput:

type: object

properties:

mobilityPerformanceIssueIdentifier:

type: integer

mobilityPerformanceIssueRootCause:

type: string

enum:

- TooLongMobilityInterruptionTime

- PoorCoverageOfTheCelledge

- InappropriateHandoverParameters

- Other

mobilityPerformanceIssueLocation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

#-------- Definition of specific CoverageProblemAnalysis MDAoutput -----------------------------

CoverageProblemAnalysisoutput:

type: object

properties:

coverageProblemId:

type: string

coverageProblemType:

type: string

enum:

- WEAK_COVERAGE

- COVERAGE_HOLE

- PILOT_POLLUTION

- OVERSHOOT_COVERAGE

- DL_ULCHANNEL_COVERAGE_MISMATCH

- Other

coverageProblemAreas:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

problematicCells:

type: integer

recommendedActions:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/RecommendedAction'

radioEnvironmentMap:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/RadioEnvironmentMap'

cellConfigurations:

description: A map (list of key-value pairs)

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

RadioEnvironmentMap:

type: object

properties:

geoCoordinate:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

coverageCharacterization:

$ref: '#/components/schemas/CoverageCharacterization'

CoverageCharacterization:

type: object

properties:

rsrp:

type: number

format: float

sinr:

type: number

format: float

#-------- Definition of specific TrainingDataAnalysis MDAoutput -----------------------------

TrainingDataAnalysisOutput:

type: object

properties:

measurementDataCorrelationRecommendation:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/MeasurementDataCorrelationRecommendation'

MeasurementDataCorrelationRecommendation:

type: object

properties:

recommendedMeasurementDataToCollect:

type: array

uniqueItems: true

items:

type: string

recommendedMeasurementDataNotToCollect:

type: array

uniqueItems: true

items:

type: string

modelPerformanceImpact:

type: integer

minimum: 0

maximum: 100

#-------- Definition of specific NFScalingDimensioningDataAnalysis MDAoutput -----------------------------

NFScalingDimensioningDataAnalysisOutput:

type: object

properties:

recommendedActions:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/RecommendedAction'

#-------- Definition of specific PMData MDAoutput -----------------------------

PMDataOutput:

type: object

properties:

pmPredictions:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/PmPrediction'

minItems: 1

thresholdAssessment:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/ThresholdAssessment'

thresholdAdjustmentRecommendations:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/RecommendedAction'

PmPrediction:

type: object

properties:

pmName:

type: string

pmPredictedValue:

description: >-

This specifies the predicted value of the PM specified by “pmName”

attribute.

oneOf:

- type: integer

- type: number

format: float

thresholdAssessment:

$ref: '#/components/schemas/ThresholdAssessment'

ThresholdAssessment:

type: object

properties:

performanceMetrics:

description: >-

It indicates list of performance metrics with threshold

configuration issue.

Performance metrics include measurements defined in TS 28.552

and KPIs defined in TS 28.554.

For non-3GPP specified measurements the name is defined elsewhere.

type: array

uniqueItems: true

items:

type: string

timeWindow:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

confidenceScore:

type: number

format: float

#-------- Definition of specific FailurePrediction MDAoutput -----------------------------

FailurePredictionOutput:

type: object

properties:

failurePredictionObject:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

potentialFailureType:

type: string

potentialFailureCause:

oneOf:

- type: integer

- type: string

eventTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

issueID:

type: string

issueDomain:

type: string

enum:

- RAN_ISSUE

- CN_ISSUE

- UNKNOWN

perceivedSeverity:

type: string

enum:

- CRITICAL

- MAJOR

- MINOR

- WARNING

- INTERMEDIATE

- CLEARED

trendIndication:

type: string

enum:

- MORE_SEVERE

- NO_CHANGE

- LESS_SEVERE

predictedFailureEndTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

statisticsInfoList:

type: array

items:

type: integer

managementDataCollectionRecommendations:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/ManagementDataCollectionInfo'

recommendedActions:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/RecommendedAction'

ManagementDataCollectionInfo:

type: object

properties:

managementDataType:

type: string

enum:

- MEASUREMENT

- KPI

- TRACE_MDT

- QOE

managementData:

$ref: 'TS28623_ManagementDataCollectionNrm.yaml#/components/schemas/ManagementData'

targetEntities:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

collectionDuration:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

<CODE ENDS>

*** END OF CHANGE 1 ***

End of change