3GPP TSG-SA5 Meeting #161	S5-252873

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0340 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.312 Add procedure for intent negotiation during fulfilment phase |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2025-04-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The IntentFulfilmentNegotiationReport was introduced in TS 28.312 and updated in CR S5-251916 agreed in SA5#160 meeting. However, the procedure of for intent negotiation during fulfilment phase is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure for intent negotiation during fulfilment phase. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure for intent negotiation during fulfilment phase is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.X(new), A.1.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.312 CR 0308 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.3.X	Intent negotiation during fulfilment phase

Figure 6.3.X-1 illustrates the procedure for intent negotiation during fulfilment phase.

Figure 6.3.X-1 Procedure for intent negotiation during fulfilment phase

1. MnS Consumer sends a request to create or modify an intent instance (see createMOI operation or modifyMOIAttributes operation defined in TS 28.532 [3]) to MnS Producer, providing intent information for the new intent to be created. The detailed intent information sees attributes (attribute which "isWritable" property is "True") of the concrete intent IOC defined in clause 6.2. The value for attribute "intentMgmtPurpose" is "FULFILMENT_WITH_NEGOTIATION".

2. Based on the received request, the MnS Producer creates the concrete intent instance (i.e., instance of intent IOC) and configures the new created intent MOI with the received intent information.

3. MnS Producer sends a response (see createMOI operation or modifyMOIAttributes operation defined in TS 28.532[3]) to the MnS Consumer with attribute "objectInstance" of the created or modified intent instance.

4. Based on the created or modified intent, MnS Producer performs the management tasks to fulfil the intent and derives the possible outcomes.

5. MnS producer should notify (see notifyMOIAttributeValueChanges notification) MnS Consumer about attribute "objectInstance" of intent report instance and corresponding intentFulfilmentReport and intentFulfilmentNegotiationReport. Regarding the IntentReport, following are examples of different intent report information.

-	Example#1: Intent is fulfilled, MnS Producer recommend better outcomes to enable MnS consumer to decide whether to change the intent content.

IntentReport MOI: - intentFulfilmentReport - fulfilmentStatus = Fulfilled, list of targetAchievedValue=XXX - intentFulfilmentNegotiationReport - possibleOutcomesList= [fulfilmentStatus = Fulfilled, expectationFulfilmentResults]

-	Example#2: Intent is fulfilled, MnS producer could fulfil the with multiple ways and MnS producer provides the possible outcomes with possible impact to enable MnS consumer to select the preferred outcome.

IntentReport MOI: - intentFulfilmentReport - fulfilmentStatus = Fulfilled, list of targetAchievedValue=XXX - intentFulfilmentNegotiationReport - possibleOutcomesList= [fulfilmentStatus = Fulfilled, expectationFulfilmentResults, possibleImpact (O)]

-	Example#3: Intent is not fulfilled, MnS producer recommend fulfillable outcomes with possible impact to enable MnS consumer to decide whether to change the intent content or select the preferred outcome.

IntentReport MOI: - intentFulfilmentReport - fulfilmentStatus = notFulfilled, list of targetAchievedValue=XXX - intentFulfilmentNegotiationReport - possibleOutcomesList= [fulfilmentStatus = Fulfilled, expectationFulfilmentResults , possibleImpact(O)]

6.	MnS consumer decides to modify the intent or provide the feedback for fulfilment based on the received intentFulfilmentNegotiationReport.

Following step 7a-8a describe the sub-procedures for providing the feedback for fulfilment based on the received intentFulfilmentNegotiationReport

7a. MnS consumer sends a request to modify an IntentReport instance (see modifyMOIAttributes operation defined in TS 28.532 [3]) to MnS Producer with 'objectInstance' of the intentReport MOI and detailed values for ConsumerFeedback. The ConsumerFeedback includes preferedIntentOutcomeId.

8a. MnS producer modify the specified IntentReport instance with detailed values for ConsumerFeedback, and perform management tasks to fulfil the intent based on the specified ConsumerFeedback.

Following step 7b-8b describe the sub-procedures for modifying the intent based on the received intentFulfilmentNegotiationReport

7b. MnS consumer sends a request to modify an Intent instance (see modifyMOIAttributes operation defined in TS 28.532 [3]) to MnS Producer with 'objectInstance' of the intent MOI and the intent information to be modified.

8b. MnS producer modifies the specified intent instance and performs management tasks to fulfil the modified intent.

2nd Change

A.1.X	Intent inegotiation during fulfilment phase

title "[Intent Fulfilment Negotiation]"

actor "MnS Consumer" as MnS_Consumer

participant "MnS Producer" as MnS_Producer

Collections "ManagedEntities" as ManagedEntity

MnS_Consumer -> MnS_Producer: 1. Request to create/modify an intent instance \n (intentMgmtPurpose=FULFILMENT_WITH_NEGOTIATION, \n list of targets and contexts)

MnS_Producer -> MnS_Producer: 2. Create/modify and configure intent MOI

MnS_Producer -> MnS_Consumer: 3. Response for create/modify an intent instance

Loop For each observation period until the intentMgmtPurpose=FULFILMENT_WITH_NEGOTIATION udpated to FULFILMENT_WITHOUT_NEGOTIATION

Ref over MnS_Producer, ManagedEntity: 4. Perform the managment tasks to fulfil \n the intent  and derives the possible outcomes

MnS_Producer -> MnS_Consumer: 5. Notify of intent fulfilment report and fulfilment  \n negotiation report  (DN of IntentReport MOI, \n intentFulfilmentReport,  intentFulfilmentNegotiationReport)

MnS_Consumer  ->MnS_Consumer: 6. Decide to modify the intent or \n provide feedback for the intentFulfilmentNegotiationReport

Alt provide the feedback

MnS_Consumer  -> MnS_Producer: 7a. Request to configure the consumerFeedback \n of the IntentFulfilmentNegotiationReport \n of the IntentReport MOI

Ref over MnS_Producer, ManagedEntity: 8a.Fulfils the intent based on consumerFeedback

Else modify the intent

MnS_Consumer  -> MnS_Producer: 7b. Request to modify the Intent MOI with new target values

Ref over MnS_Producer, ManagedEntity: 8b. MnS producer modify the Intent MOI \n and fulfils the intent

End

hide footbox

@enduml

End of Changes