3GPP TSG-SA5 Meeting #161	S5-252887

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0354 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.312 Add description of intent negotiation initiated by MnS producer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is the additional solutions to agreed Tdoc S5-251916. Add description of intent negotiation initiated by MnS producer. In the IntentFulfilmentNegotiationReport, intent identifiers to be negotiated is provided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add description of intent negotiation initiated by MnS producer in 5.3.9.4. - Add "IntentFulfilmentNegotiationReport" in expectedReportTypes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Intent negotiation initiated by MnS producer will not be clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.9.4, 6.2.1.3.22, 6.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-252607. |  |  |  |  |  |  |  |  |

1st Change

#### 5.3.9.4	MnS producer to provide information about possible fulfilment of the intent

The MnS consumer wants and requests to know for a specific property what the MnS producer recommends what could be applied for particular intent characteristics, i.e. the MnS consumer wants to query MnS producer for recommendations for particular intent characteristics. The MnS producer has attempted to fulfil the intent and indicated that it cannot be fulfilled, so the MnS consumer asks the MnS producer to recommend what changes could be made to their intent or another of their intents to make the intent fulfillable. The MnS consumer can receive recommendations from MnS producer and use the response to modify their existing intents in an attempt to improve the ability to fulfil the new intents.

Figure 5.3.9.4-1: Enabling the MnS consumer to request and receive a recommendation on s
the possible fulfillable intent properties prior to or in case of failure of fulfilment.

Subsequently, the MnS producer provides a recommended-changes report indicating the information about possible changes that would enable fulfilment of the intent. The MnS producer should support a recommended-changes report that lists information about possible fulfilment of intent, e.g., candidate changes to unfulfillable targets within an unfulfillable expectation.

2nd Change

##### 6.2.1.3.16	IntentReportControl <<dataType>>

###### 6.2.1.3.16.1	Definition

This <<dataType>> describes intent report subscription information, including customized requirements on intent report.

The attribute "reportRecipientAddress" indicates the address of report recipient for MnS consumer.

The attribute "observationPeriod" indicates the time period for which the fulfilment process is observed and at the end of which the fulfilmentInfo for corresponding ExpectationTargets, IntentExpectations and Intent is updated. The observation period can be set by the MnS consumer or by the MnS producer if the MnS consumer does not provide a value.

The attribute "expectedReportTypes" indicates the type of IntentReports, which can be one/any/all of "IntentFulfilmentReport", "IntentConflictReport", "IntentFeasibilityCheckReport" and "IntentFulfilmentNegotiationReport".

The attribute " reportingConditions" indicates the specified conditions for intent reporting. TimeCondition is one choice for reportingCondition. For example, TimeCondition can be an interval, a specific time, or a time window. TagrtFulfilmentCondition in another choice for reportCondition. For example, the intent report needs to be sent by MnS producer when the achiveVlaue for specific target value crosses the pre-defined threshold.

The intent report will be sent to the MnS Consumer when the specified reportingConditions is satisfied

The attribute "reportingTargets" indicates the specified targets needed to be reported. All the targets described in the corresponding Intent instance need to be reported if the MnS consumer does not provide values for the attribute "reportingTargets".

###### 6.2.1.3.16.2	Attributes

Table 6.2.1.3.16.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| reportRecipientAddress | M | T | T | F | F |
| observationPeriod | M | T | T | F | F |
| expectedReportTypes | O | T | T | F | F |
| reportingConditions | O | T | T | F | F |
| reportingTargets | O | T | T | F | F |

###### 6.2.1.3.16.3	Attribute constrains

None.

3rd Change

#### 6.2.1.4	Attribute definition

Table 6.2.1.4-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentExpectations | It describes the expectations including requirements, goals and contexts (including constraints and filter information) given to a 3GPP system. It states the list of specific outcomes desired to be realized for expectation object(s). The intentExpectations are arranged in an ordered list such that the most important intentExpectations are on the top of the list. allowedValues: Not Applicable | type: IntentExpectation multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| intentFulfilmentInfo | It describes status of fulfilment of an intent and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| additionalFulfilmentInfo | It describes the additional information of intent intent fulfilment. The content and format is vendor specific. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationFulfilmentInfo | It describes status of fulfilment of an intentExpectation and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| targetFulfilmentInfo | It describes status of fulfilment of an expectationTarget and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fulfilmentStatus | It describes the current status of the fulfilment result for intent, intentExpectation or expectationTarget, which is configured by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", "NOT_FULFILLED" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "NOT_FULFILLED" isNullable: False |
| notFulfilledState | It describes the current state for not achieving fulfilment for the intent, intentExpectation or expectationTarget. It is configured/written by MnS producer and can be read by MnS consumer. allowedValues: "ACKNOWLEDGED", "COMPLIANT", "DEGRADED", "SUSPENDED", "TERMINATED" "FULFILMENTFAILED" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "ACKNOWLEDGED" isNullable: False |
| notFulfilledReasons | It describes the reasons/observations related to the specific notFulfilledState allowedValues: Not Applicable | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentContexts | It describes the list of IntentContext(s) which represents the constraints and conditions that should apply for the entire intent even if there may be specific contexts defined for specific parts of the intent. allowedValues: triple of (attribute, condition, value range) | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationId | A unique identifier of the intentExpectation within the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationVerb | It describes the characteristic of the intentExpectation and is the property that describes the types of intentExpectations. Examples of verbs and their related types of expectation are Deliver: DeliveryIntentExpectation, e.g. Deliver a RAN network, Service, Slice, function Ensure: AssuranceintentExpectation, e.g. Ensure the target performance value. Maintain: MaintenanceIntentExpectation, e.g. Maintain the network element according to a target version. allowedValues: DELIVER, ENSURE, MAINTAIN Vendor extensions are allowed | type: String multiplicity: 1 isOrdered:N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationObject | It describes the expectation objects to which the IntentExpectation should apply. allowedValues: Not Applicable | type: ExpectationObject multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectType | It describes the type of expectation object of the IntentExpectation that is required to be applied to. It can be class name of the managed object. allowedValues: see scenario specific IntentExpectation | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectInstance | It describes a specific object instance (e.g. instance of managed object) to which the intentExpectation should apply. allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectContexts | It describes the list of ObjectContext(s) which represents the constraints and conditions to be used as filter information to identify the object(s) to which a given intentExpectation should apply. Note there may be other constraints and conditions defined either for the entire intent, for the specific intentExpectation or for the expectationTarget of the considered intentExpectation. The concrete ObjectContext depends on the ExpectationObject, which is defined in clause 6.2.2. All the concrete ObjectContexts follow the common structure of ObjectContext. | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationTargets | It describes the list of ExpectationTarget(s) which represent specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation. The concrete ExpectationTarget depends on the ExpectationObject, which is defined in clause 6.2.2. All the concrete ExpectationTargets follow the common structure of ExpectationTarget. The expectionTargets are arranged in an ordered list such that the most important expectionTargets are on the top of the list. | type: ExpectationTarget multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| expectationContexts | It describes the list of context(s) which represents the constraints and conditions that should apply for a specific intentExpectation. Note there may be other constraints and conditions defined for the entire intent or for specific parts of the intentExpectation. allowedValues: depends on Expectation Object in the IntentExpectation | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetName | It describes the name of the expectation target which represents specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation. allowedValues: depends on ExpectationObject in the IntentExpectation | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| targetCondition | It expresses the limits within which the targetName is allowed/supposed to be. allowedValues: "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE", "IS_ONE_OF", " IS_EQUAL_TO_OR_LESS_THAN”, "IS_EQUAL_TO_OR_GREATER_THAN", "IS_NOT_ONE_OF", "IS_ALL_OF" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "IS_EQUAL_TO" isNullable: False |
| targetValueRange | It describes the range of values that applicable to the targetName and the targetCondition. allowedValues: depends on the targetCondition. The value will be a single value when the targetCondition is either "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS EQUAL TO OR LESS THAN", "IS EQUAL TO OR GREATER THAN” The value will be a pair of values when the targetCondition is either "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE" The value will be a list when the targetCondition is "IS_ONE_OF", "IS_NOT_ONE_OF","IS_ALL_OF". See NOTE 1. | type: ValueRangeType multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| targetContexts | It describes the list of constraints and conditions that should apply for a specific expectationTarget. Note there may be other constraints and conditions defined for the entire intent or the intentExpectation. allowedValues: Not Applicable | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| contextAttribute | It describes a specific attribute of or related to the object or to characteristics thereof (e.g. its control parameter, gauge, counter, KPI, weighted metric, etc) to which the expectation should apply or an attribute related to the operating conditions of the object (such as weather conditions, load conditions, etc). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| contextCondition | It expresses the limits within which the ContextAttribute is allowed/supposed to be allowedValues: "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE, "IS_ONE_OF", "IS_EQUAL_TO_OR LESS_THAN", "IS_EQUAL_TO_OR_GREATER_THAN", "IS_NOT_ONE_OF", "IS_ALL_OF" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "IS_EQUAL_TO" isNullable: False |
| contextValueRange | It describes the range of values that applicable to the ContextAttribute and the ContextCondition. AllowedValue: depends on the contextCondition The value will be a single value when the contextCondition is either "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_EQUAL_TO_OR_LESS_THAN", "IS_EQUAL_TO_OR_GREATER_THAN". The value will be a pair of values when the contextCondition is either "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE" The value will be a list when the contextCondition is "IS_ONE_OF", "IS_NOT_ONE_OF","IS_ALL_OF". See NOTE 1. | type: ValueRangeType multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| intentPriority | It expresses the priority of the stated intent within an MnS consumer. AllowedValue: values in the range [1-100] where 1 indicates the highest priority and 100 indicates the lowest priority. NOTE: The handing of the priorities across MnS consumers is left to implementation | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 1 isNullable: False |
| geoArea | It describes a geographical area defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: GeoArea multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| pLMNId | It describes the information of a PLMN identification defined in 3GPP 28.658[10] AllowedValue: As defined by the data type | type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dateTime | It describes the information of a date time defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| timeWindow | It describes the information of a time window (including startTime, endTime) defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| geoCoordinate | It describes the information of a geoCoordinate defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: GeoCoordinate multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| pLMNInfo | It describes the information of PLMNInfo (including PLMN and S-NSSAI in the PLMN in case of network slicing feature) defined in 3GPP TS 28.541[5]. AllowedValue: As defined by the data type | type: PLMNInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| schedulingTime | It describes the information of SchedulingTime (including one-time interval, daily periodicity, weekly periodicity or monthly periodicity) defined in 3GPP TS 28.622 [6]. | type: SchedulingTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| frequency | It desribes the RF reference frequency (i.e. Absolute Radio Frequency Channel Number) and/or the frequency operating band used for a given direction (UL or DL) in FDD or for both UL and DL directions in TDD. AllowedValue: As defined by the data type | type: Frequency multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| arfcn | It desribes the RF reference frequency (i.e. Absolute Radio Frequency Channel Number). Allowed Value: For NR, see TS 38.104 [8] clause 5.4.2.1. For EUTRAN, see TS 36.104 [14] clause 5.7.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| freqband | It desribes the the frequency operating band. Allowed Value: For NR, see TS 38.104 [8] clause 5.4.2.3. For EUTRAN, see TS 36.104 [14] clause 5.7.3. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| uEGroup | It describes the information of a UE Group (represented by specific 5QI, specific S-NSSAI, or a specific combination of S-NSSAI and 5QI). AllowedValue: As defined by the data type | type: UEGroup multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| fiveQI | It describes the information of a 5QI defined in 3GPP TS 28.541[5]. AllowedValue: 0 - 255 | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| sNSSAI | It describes the information of a S-NSSAI defined in 3GPP TS 28.541[5]. AllowedValue: As defined by the data type | type: S-NSSAI multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| intentAdminState | It describes the intent administrative state, which enables the MnS consumer to suspend an intent or cancel the suspension for a suspended intent. A suspended intent means this intent is not considered for fulfilment allowedValues: "ACTIVATED", "DEACTIVATED" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "ACTIVATED" isNullable: False |
| intentReference | It indicates the associated intent instance allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentReportReference | It indicates the associated intent report instance(s) allowedValues: Not Applicable | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentReportControl | It indicates the intent report control and subscription information. allowedValues: Not Applicable | type: IntentReportControl multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportRecipientAddress | It indicates the address of report recipient for MnS consumer. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| observationPeriod | It represents the observation period of the fulfilmentInfo for corresponding ExpectationTargets, IntentExpectations and Intent. At the end of the observation period, the corresponding fulfilment info is updated in the intent report. The observation period can be assigned by MnS consumer through requesting the MnS producer to set attribute "observationPeriod". MnS producer also can assign the observation period if MnS consumer didn’t assign it. The observation time is expressed in seconds. allowedValues: Not Applicable | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectedReportTypes | It indicates the type of IntentReports, which can be one/any/all of "IntentFulfilmentReport", "IntentConflictReport", "IntentFeasibilityCheckReport" and "IntentFulfilmentNegotiationReport" allowedValues: INTENT_FULFILMENT_REPORT, INTENT_CONFLICT_REPORT, INTENT_FEASIBILITY_CHECK_REPORT, INTENT_FULFILMENT_NEGOTIATION_REPORT | type: ENUM multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| reportingConditions | It indicates the specified conditions for intent reporting. The intent report will be sent when the specified reportingConditions is satisfied. allowedValues: Not Applicable | type: ReportingCondition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| reportingTargets | It indicates the specified targets needed to be reported. allowedValues: the scenario specific targetName defined in clause 6.2.2 Scenario specific IntentExpectation definition | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| timeCondition | It indicates the specified times for intent reporting. The specified times can be one-time interval, daily periodicity, weekly periodicity or monthly periodicity allowedValues: Not Applicable | type: SchedulingTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| targetFulfilmentCondition | It indicates the specified conditions of target Fulfilment for intent reporting. The targetFulfimentConfition can be described based on the achieved value for a specific targetName. allowedValues: Not Applicable | type: TargetFulfilmentCondition multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentFulfilmentReport | It describes the fulfillment information which is reported for the associated intent instance. allowedValues: Not Applicable | type: IntentFulfilmentReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentConflictReports | It describes the conflict information which is reported for associated intent instance if needed. allowedValues: Not Applicable | type: IntentConflictReport multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| conflictId | It is used to identify the detected conflict within an IntentReport instance. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictType | It describes the type of intent conflict. allowedValues: INTENT_CONFLICT, EXPECTATION_CONFLICT, TARGET_CONFLICT | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingIntent | It describes the DN of the conflicting intent allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingExpectation | It describes the expectationId of the conflicting IntentExpectation within an Intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingTarget | It describes the targetName of the conflicting ExpectationTarget within an IntentExpectation. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| recommendedSolutions | It describes the action recommended by the MnS producer to be undertaken by the MnS consumer to resolve intent conflict. The recommended solution applies only for the specific intent whose intent report contains this attribute. allowedValues: "MODIFY", "DELETE" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationFulfilmentResults | It includes the expectationFulfilmentInfo and targetFulfilmentResults for each IntentExpectation. The expectationFulfilmentInfo describes status of fulfilment of an intentExpectation and the related reasons for infeasible status. allowedValues: Not Applicable | type: ExpectationFulfilmentResult multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetFulfilmentResults | It includes targetFulfilmentInfo and targetAchievedValue for each ExpectationTarget. The targetFulfilmentInfo describes status of fulfilment of an expectationTarget and the related reasons for infeasible status. The targetAchieveValue describes current performance value for the ExpectationTarget. allowedValues: Not Applicable | type: TargetFulfilmentResult multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetAchievedValue | It describes the value that has been achieved for the expectation target at the time at which the report is generated. allowedValues: Not Applicable | type: Number multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentFeasibilityCheckReport | It describes the intent feasibility check information which is reported if needed. allowedValues: Not Applicable | type: IntentFeasibilityCheckReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| feasibilityCheckResult | It describes the result of intent fulfilment feasibility check allowedValues: FEASIBLE, INFEASIBLE | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| infeasibilityReasons | It describes the reason (e.g. invalid intent expression, the intent conflict) of the result of intent fulfilment feasibility check is INFEASIBLE allowedValues: INVALID_INTENT_EXPRESSION, INTENT_CONFLICT | type: ENUM multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationExplorationResults | It describes the intent exploration result, which includes the list of expectation exploration results. Intent exploration result is provided after MnS producer performs intent exploration pre-evaluation process as requested by MnS consumer. | type: ExpectationExplorationResult multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetExplorationResults | It describes the list of exploration results (i.e. recommended best values) for the expectation targets. Each TargetExplorationResult includes the TargetName, Target Condition and a recommended TargetValueRange | type: ExpectationTarget multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| contextExplorationResults | It describes the list of exploration results (i.e. recommended best values) for the expectation context (e.g. coverageAreaPolygonContext). Each ContextExplorationResult includes the ContextAttribute, Context Condition and a recommended ContextValueRange. | type: Context multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentHandlingCapabilityList | It describes the list of expectation object information and expectation target information which can be supported by intent handling function. allowedValues: Not Applicable | type: IntentHandlingCapability multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentHandlingCapabilityId | A unique identifier of property of intent handling capability should be supported by the intent handling function of MnS producer. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedExpectationObjectType | It describes the expectation object type which can be supported by a specific intent handling function of MnS producer. allowedValues: objectType defined in clause 6.2.1.3.2. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedExpectationTargetNames | It describes the supported expectation targets for the supported expectation object type. allowedValues: targetName defined in clause 6.2.1.3.3 | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| lastUpdatedTime | It describes the time for the latest update of the IntentReport Instance. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ContextSelectivity | It expresses the way in which all or a subset of the contexts may be applied. The contextSelectivity indicates which contexts are to be applied, i.e., "ALL_OF", "ONE_OF", or "ANY_OF" the contexts. AllowedValue: "ALL_OF", "ONE_OF", "ANY_OF" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "ALL_OF" isNullable: False |
| expectationSelectivity | It expresses the way in which the set of intentExpectations in an intent may be applied, e.g. for validation. The MnS consumer can provide intent expectations that describes the different alternatives candidate characteristics of the desired service from the MnS consumer's point of view that the MnS consumer wants to be validated. The expectationSelectivity indicates which intentExpectations are to be validated, i.e., "ALL_OF", "ONE_OF", "ANY_OF" the intentExpectations AllowedValue: "ALL_OF", "ONE_OF", "ANY_OF" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "ALL_OF" isNullable: False |
| intentPreemptionCapability | It describes the pre-emption capability. The attribute is used by MnS producer to decide the target of intent deletion or intent modification allowedValue: TRUE, FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "FALSE" isNullable: False |
| intentMgmtPurpose | It describes the MnS consumer requirements for the management purpose (required procedures) of the created or modified intent instance. allowedValue: FEASIBILITYCHECK, FULFILMENT | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "FULFILMENT" isNullable: False |
| inFeasibleExpectationInfos | It describes the list of InFeasibleExpectationInfo for all infeasible IntentExpectations in the intent. | type: InFeasibleExpectationInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| inFeasibleTargets | It describes the list of TargetNames for the InFeasibleTargets | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| Expectation.preferenceWeight | It represents the preference information of the Consumer on the intent expectation, which indicates the preference weight on the preferred intentExpectations. It is an integer in the range [0,10] used to indicate the extent of preference among intent expectations: - When the allowed value is larger than 0, it means that preference information is specified by the consumer for the intentExpectation. The larger value indicates the intentExpectation is preferred by the Consumer. - When the allowed value is equal to 0, it means that no preference information is specified by the consumer for the intentExpectation, i.e., intentExpectation is considered optional. AllowedValue: values in the range [0-10] | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| Target.preferenceWeight | It represents the preference information of the Consumer on the expectation target, which indicates the preference weight on the preferred expectationTargets. It is an integer in the range [0,10] used to indicate the extent of preference among expectation targets: - When the allowed value is larger than 0, it means that preference information is specified by the consumer for the expectationTarget. The larger value indicates the expectationTarget is preferred by the Consumer. - When the allowed value is equal to 0, it means that no preference information is specified by the consumer for the expectationTarget, i.e., expectationTarget is considered optional. AllowedValue: values in the range [0-10] | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| implicitIntentIndex | It indicates whether the MnS consumer enables the MnS producer to perform implicit intent discovery the implicit information like additional intent expectation which is not explicitly pointed out. allowedValue: TRUE, FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "FALSE" isNullable: False |
| implicitIntentExpectations | It describes the implicit intent expectation of the MnS consumer pointed out by the MnS producer. | type: IntentExpectation multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| implicitIntentContexts | It describes the list of implicit IntentContext(s) which represents the constraints and conditions that should apply for the entire implicit intent. allowedValues: triple of (attribute, condition, value range) | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentFulfilmentNegotiationReport | It contains the information that the MnS producer provides to the MnS consumer during intent fulfilment negotiations. | type: IntentFulfilmentNegotiationReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| possibleIntentOutcomeList | it indicates the possible outcomes for the intent expectations and expectationTargets within that intent and the relative cost/impact (on the related ExpecationObjects) of achieving that outcome For each possible outcome, a PossibleIntentOutcome is provided. Impacts on ExpecationObjects that were not in the original intent may be added to the report as new intentExpectations | type: PossibleIntentOutcome multiplicity: 1 .. * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| intentNegotiationConsumerFeedback | It contains the feedback information that the MnS consumer's provides to the MnS producer as response during intent Negotiation. | type: IntentNegotiationFeedback multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| preferredIntentOutcomeId | It indicates for a specific possible intent outcome confirmed by MnS consumer among those indicated by the MnS producer. It indicates the identifier of one of the possible intent outcomes among those provided to the MnS consumer by the MnS producer | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| consumerUtilityFunction | It indicates the consumer’s utility function for the intent or intent expectation. The utility function indicates a description of its preferences that should be used by the MnS producer to select among the alternatives. Note: The term "description of preference" needs to be clarified | type: UtilityFunction multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| consumerSatisfactionIndex | It indicates the MnS consumer's satisfaction with one or more of the MnS producer’s alternatives. . It represents the computed outcomes of MnS consumer's utility function an integer in the range [0,100]. The highest possible value indicates that the solution provided by the MnS producer achieves the best possible outcomes that the MnS consumer expected, e.g., that it achieves the highest range of a target whose desired values were defining as falling in a range. If it is provided in response to a report from an MnS producer indicating several candidate alternatives and their impacts, the satisfaction index is ordered according to the order of the reports. If it is provided as feedback for a single solution that was selected by the MnS producer and deployed, it indicates the MnS consumer’s satisfaction with the deployed solution.: | type: integer multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| possibleImpacts | It refers to the possible impacts of the possible candidate alternative. allowedValues: Not Applicable | type: PossibleImpact multiplicityl: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| possibleIntentOutcomeID | It identifies the number the outcome in the possible possibleOutcomeList | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| impactedObjects | It indicates the DN of managed object that may be impacted by the corresponding possible intent outcomes. allowedValues: Not Applicable | type: DN multiplicity: 1..* isOrdered: True isUnique: False defaultValue: None isNullable: False |
| impactedAttributes | It defines the name-value pair, where the name indicates the name of the attribute that is impacted and the value indicates the updated value. | type: ttributeNameValuePairSet multiplicity: 1..* isOrdered: True isUnique: False defaultValue: None isNullable: False |
| supportedExpectationTargetInfo | It describes the supported expectation targets for the supported expectation object type. | type: SupportedExpectationTargetInfo multiplicity: 1 … * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedExpectationTargetName | It indicates the name of the supported expectation targets for the supported expectation object type. allowedValues: depends on ExpectationObject in the IntentExpectation | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| supportedTargetCondition | It expresses the limits within which the supportedExpectationTargetName shall be supported. allowedValues: targetCondition defined in clause 6.2.1.3.3 | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "IS_EQUAL_TO" isNullable: False |
| SupportedTargetValueRange | It describes the range of values that applicable to the supportedExpectationTargetName and the supportedTargetCondition. allowedValues: targetValueRange defined in clause 6.2.1.3.3 | type: ValueRangeType multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| NOTE: For "IS_ALL_OF", the value shall be a match of the entire list. |  |  |

End of changes