3GPP TSG-SA5 Meeting #161	S5-253011

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0233 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.554 Add EE KPI evaluated from network availability performance dimension |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Unicom, China Mobile, China Telecom, Orange, Verizon, ATT, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the conclusion for multi-dimensional network energy efficiency metrics in TR 28.880, it is proposed to add EE KPI evaluated from network availability performance dimension. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding EE KPI evaluated from network availability performance dimension. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 6.7.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 6.7.X	Energy efficiency evaluated from network availability

#### 6.7.X.1	Introduction

The KPI is defined from a network availability performance dimension perspective.

#### 6.7.X.2	Energy efficiency of cell based on availability

a) EECell,AvailAvgTimeCU

b) A KPI that shows the energy efficiency of a cell in a gNB based on availability performance. This KPI is obtained by dividing cell availability KPI by the average energy consumption per cell in a gNB over the same observation period. The unit of this KPI is sec/J.

c)

EECell,AvailAvgTimeCU=CellAvailAvgTimeCUECper cell of gNB

d) GNBCUCPFunction

e) The cell availability KPI is defined in clause 6.10.1.1.1. The average energy consumption per cell in a gNB is obtained by considering energy consumption of gNB (ECgNB) for a given time period and dividing it by total number of cells in a gNB that are operational in same time duration/period. ECgNB is defined in clause 6.7.3.4.2.

NOTE:	The cell availability KPI is for the split gNB deployment scenario. The average energy consumption for cell is derived by averaging of ECgNB over the number of cells.

End of change