3GPP TSG-SA5 Meeting #161	S5-253020

Fukuoka, Japan, 19 - 23 May 2025

Source:	Samsung

Title:	Solution for triggered CCL

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS 28.567

Version:	0.2.0

Work Item:	CCL

Comments

This contribution proposes the solution for the use case of CCL instantiation based on conditions.

Proposed Changes

* * * First Change * * * *

* * * Next Change * * * *

# 7	Procedures

## 7.C	Procedure for conditional trigger/instantiation of CCLs

Figure 7.C-1: Procedure and interactions for conditional trigger/instantiation of CCLs

Step 0: There exists an object exposing the MnS producer responsible for instantiating CCLs. This object may be represented by a subnetwork or managed element.

Step 1: The MnS consumer creates on the MnS producer responsible for instantiating CCLs the set of conditios to be evaluated for instantiation of the CCL. These conditions are created as an instance of TriggerConditionDescriptor defned in 28.572. TriggerConditionDescriptor describes the conditions that should be evaluated including performance, provisioning and fault management conditions. The performance conditions includes managed object, measurement/KPI name and the trigger value. The provisioning conditions includes the managed object, location, event and time of the provisioning events. The fault conditions includes managed object, alarmSeverityThreshold and alarmTypeThreshold.

Step 2: The MnS producer monitors the network to detect when the conditions defined in TriggerConditionDescriptor evaluate to TRUE.

Step 3: If conditions in TriggerConditionDescriptor evaluate to TRUE, the MnS producer instantiates the CCL.

Step 4: For the instantiated CCL, the MnS producer may notify the conditions that triggered the CCL.

* * * Next Change * * * *

## X.2.1	Procedure for conditional instantiation of CCLs (Figure 7.C-1)

@startuml Procedure for conditional composition of CCLs

skinparam Shadowing false

autonumber

skinparam monochrome true

participant "CCL MnS consumer" as CMC

participant "CCL MnS producer" as CMP

CMC -> CMP: create CCL instantiation conditions

CMP -> CMC: Monitor conditions defined

CMP -> CMP: If conditions in TriggerConditionDescriptor\n evaluate to TRUE instantiate CCL

CMP -> CMC: Notify  conditions.

@enduml

PlantUML source code for Figure 7.C-1 Procedure for conditional instantiation of CCLs

* * * End of Changes * * * *