3GPP TSG-SA5 Meeting #161	S5-253045

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025                          revision of S5-252282

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.156 | CR | 0114 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.156 Clarify usage of DN type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is unclear if the values of attributes define with type DN (distinguished name) should or should not include configured the DnPrefix. It was discovered that different implementations use the type DN in different ways. - Some populate it with a full DN including both the DnPrefix and a LocalDN (LDN) - Some only include an LDN utilizing the fact that all attributes with type DN actually refer to MOIs under the same root MOI ManagedElement/Subnetwork. In this case, including the DnPrefix as part of the value when reading or writing the attribute is not needed, not useful and would lead to interoperability errors. The 32.156 definition of the DN data type does not clearly decide between Full-DN or LDN. Furthermore even for a Full DN, it is allowed to define a nullPrefix. It is also not described that the dnPrefix used as part of type-DN attributes should match the dnPrefix atribute configured. However, consumer’s need to know when writing or reading a DN-typed attribute whether a full DN or an LDN should be used and whether the dnPrefix is corresponds to the value in the dnPrefix attribute (of the ManagedElement, SubNetwork or MeContext). (The dnPrefix is a mandatory attribute with multiplicity strictly "1" but is supported only on the root MOI which can be a Subnetwork, ManagedElement or MeContext.) The uncertainity about whether the dnPrefix should or should not be included in a DN typed attribute (e.g. schedulerRef) can prevent a standard compliant consumer from configuring a standard compliant producer. E.g. - If the consumer (e.g. an element manager) tries to write a DN-typed attribute into the producer without including a dnPrefix the producer might reject it, as it expects a Full DN including the dnPrefix. - If the consumer tries to write a DN-typed attribute into the producer including the dnPrefix the producer might reject it, if the the dnPrefix in this attribute does not match the configured dnPrefix (e.g.) in the root Subnetwork MOI. - If the consumer tries to write a DN-typed attribute into the producer including the configured dnPrefix the producer might reject it, claiming that it uses a nullPrefix in the reference attributes as allowed by TS 32.300. Today it is not specified that the dnPrefix used in DN-typed attributes must match the dnPrefix configured (e.g.) in the root SubNetwork MOI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the configured DnPrefix shall be part of attributes that have the type DN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.4.3.1	Description

It represents the general notion of being a data type (i.e. a type whose instances are identified only by their values) whose definition is defined by this specification and not by the user (e.g. specification authors).

This repertoire uses two kinds of data types: predefined data types and user-defined data types.  The latter is defined in 5.3.4 <<dataType>> and 5.3.5 <<enumeration>>.

The following table lists the UML data types selected for use as predefined data type.

Table 5.4.3.1-1: UML defined data types

| Name | Description and reference |
| --- | --- |
| Boolean | See Boolean type of ITU-T X.680 [7]. |
| Integer | See Integer type of ITU-T X.680 [7]. |
| String | See PrintableString type of ITU-T X.680 [7]. |

The following table lists data types that are defined by this repertoire.

Table 5.4.3.1-2: Non-UML defined data types

| Name | Description and reference |
| --- | --- |
| AttributeValuePair | This data type defines an attribute name and the attribute’s value. |
| BitString | This data type is defined by Bit string of subclause 3 and subclause G.2.5 of ITU-T X.680 [7]. |
| DateTime | This data type defines Date/Time Format, and it is protocol specific. |
| DN | This data type defines the DN (see Distinguished Name of TS 32.300 [3]) of an object. It contains a sequence of one or more name components. The “initial sub-sequence” (note 1) of a DN is also a DN of an object. A DN shall contain the DnPrefix (as configured by the dnPrefix attribute of either Subnetwork, ManagedElement or MeContext) followed by the Local DN. Note 1: Suppose an object’s DN is composed of a sequence of 4 name components, i.e. 1st, 2nd, 3rd and 4th components. The “initial sub-sequence” of this DN is composed of the 1st, 2nd and 3rd components. |
| External | This data type is defined by another organization. |
| Real | This data type is defined by Real type of ITU-T X.680 [7] |

End of  changes