3GPP TSG-SA5 Meeting #161	S5-253052

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0387 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel 19 CR TS 28.532 correct the format of input parameters define in stage 3 to align with stage 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | HUAWEI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The input parameters of getMOIAttributes, modifyMOIAttributes and deleteMOI for the YANG/Netconf-based solution set defined in stage 3 in clause 12.1.3 are inconsistent with that defined in clause 11.1.1 in stage 2. - In the definition of clasue 11.1.1.2.2, the input parameters include scope and two sub-options, scopeType and scopeLevel, while in the definition defined in clause 12.1.3.1.3 in stage3, only scope and level are included. - In the definition of clasue 11.1.1.3.2, the input parameters include scopeType and scopeLevel, while in the definition of clause 12.1.3.1.4, the IS operation parameter name only include scope. - In the definition of clasue 11.1.1.4.2, the input parameters include scopeType and scopeLevel, while in the definition of clause 12.1.3.1.5, the IS operation parameter name only include scope. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the first column of input parameters and third colum of support qualifier in Table 12.1.3.1.3-1 in stage3 to be consistent with the names of the input parameters - Update the first column of input parameters and third colum of support qualifier in Table 12.1.3.1.4-1 in stage3 to be consistent with the names of the input parameters - Update the first column of input parameters and third colum of support qualifier in Table 12.1.3.1.5-1 in stage3 to be consistent with the names of the input parameters |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The implementation of input parameters in stage 3 will not align with input parameters in stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1.2.2, 11.1.1.3.2, 11.1.1.4.2, 11.1.1.4.3, 12.1.3.1.3, 12.1.3.1.4, 12.1.3.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

11.1.1.2	getMOIAttributes operation

11.1.1.2.1	Definition

This operation is invoked by MnS consumer to request the retrieval of management information (Managed Object attribute names and values) from the MIB maintained by MnS producer. One or several Managed Objects may be retrieved - based on the containment hierarchy.

The operation allows a MnS consumer to specify the data nodes to be returned with two optional methods. The first method allows to select objects with the "scope" and "filter" parameters. The second method uses the "dataNodeSelector" to specify the data nodes to be returned. All kinds of data nodes (i.e. objects, attributes, attribute fields and attribute elements) can be selected. The selection may be based on conditions. The value of "dataNodeSelector" is an expression constructed based on a SS specific grammar.

Note that the functionality of the "scope" and "filter" parameters is fully covered by the functionality of the "dataNodeSelector" parameter. Therefore, a MnS producer supporting the "dataNodeSelector" parameter is not expected to support the "filter" parameter.

A SS may choose to split this operation in several operations (e.g. operations to get "handlers" or "iterators" to Managed Objects fulfilling the scope/filter criteria and other operations to retrieve attribute names/values from these "handlers").

11.1.1.2.2	Input Parameters

| Name | S | Information Type | Comment |
| --- | --- | --- | --- |
| baseObjectInstance | M | DN | This parameter specifies the base object instance. If the "scope" parameter is absent, then either only the base object or the complete subtree below and including the base object shall be selected. The default behaviour is protocol specific. |
| CHOICE 1.1 scope | O | n/a | This parameter specifies the scope. It is a structured parameter and consists of the sub-parameters "scopeType" and "scopeLevel". The scope describes which object instances are selected with respect to a base object instance. The base object instance needs to be specified using a dedicated attribute. |
| scopeType | O | ENUM { BASE_ONLY, BASE_ALL, BASE_NTH_LEVEL, BASE_SUBTREE } | If the optional "scopeLevel" parameter is not supported or absent, allowed values of "scopeType" are "BASE_ONLY" and "BASE_ALL". The value "BASE_ONLY" indicates only the base object is selected. The value "BASE_ALL" indicates the base object and all of its subordinate objects (incl. the leaf objects) are selected. If the "scopeLevel" parameter is supported and present, allowed values of "scopeType" are "BASE_NTH_LEVEL" and "BASE_SUBTREE". The value "BASE_NTH_LEVEL" indicates all objects on the level, which is specified by the "scopeLevel" parameter, below the base object are selected. The base object is at "scopeLevel" zero. The value "BASE_SUBTREE" indicates the base object and all of its subordinate objects down to and including the objects on the level, which is specified by the "scopeLevel" parameter, are selected. The base object is at "scopeLevel" zero. This parameter is redundant and can be omitted when confirming only the protocol specific default behaviour. |
| >scopeLevel | O | Integer | This parameter is valid only when scopeType is set to BASE_NTH_LEVEL or BASE_SUBTREE. |
| CHOICE 1.2 filter | O | See Comment. | This parameter defines filter criteria to be applied to the objects selected by the "baseObjectInstance", "scope" and "scopeLevel" parameters. The actual syntax and capabilities of the filter is SS specific. However, each SS should support a filter consisting of one or several assertions that may be grouped using the logical operators AND, OR and NOT. Each assertion is a logical expression of attribute existence, attribute value comparison ("equal to X, less than Y" etc.) and MO Class. |
| CHOICE 1.34 attributeListIn | O | LIST OF attribute name. | This parameter identifies the attributes to be returned by this operation. If the parameter is absent or empty all attributes shall be returned. |
| CHOICE 2.1 dataNodeSelector | O | string | This parameter contains an expression allowing to conditionally select data nodes. The expression semantic and syntax is SS specific. |

11.1.1.2.3	Output Parameters

| Name | S | Matching Information | Comment |
| --- | --- | --- | --- |
| managedObjectClass | M | ManagedEntity class | For each returned MO: The class of the MO. |
| managedObjectInstance | M | ManagedEntity DN | For each returned MO: The name of the MO. This is a full DN according to TS 32.300 [25]. |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value > | For each returned MO: A list of name/value pairs for MO. |
| status | M | ENUM (OperationSucceeded, OperationFailed) | An operation may fail because of a specified or unspecified reason. |

11.1.1.2.4	Results

In case of success, all of the ManagedEntity instances selected for retrieval are returned. In case of failure, a specified or unspecified reason may be provided in the Output parameters.

11.1.1.3	modifyMOIAttributes operation

11.1.1.3.1	Description

This operation is invoked by MnS consumers to request a MnS producer to modify one or more attributes of one or more managed objects on that MnS producer. More specifically, this operation allows to modify (replace, add, remove) complete attributes, attribute fields and attribute elements.

The selection of the objects to be modified is achieved with the parameters "baseObjectInstance", "scopeType", "scopeLevel" and "filter". If no "scopeType" is specified, the value defaults to "BASE_ONLY". Also, when no scoping is supported by the MnS producer, the value in "baseObjectInstance" identifies the object to be modified.

A specific protocol solution may choose to split the selection of objects with scoping and filtering and the modification of the attributes of the selected objects into different operations.

The modifications to be applied to the selected objects are described in the "modificationList" parameter. This is a multi-valued parameter. Each value is a structure composed of "modifyOperator", "nodeIdentifier" and "nodeValue". The values of "modificationList" are ordered and shall be applied in the sequence as they occur in the list.

The parameter "nodeIdentifier" is used to identify the attribute, attribute field or attribute element to which the modification shall be applied. Attributes within one managed object can be identified with their name only. Unambiguous identification of attribute fields is not possible with their names only, because multiple attribute fields within one object may have the same name. Therefore, the identification needs to be based e.g. on global or local identifiers, or on the specification of a path allowing to navigate to the attribute field. Details are protocol specific.

For the modification of multi-valued attributes or multi-valued attribute fields two cases are distinguished:

-	All values (elements) are considered as a single value. Elements cannot be addressed individually. If replaced, all existing elements are replaced with the new elements received in the modification request. If deleted, all existing elements are deleted. Addition of new elements to existing elements is not possible.

-	Each element can be addressed individually. Single elements can be added, replaced, and deleted.

The first case does not need any further considerations. The second case requires the identification of attribute elements and attribute field elements in the modification request, when replacing and deleting elements in ordered and unordered lists, and when adding elements to ordered lists. Details are not defined at stage 2. They are protocol specific and include identification by the element value, identification by a positional index or identification by an auxiliary key added at stage 3. Note that the concept of element keys is not defined at stage 2. Identification of elements may be provided also by the "nodeIdentifier" parameter.

The "modifyOperator" parameter specifies the modification to be applied to the value of the attribute or attribute field, or the attribute element or attribute field element identified by the "nodeIdentifier". The parameter can have the values "replace", "add", "remove" or "setToDefault":

-	For "replace", the "nodeValue" specifies the attribute value, attribute field value, attribute element or attribute field element that shall replace the existing value.

-	For "add", the "nodeValue" specifies the attribute value or attribute field value to be added to an attribute or attribute field without value, or the new attribute element or attribute field element to be added to a multi-valued attribute.

-	For "remove", the "nodeValue" is absent when an attribute value or attribute field value is removed. When an attribute element or attribute field element is removed, "nodeValue" may carry the element to be removed, depending on how on protocol level attribute elements and attribute field elements are identified.

-	For "setToDefault", the "nodeValue" is absent.

Attributes and attribute fields without value can be represented in different protocol specific ways, for example by an attribute name without attribute value, by an absent attribute name/value pair, or by a specific attribute value (such as "null" or "nil").

The "modifyMOIAttributes" operation allows to modify one or more attributes in one or more objects. When not all attribute modifications can be applied successfully, the MnS producer has different options how to proceed. He may not perform any of the modifications and roll back to the state at the reception of the modification request. He may apply the changes that can be applied, so that some of the requested modifications are applied and some are not applied. He may stop processing the modification request when the first error occurs. The stage 2 definition of this operation does not include any provisions on how to proceed in case an error occurs. These provisions are left to stage 3.

The model state after applying the " modifyMOIAttributes " request shall fulfill all model constraints such as cardinality, multiplicity, allowed values, or data types, otherwise the operation shall fail.

11.1.1.3.2	Input parameters

| Parameter Name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| baseObjectInstance | M | ManagedEntity.objectInstance | Base object used for scoping the target objects of the operation. If no scoping is applied, the base object is the only target object. |
| scope |  |  |  |
| CHOICE 1.1 scopeType | O | See corresponding parameter in "getMOIAttributes". | See corresponding parameter in "getMOIAttributes". |
| CHOICE 1.2 scopeLevel | O | See corresponding parameter in "getMOIAttributes". | See corresponding parameter in "getMOIAttributes". |
| CHOICE 1.3 filter | O | See corresponding parameter in "getMOIAttributes". | See corresponding parameter in "getMOIAttributes". |
| modificationList | M | LIST OF SEQUENCE < nodeIdentifier modifyOperator, nodeValue > | Set of sub-operations to be applied to attributes and attribute fields of the target objects. The "nodeIdentifier" specifies the target attribute or target attribute field of the sub-operation. The "modifyOperator" specifies the operation to be applied to the target attribute or target attribute field . The parameter can have the values "replace", "add", "remove" or "setToDefault". The "nodeValue" specifies the value used by the sub-operation. This parameter is absent for "remove" operations. |

11.1.1.3.3	Output parameters

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| modificationsOut | O | LIST OF SEQUENCE < objectInstance DN, objectClass string, LIST OF SEQUENCE< attribute name, attribute value > > | Provides for each object, that is selected by the request, the object name, the object class, and a list of name/value pairs with the values of all attributes after modification. If all requested modifications are applied, the parameter may be absent. If no requested modification is applied and an error response is returned, the parameter may be absent, too.. |
| status | M | ENUM ( SUCCEEDED, PARTIALLY_FAILED, FAILED ) | Indicates if all, some or none of the requested modifications were applied. Details on the error, such as which modification could not be applied and the corresponding reason, may be returned as well. |

11.1.1.3.4	Results

In case of success, all of the ManagedEntity instances selected for modification are modified. In case of failure, a specified or unspecified reason may be provided in the Output parameters.

11.1.1.4	deleteMOI operation

11.1.1.4.1	Description

This operation is invoked by MnS consumer to request the deletion of one or more Managed Object instances in the MIB maintained by the MnS producer.

11.1.1.4.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| baseObjectInstance | M | DN | The MO instance that is to be used as the starting point for the selection of managed objects to which the filter (when supplied) is to be applied. This is a full DN according to TS 32.300 [25]. |
| scope |  |  |  |
| CHOICE 1.1 scopeType | O | See corresponding parameter in getMOIAttributes. | See corresponding parameter in getMOIAttributes. |
| CHOICE 1.2 scopeLevel | O | See corresponding parameter in getMOIAttributes. | See corresponding parameter in getMOIAttributes. |
| CHOICE 1.3 filter | O | See comment. | See corresponding parameter in getMOIAttributes. |

11.1.1.4.3	Output parameters

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| deletionList | M | LIST OF SEQUENCE< ManagedEntity DN, ManagedEntity class name> | If the base object alone is specified, then this parameter is optional; otherwise it contains a list of managedObjectInstance/managedObjectClass pairs identifying the managed objects deleted. |
| status | M | ENUM (OperationSucceeded, OperationFailed, OperationPartiallySucceeded) | An operation may fail because of a specified or unspecified reason. The operation is partially successful if some, but not all, objects selected to be deleted are actually deleted. |

In lieu of a synchronization parameter, best effort synchronization will apply; that is, all managed objects selected for this operation will perform the operation if possible regardless of whether some managed objects fail to perform it.

11.1.1.4.4	Results

In case of success, all of the ManagedEntity instances selected for deletion are deleted. In case of failure, a specified or unspecified reason may be provided in the Output parameters.

Next Change

12.1.3.1.3	Operation getMOIAttributes

This IS operation is mapped to NETCONF <get> or <get-config> operation, depending on whether all configuration and state information is to be retrieved, or configuration data only.  (In the next paragraphs only <get> operation is mentioned but <get-config> is always an alternative).

The IS operation patameters baseObjectInstance , (3GPP-)filter, scope, level, dataNodeSelector  and attributeListIn are all combined and mapped into the Netconf-filter element. The scopes BASE_ONLY and BASE_ALL can be mapped to both subtree and Xpath filtering. The scopes BASE_NTH_LEVEL and BASE_SUBTREE can only be mapped to Xpath filtering.

The IS operation parameters are mapped to SS equivalents according to table 12.1.3.1.3-1 and table 12.1.3.1.3-2.

Table 12.1.3.1.3-1: Mapping of IS getMOIAttributes input parameters to SS equivalents

| IS operation parameter name | SS parameter name | S | Remark |
| --- | --- | --- | --- |
| baseObjectInstance | filter | M | Initial part of the filter element. For subtree filter this is a set of XML element representing lists containing MOIs together with the leafs representing key values for these MOIs from the root MOI (e.g. ManagedElement) to the baseObjectInstance. For Xpath filter it is the initial parts of the Xpath expression representing the same information. |
| scopeType | filter | M | BASE_ONLY and BASE_ALL realized by the initial XML elements of the <get> operation. BASE_SUBTREE and BASE_NTH_LEVEL is encoded in the Xpath filter. |
| scopeLevel | filter | M | Included in the Xpath filter, see examples. (If level is used Xpath filtering must be used. For BASE_SUBTREE the levels number is transformed into a number of filter sub-expressions joined by the OR operator. For BASE_NTH_LEVEL included in the Xpath expression as a sequence of ‘*’ parts (descendant axis) The number of ‘*’ correspond to the number of levels. |
| filter | filter | M | Netconf Subtree or Xpath filter |
| attributeListIn | filter | M | add the attributes to the subtree or Xpath filter |
| dataNodeSelector | filter | M | Included in the Xpath filter |

Table 12.1.3.1.3-2: Mapping of IS getMOIAttributes output parameters to SS equivalents

| IS operation parameter name | SS parameter name | S | Remark |
| --- | --- | --- | --- |
| managedObjectClass | data | M | Can be extracted from the NETCONF <rpc-reply> <data> elements |
| managedObjectInstance | data | M | Can be extracted from the NETCONF <rpc-reply> <data> elements |
| attributeListOut | data | M | Can be extracted from the NETCONF <rpc-reply> <data> elements |
| status | data | M | rpc-reply or rpc-error indicates general status. |

Next Change

12.1.3.1.4	Operation modifyMOIAttributes

This IS operation modifies one or multiple managed object instances. It is mapped to the NETCONF <edit-config> operation. The NETCONF <edit-config> operation can modify attributes in a given MOI or set of MOIs but only indirectly supports scope or filtered sets of MOIs that are part of the modifyMOIAttributes 3GPP operation specification. <edit-config> needs a config block, containing the explicit config changes to be made for each MOI.

The default-operation parameter should be set to none.

The Netconf operation attribute on the list representing modified MOI(s) should be set to create, replace or delete according to the ENUM in the modificationList.

The IS operation parameters are mapped to SS equivalents according to table 12.1.3.1.4-1 and table 12.1.3.1.4-2.

Table 12.1.3.1.4-1: Mapping of IS modifyMOIAttributes input parameters to SS equivalents

| IS operation parameter name | SS parameter name | S | Remark |
| --- | --- | --- | --- |
| baseObjectInstance | config | M | A sequence of embedded XML elements inside the <config> element. XML elements for all containing MOIs and their ids(keys) shall be included together with the XML elements representing the to be modified MOI and its key. |
| scopeType | config | M | BASE_ONLY supported as default. Multiple MOIs can be specified in the same operation, emulating other scopes. |
| scopeLevel | config | M |  |
| filter | config | M | Multiple MOIs can be specified in the same operation, emulating filtering. |
| modificationListIn | config | M | The “attributes container” and leaf, leaf-list or list entries representing the attributes. |

Table 12.1.3.1.4-2: Mapping of IS modifyMOIAttributes output parameters to SS equivalents

| IS operation parameter name | SS parameter name | S | Remark |
| --- | --- | --- | --- |
| modificationListOut | no corresponding SS parameter | M | Not supported. (note 1) |
| status | - | M | rpc-reply or rpc-error indicates general status. The following elements give detailed error information: <error-tag> <error-path> |

Note 1:	Successful Netconf <edit-config> operations only return an <ok> element. Therefore, the attributeListOut can be retrieved via a separate <get-config> operation.

Next Change

12.1.3.1.5	Operation deleteMOI

This IS operation deletes one or multiple managed object instances. It is mapped to the NETCONF <edit-config> operation. <edit-config> can delete one or more specific MOIs but only indirectly supports scope or filtered sets of MOIs that are part of the generic deleteMOI 3GPP operation specification. <edit-config> uses a config block, indicating the MOI(s) to be deleted.

The Netconf operation attribute on the list representing the baseObjectInstance should be set to delete or remove.

The default-operation parameter should be set to none.

The IS operation parameters are mapped to SS equivalents according to table 12.1.3.1.5-1 and table 12.1.3.1.5-2.

Table 12.1.3.1.5-1: Mapping of IS deleteMOI input parameters to SS equivalents

| IS operation parameter name | SS parameter name | S | Remark |
| --- | --- | --- | --- |
| baseObjectInstance | config | M | A sequence of embedded XML elements inside the <config> element. XML elements for all containing MOIs and their ids(keys) shall be included together wilt the XML elements representing the to be deleted MOI and its key. |
| scopeType | config | M | BASE_ONLY supported as default. Multiple MOIs can be specified in the same operation, emulating other scopes. |
| scopeLevel | config | M |  |
| filter | config | M | Multiple MOIs can be specified in the same operation, emulating filtering. |

Table 12.1.3.1.5-2: Mapping of IS deleteMOI output parameters to SS equivalents

| IS operation parameter name | SS parameter name | S | Remark |
| --- | --- | --- | --- |
| deletionList | no corresponding SS parameter | M | Not supported. (note 1) |
| status | - | M | rpc-reply or rpc-error indicates general status. The following elements give detailed error information: <error-tag> <error-path> |

NOTE 1: Successful Netconf <edit-config> operations only return an <ok> element. Therefore, the deletionList can be retrieved via a separate <get-config> operation.

End of changes