3GPP TSG-SA5 Meeting #161	S5-253093

Fukuoka, Japan, 19 - 23 May 2025

# Source:	ZTE Corporation

# Title:	New WID on OAM for Extended Reality and Media service (XRM) Phase 2

# Document for:	Approval

# Agenda Item:	5.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:

OAM for Extended Reality and Media service (XRM) Phase 2

Acronym:

XRM_Ph2-OAM

Unique identifier:

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
| X | Normative – Stage 1 |
| X | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 1040032 | (Stage 2) Extended Reality and Media service (XRM) Phase 2 | SA2 work item to specify the enhancements on XR and media services in Rel-19. |
| 1060027 | (Stage 3) Extended Reality and Media service (XRM) Phase 2 | CT work item to specify the enhancements on XR and media services (stage 3) in Rel-19. |

# 3	Justification

In the 5G era, mobile media services such as cloud AR/VR, cloud gaming, and video-based tele-control for machines or drones are expected to contribute increasingly to 5G network traffic. All media traffic, regardless of the codec used, shares certain common characteristics that can be leveraged for improved transmission control and efficiency.

To better manage media services alongside other data services by fully utilizing this information, SA2 has worked on extended Reality and Media service (XRM) Phase 2 in the Rel-19 and may further enhance it in Rel-20. The 5GS is enhanced to support the following aspects:

- Support of ECN marking for L4S to expose the congestion information in both 3GPP access and non-3GPP access

- Support of XRM services to expose additional information related to XRM

- Support of PDU Set based Handling including PDU Set identification and marking in both 3GPP access and non-3GPP access

- Support of UL/DL policy control based on round-trip latency requirement

- Support of Packet Delay Variation monitoring and reporting

- Support of providing traffic assistance information to enable Connected mode DRX power saving

- Support of PDU Set Importance based transport level packet marking

- Support of transferring media related information over N6

- Support of dynamically changing traffic characteristics handling for better resource management

- Support of traffic identification and differentiated QoS for multiplexed media flows

Based on the enhancement aspects above in SA2, to support XRM functionality, following new requirements are proposed for the 3GPP management system.

- Management support for ECN marking in both 3GPP access and non-3GPP access. The 5G System supports ECN (Explicit Congestion Notification) marking in both 3GPP access and non-3GPP access to facilitate Low Latency, Low Loss, and Scalable Throughput (L4S) services for uplink and/or downlink QoS Flows. Specifically, for 3GPP access, ECN marking for L4S in the IP header is supported either in the NG-RAN or in the PSA UPF, as detailed in clause 5.37.3 of TS 23.501. The SMF can be instructed, based on either dynamic or predefined PCC rules, to signal ECN marking for L4S to the PSA UPF. In the case of non-3GPP access, ECN marking for L4S in the IP header is supported in the N3IWF, TNGF, W-AGF, or 5G-RG. From a management perspective, to fully support ECN marking, enhancements are required in pre-defined PCC rules, UPF functions, and managed non-3GPP access NFs, such as the N3IWF function and TNGF function.

- Management support for XRM services to expose additional information related to XRM. To reduce latency, congestion, and ensure an optimal user experience, interactions between applications and the 5G System are essential. Specifically, XR application servers can adjust media codecs and traffic rates to adapt to network conditions and maintain a high-quality user experience. As specified in the clause 5.37.4 of TS 23.501, the 5GS can expose the information related to XR/media services (e.g., Packet delay variation) based on the QoS Monitoring. QoS monitoring comprises of measurements of QoS monitoring parameters and reports of the measurement result for a QoS Flow and can be enabled based on 3rd party application requests and/or operator policies configured in the PCF. Enhancements on the pre-defined PCC rule and QoS Monitoring Control are needed and pre-defined N4 rules including QER and SSR may be configured by the management system.

- Management support for PDU set based handling including PDU Set identification and marking in both 3GPP access and non-3GPP access networks. A PDU Set is comprised of one or more PDUs carrying an application layer payload. AF provided PDU Set QoS Parameters and Protocol Description may be used in determining the PCC Rules by the PCF as defined in clause 6.1.3.27.4 of TS 23.503 and the DL Protocol Description may be used for identifying the PDU Set Information and PDU Set Information marking by the PSA UPF. For QoS Flows enabled with PDU Set based QoS handling, PDU Set QoS Parameters are determined by the PCF and provided by SMF to the 5G-AN as part of the QoS profile. To support PDU set based handling, pre-defined PCC rules, UPFFunction, and managed non-3GPP access NFs should be enhanced.

- Support of UL/DL policy control based on round-trip latency requirement. For XR and other interactive media services that require very low Round-Trip (RT) latency, Uplink-Downlink policy control may be supported to meet the RT latency requirement. To support UL and DL delay tracking, the QoS monitoring for UL packet delay and the DL packet delay shall be triggered respectively to request tracking the UL packet delay of the QoS flow used in UL and DL packet delay of the QoS flow used in DL independently. Based on the QoS monitoring results, the PCF may readjust the UL PDB and/or DL PDB under the consideration of the RT latency requirement to better fit the new situation. From a management perspective, the QoS Monitoring Control should be enhanced.

- Management support for Packet Delay Variation reporting. The Packet Delay Variation is the variation of packet delay measured between UE and PSA UPF. Upon AF request for Packet Delay Variation monitoring together with packet delay monitoring, the PCF triggers the QoS monitoring procedure and obtains the UL, DL or RT QoS Monitoring result from the SMF. New PM for Packet Delay Variation should be defined and pre-defined N4 rules (i.e., SSR in this bullet) may need to be configured by the management system.

- Management support for providing traffic assistance information to enable Connected mode DRX power saving. In order to improve power management schemes like CDRX to achieve the best trade-off among key performance indicators like latency and device battery lifetime, the following traffic assistance information may be provided by the CN to NG-RAN at PDU Session Establishment/Modification via an NGAP Message to configure UE power saving management scheme.

  - For Periodicity and N6 Jitter Information associated with Periodicity, the PCF may include an indication for SMF to request the UPF to perform N6 Traffic Parameter(s) measurement (i.e. N6 Jitter Information associated with the DL Periodicity and if not provided by the AF, UL/DL periodicity) within the PCC Rules. Enhancements on the PCC rule is needed.

  - For the End of Data Burst Marking, an indication can be included in a PCC rule and/or on local operator policies, and then the SMF should request the PSA UPF to detect the last PDU of the data burst and mark the End of Data burst based on the PCC rule. Enhancements on UPFFunction is needed to indicate if end of data burst marking is supported or not.

- Support of PDU Set Importance based transport level packet marking. For every QoS Flow, the SMF shall determine the transport level packet marking value (e.g. the DSCP in the outer IP header) based on the 5QI, the Priority Level (if explicitly signalled) and optionally, the ARP priority level and provide the transport level packet marking value to the UPF as described in clause 5.8.2.7 of TS 23.501. For QoS Flows involving PDU Set QoS handling, the SMF may also consider PDU Set Importance values. It can provide a list of DSCP marking values to the UPF via a FAR, with each value corresponding to one or more PDU Set Importance values, which enables differentiated handling of PDU Sets within the QoS Flow for the downlink direction. Besides, 5G-AN could also consider the relative PDU Set Importance across QoS Flows of the same Priority Level when determining which PDU Set needs to be discarded. This feature indicates the management support for configuring FAR and enhancing UPFFunction.

- Management support for transferring media related information over N6. End-to-end encryption is widely used in current networks to ensure security, and this is also expected for XR and media service. However, when the end-to-end connection for XR and media traffic between the UE and Application Server is fully encrypted, the UPF cannot identify certain information, such as PDU Set information. As specified in the clause 5.37.9, on-path N6 signalling methods (Media Over QUIC, connect-UDP, and UDP-Option) are used to transfer media related information over N6. These methods allow media-related information to be sent together with the encrypted payload, ensuring that media related information is received synchronously with the media packet. For example, with MoQ, PCF determines PCC rules based on AF request including Protocol Description and the requirement for MoQ, to instruct SMF/UPF to identify PDU Set information for MoQ traffic. The UPF supporting the MoQ relay functionality could be selected by SMF during the PDU session establishment. From a management perspective, enhancements are needed in pre-defined PCC rules, N4 rules, and UPFFunction to support these processes effectively.

- Management support for dynamically changing traffic characteristics. The traffic pattern of an XR session changes can be based on the usage and applications. For instance, the traffic pattern may change dynamically when the size of media frames fluctuates due to scene changes, such as during initial start-up or when a user interacts with a progress bar. To handle data burst in dynamic traffic, as described in clause 6.1.3.27.5 of TS 23.503, the PCF may include a Data Burst Size Marking Indication/Time to Next Burst Marking Indication within a PCC Rule to request the UPF to identify and mark the Data Burst Size of the data burst/ time to next data burst for the corresponding QoS Flow. Furthermore, Expedited Data Transfer with reflective QoS can be requested by AF to expedite the data transfer of larger payload for IP flow(s) of XR application. From a management perspective, enhancements on pre-defined PCC rules, N4 rules, and UPFFunction are needed to support these dynamic traffic handling capabilities.

- Management support for traffic identification and differentiated QoS for multiplexed media flows. XR and interactive media services can send data traffic of different media components with different QoS requirements. Several media flows could be multiplexed on the same end-to-end transport layer connection. In order to uniquely identify each media flow, the IP Packet Filter can further include multiplexed media specific (S)RTP Multiplexed Media Identification Information to differentiate the media flow among multiple media flows that share the same transport layer related elements in the packet filter. PCC rule and N4 Rules can be enhanced or defined by adding Multiplexed Media Identification Information.

According to the analysis above, the potential enhancements may summarized as follows:

- Enhancements on Configuration reading capability of Core Network function to support the handling capability for XRM service.

- Enhancements on Configuration of QFQoSMonitoringControl to expose additional information related to XRM .

- Enhancements on measurements of 5GC for XRM service including XRM related QoS monitoring parameters (e.g., Packet Delay Variation) and XRM related N6 measurements (e.g., Periodicity and N6 jitter).

- Enhancements on Pre-defined PCC rules to support the policy control for XRM service.

- Define pre-defined N4 rules for handling packets for XRM service.

Therefore, it is necessary for a new SA5 work item to provide the management support for Extended Reality and Media service (XRM) Phase 2.

# 4	Objective

The objective of the work item is to specify the details on how to enhance the management aspects for XRM_Ph2, including:

WT-1: Enhancements on configuration management of 5GC to support XRM service related functionalities

WT-1.1: Support on reading capability of core network functions to indicate the supporting capability for the functionalities of XRM service.

WT-1.2: Support pre-defined PCC rules for the policy control of XRM service.

WT-1.3: Support pre-defined N4 rules for packets handling of XRM service

WT-1.4: Enhancement on QoS Monitoring Control to expose additional information related to XRM

WT-2: Enhancements on performance metrics for 5GC to evaluate the performance of XRM service

## TU estimates and dependencies

| Work Task ID | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | SA Dependency (Yes/No/Maybe) | Non-3GPP Dependency |
| --- | --- | --- | --- | --- |
| WT-1 | 1 | No | SA2 | No |
| WT-2 | 1 | No | SA2 | No |

Total TU estimates for the normative phase: 2

Total TU estimates: 2

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 28.540 | Stage 1 enhancement of 5GC NRMs for XRM_Ph2 | Jun 2026(SA#112) |  |
| TS 28.541 | Stage 2 and stage 3 enhancement of 5GC NRMs for XRM_Ph2 | Jun 2026(SA#112) |  |
| TS 28.552 | Define new and/or enhance existing 5G performance measurements for XRM_Ph2 | Jun 2026(SA#112) |  |
| TS 28.554 | Define new and/or enhance existing 5G KPIs for XRM_Ph2 | Jun 2026(SA#112) |  |

# 6	Work item Rapporteur(s)

# 7	Work item leadership

SA5

# 8	Aspects that involve other WGs

Coordination with SA2.

# 9	Supporting Individual Members

Supporting IM name

ZTE

China Mobile

CATT

China Unicom

NEC

Intel

AsiaInfo

Huawei

Deutsche Telekom