Draft ETSI TR 104 084 V0.0.6 (2025-06)

Environmental Engineering (EE);

Study of multi-dimensional network energy efficiency metrics

<

TECHNICAL REPORT

Reference

DTR/EE-EEPS58

Keywords

Energy efficiency, network, metrics

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - APE 7112B

Association à but non lucratif enregistrée à la

Sous-préfecture de Grasse (06) N° w061004871

Important notice

The present document can be downloaded from:
http://www.etsi.org/standards-search

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the prevailing version of an ETSI deliverable is the one made publicly available in PDF format at www.etsi.org/deliver.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at https://portal.etsi.org/TB/ETSIDeliverableStatus.aspx

If you find errors in the present document, please send your comment to one of the following services:
https://portal.etsi.org/People/CommiteeSupportStaff.aspx

If you find a security vulnerability in the present document, please report it through our

Coordinated Vulnerability Disclosure Program:

https://www.etsi.org/standards/coordinated-vulnerability-disclosure

Notice of disclaimer & limitation of liability

The information provided in the present deliverable is directed solely to professionals who have the appropriate degree of experience to understand and interpret its content in accordance with generally accepted engineering or

other professional standard and applicable regulations.

No recommendation as to products and services or vendors is made or should be implied.

No representation or warranty is made that this deliverable is technically accurate or sufficient or conforms to any law and/or governmental rule and/or regulation and further, no representation or warranty is made of merchantability or fitness for any particular purpose or against infringement of intellectual property rights.

In no event shall ETSI be held liable for loss of profits or any other incidental or consequential damages.

Any software contained in this deliverable is provided "AS IS" with no warranties, express or implied, including but not limited to, the warranties of merchantability, fitness for a particular purpose and non-infringement of intellectual property rights and ETSI shall not be held liable in any event for any damages whatsoever (including, without limitation, damages for loss of profits, business interruption, loss of information, or any other pecuniary loss) arising out of or related to the use of or inability to use the software.

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.
The content of the PDF version shall not be modified without the written authorization of ETSI.
The copyright and the foregoing restriction extend to reproduction in all media.

© ETSI yyyy.

All rights reserved.

Contents

Intellectual Property Rights	4

Foreword	4

Modal verbs terminology	4

Executive summary	4

Introduction	4

1	Scope	6

2	References	6

2.1	Normative references	6

2.2	Informative references	6

3	Definition of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Multi-dimensional network energy efficiency	7

5	Existing network energy efficiency metrics	7

6	Performance dimensions	8

6.1	Service volume	9

6.2	Service quality	9

6.3	Service availability	10

6.4	Summary	10

7	Multi-dimensional network energy efficiency metric proposals	11

7.1	General formulation	11

7.2	Metric based on adjustment factor(s)	11

7.3	Multiplicative metric	14

7.4	Separate metrics per performance dimension	14

8	Analysis of metric proposals	17

8.1	Use cases	17

9	Concluding remarks	18

Annex A: Required measurements for adjustment factors	19

A.1	Throughput measurement	19

A.2	Coverage quality measurement	20

Annex B: Comparison of existing and proposed quality adjustment factors	22

B.1	Introduction	22

B.2	Adjustment factors in the present document	22

B.3	Existing factors in ETSI ES 203 228	23

B.4	Observations and discussion	24

Annex C: Example application of metric based on adjustment factor	26

C.1	Methodology	26

C.2	Reference implementation	26

C.3	Optimization performance	27

History	32

# Intellectual Property Rights

Essential patents

IPRs essential or potentially essential to normative deliverables may have been declared to ETSI. The declarations pertaining to these essential IPRs, if any, are publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (https://ipr.etsi.org).

Pursuant to the ETSI Directives including the ETSI IPR Policy, no investigation regarding the essentiality of IPRs, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

Trademarks

The present document may include trademarks and/or tradenames which are asserted and/or registered by their owners. ETSI claims no ownership of these except for any which are indicated as being the property of ETSI, and conveys no right to use or reproduce any trademark and/or tradename. Mention of those trademarks in the present document does not constitute an endorsement by ETSI of products, services or organizations associated with those trademarks.

DECT™, PLUGTESTS™, UMTS™ and the ETSI logo are trademarks of ETSI registered for the benefit of its Members. 3GPP™ and LTE™ are trademarks of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners. oneM2M™ logo is a trademark of ETSI registered for the benefit of its Members and of the oneM2M Partners. GSM® and the GSM logo are trademarks registered and owned by the GSM Association.

# Foreword

This Technical Report (TR) has been produced by ETSI Technical Committee Environmental Engineering (EE).

The present document was developed jointly by ETSI TC EE and ITU-T Study Group 5 and published respectively by ITU and ETSI as Supplement ITU-T Y.XXX [i.z] and ETSI Technical Report ETSI TR XXX XXX [i.z], which are technically equivalent.

# Modal verbs terminology

In the present document "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# Executive summary

# Introduction

A mobile network need to fulfil several service requirements, such as being able to deliver a certain service volume with a given service quality and service availability. A number of different energy efficiency metrics have been defined throughout the years, e.g. for mobile networks in ETSI ES 203 228 [i.1]. These metrics are typically used to assess the energy efficiency of the network over time, but can also be used in network optimization algorithms for improving the network energy efficiency in an automated manner.

These network energy efficiency metrics are typically only based on the amount of service provided, e.g. transported data volume, coverage area, or number of connections. However, the quality of the service provided is seldom reflected, and it is also not always clear if/how these metrics can be combined to give a more complete picture of the energy efficiency of a network.

This technical report studies existing network energy efficiency metrics and outlines proposals and analysis on how mobile network energy efficiency metrics capturing multiple performance dimensions (service volume, service quality, service availability, etc.) can be formulated.

# 1	Scope

This TR studies existing network energy efficiency metrics and outlines proposals how network energy efficiency metrics capturing multiple performance dimensions (service volume, service quality, service availability, etc.) could be formulated. These can be combinations of existing energy efficiency metrics, but also new proposals capturing performance dimensions not considered today. Similar to ETSI ES 203 228 [i.1], the focus is on network energy efficiency metrics to be used for monitoring and/or optimization of live networks.

# 2	References

## 2.1	Normative references

Normative references are not applicable in the present document.

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication ETSI cannot guarantee their long term validity.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	ETSI ES 203 228 V1.4.1 (2022-04): "Environmental Engineering (EE); Assessment of mobile network energy efficiency".

[i.2]	ETSI TS 128 554 V17.7.0 (2022-07): “5G; Management and orchestration; 5G end to end Key Performance Indicators (KPI) (3GPP TS 28.554 version 17.7.0 Release 17)”

[i.3]	ETSI TR 138 913 V14.2.0 (2017-05): “5G; Study on Scenarios and Requirements for Next Generation Access Technologies (3GPP TR 38.913 version 14.2.0 Release 14)”

[i.4]	ETSI TS 132 450 V16.0.0 (2020-08): “Universal Mobile Telecommunications System (UMTS); LTE; Telecommunication management; Key Performance Indicators (KPI) for Evolved Universal Terrestrial Radio Access Network (E-UTRAN): Definitions (3GPP TS 32.450 version 16.0.0 Release 16)”

[i.5]	…

# 3	Definition of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the [following] terms [given in ... and the following] apply:

## 3.2	Symbols

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

## 3.3	Abbreviations

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

BS	Base Station

DL	Downlink

eMBB	enhanced Mobile Broadband

KPI	Key Performance Indicator

MDEE	Multi-Dimensional Energy Efficiency

mMTC	massive Machine Type Communications

MN	Mobile Network

MNO	Mobile Network Operator

QoS	Quality of Service

RAN	Radio Access Network

RSRP	Reference Signal Received Power

UE	User Equipment

UL	Uplink

uRLLC	ultra-Reliable Low Latency Communication

…	…

# 4	Multi-dimensional network energy efficiency

A mobile network (MN) needs to fulfill several service requirements. It need to provide reliable coverage where the users want to access service, it need to provide enough capacity so that the traffic demand can be served, and it need to provide a service quality such that the users are satisfied with their experience of the service. Mobile Network Operators (MNOs) are continuously investing in their networks to make sure these service requirements, which evolve over time, are fulfilled. These investments are typically materialized in equipment consuming energy, and energy efficiency is an important objective. Energy efficiency is often defined as the useful output of a product or service divided by the energy consumption of the product or the service:

Energy Efficiency= Useful workEnergy consumption

A number of different energy efficiency metrics have been defined throughout the years, e.g. for mobile networks in ETSI ES 203 228 [i.1]. These network energy efficiency metrics are typically based on the amount of service provided, e.g. transported data volume, coverage area, or number of connections.

However, a drawback with these metrics is that they only consider the amount of service, but not the quality of the service provided which also is an important useful output of the network. In fact, mobile networks today may have similar data volume energy efficiency, but the user quality may vary greatly.

With the continuous development of networks, new services and new scenarios emerge. The focus of services varies in different phases and scenarios of network development, both in terms of increasing traffic demand to service, and the requirements for service quality. To allow for a more comprehensive energy efficiency evaluation as well as optimization in different network development phases and scenarios, it is desirable that also other performance dimensions than the service volume is reflected, e.g., the service quality.

# 5	Existing network energy efficiency metrics

ETSI ES 203 228 [i.1] defines mobile network energy consumption metrics and mobile network energy efficiency metrics based on a number of performance metrics coupled to use case families.

For the enhanced Mobile Broadband (eMBB) use case family, the energy efficiency metrics Mobile Network data Energy Efficiency (EEMN,DV) and Mobile Network coverage Energy Efficiency (EEMN,CoA) measured in bit/J and m2/J, respectively, are defined. They are derived from performance metrics of the Mobile Network (MN) under investigation, in these cases the total data volume (DVMN) delivered by all its equipment and its global coverage area (CoAMN), respectively, and divided by the mobile network energy consumption (ECMN):

EEMN,DV=DVMNECMN

EEMN,CoA=CoAMNECMN

Mobile Network coverage Energy Efficiency (EEMN,CoA) is mainly used to complement Mobile Network data Energy Efficiency (EEMN,DV) for mobile networks handling low data volumes, in particular in rural or deep rural areas. It is worth noting, that EEMN,CoA is the only one of the defined mobile network energy efficiency metrics that involves a quality factor. In ETSI ES 203 228 [i.1], the coverage quality factor (CoA_Q) measures the performances of the network within the actually covered fraction of the planned total coverage area. UE reports such as failed call attempts are used to determine how well the users within the coverage area are covered, and the coverage quality factor is on a high level defined as:

Q = 1 – "percentage of users/sessions with coverage failure"

For the ultra-Realiable Low Latency Communications (uRLLC) use case family, ETSI ES 203 228 [i.1] defines a mobile network energy efficiency metric based on the end-to-end latency of the mobile network (Te2e,MN). The mobile network latency energy efficiency (EEMN,L) is defined as the inverse ratio of the end-to-end user plane latency and the energy consumed by the MN:

EEMN,L=1Te2e,MN*ECMN

where EEMN,L is expressed in ms–1/J.

For the massive Machine Type Communications (mMTC) use cases, ETSI ES 203 228 [i.1] defines a mobile network energy efficiency metric based on the number of subscribers (NMMTC) registered to the network. In this case, the EE metric is defined as follows:

EEMN,MMTC=NMMTCECMN

For more detailed information on how these mobile network energy efficiency metrics are calculated, and how the mobile network energy consumption (ECMN) as well as the respective performance metrics are measured, please refer to ETSI ES 203 228 [i.1].

# 6	Performance dimensions

A mobile network needs to provide several services, and fulfil many requirements. For long time, we have used mobile networks for voice communication, then for consuming multimedia services, and with the introduction of 5G, we see more and more of machine type communications. All of these communication forms are characterized by different key performance indicators (KPIs), telling how well the network is delivering its services.

The performance indicators can typically be divided into different classes, or dimensions. Three  common performance dimensions are the following:

- Service volume, typically measuring the volume or amount of service provided. Examples can be the data volume delivered, the area covered, the number of supported connections, etc.

- Service quality, measuring the quality of the service provided. Examples are user throughput, coverage quality or connection quality.

- Service availability, typically measuring the availability of the service. An obvious example in a wireless network is the area coverage of the service, but one can also envision other KPIs belonging to this performance dimension such as service availability in time.

One can probably formulate more performance dimensions, but these are a good starting point in order to keep the complexity down. Furthermore, as can be seen of the examples, some KPIs may fall into more than one of the performance dimensions depending on the type of service and how important the different KPIs are. Therefore, how to classify different KPIs typically depends on type of service or use case. This is in line with how energy efficiency KPIs have been defined in ES 203 228 [i.1], as they are typically based on the most important performance KPI for the use case.

To exemplify this, we will examine the three 5G use case families eMBB, mMTC and uRLLC and identify KPIs belonging to the three performance dimensions for each of the use case families.

## 6.1	Service volume

### 6.1.1	Enhanced Mobile Broadband (eMBB)

Mobile broadband is still the absolutely dominating use case for mobile networks, and is expected to continue be so even though we today see other use cases emerging. As we can see from the existing eMBB energy efficiency metrics, two important performance metrics for eMBB are the total data volume (DVMN) delivered and the coverage area (CoAMN), respectively. These can be seen as belonging to the performance dimension service volume, but representing two different sub use cases; capacity sites and coverage sites.

### 6.1.2	Massive Machine Type Communications (mMTC)

For the mMTC use case family, an important performance indicator is the number of connected users, or as already used in ES 203 228 [i.1] the number of subscribers (NMMTC) registered to the network. This is clearly a service volume performance indicator.

### 6.1.3	Ultra-Reliable Low-Latency Communications (uRLLC)

The uRLLC use case family covers many different use cases. In some use cases it is the reliability that is the most important performance matric, while in others it is low latency that is the most important metric, and in some both are equally important. Furthermore, there may also be use cases where data volume is an important performance metric in addition to the reliability and low latency metrics.

So far, ES 203 228 [i.1] has defined the uRLLC energy efficiency metric based on the latency metric (or rather the inverse of it), (1/Te2e,MN). It can therefore be seen as a service volume performance indicator (the lower latency, the better).

However, 3GPP has in their Technical Specification TS 28.554 [i.2] defined an uRLLC energy efficiency metric based on both latency and data volume, (DVMN /Te2e,MN). This KPI is applicable for the cases where, for example, the uRLLC is deployed and operators want to evaluate the Energy Efficiency at different periods of time, such as the busy hours in the morning and the idle hours in the mid night, in which both latency performance and the data volume performance can vary.

## 6.2	Service quality

### 6.2.1	Enhanced Mobile Broadband (eMBB)

Already present in ES 203 228 [i.1], is the coverage quality factor (CoA_Q), which adds a service quality performance dimension to the eMBB sub use case coverage sites. However, as currently defined the coverage quality factor is more a service availability indicator, and an alternative definition may be needed. For capacity sites, where the service volume is measured in data volume, a commonly used service quality indicator is the user throughput, which can be measured both in DL and UL and considered separately or jointly.

### 6.2.2	Massive Machine Type Communications (mMTC)

For mMTC, connection quality is a natural service quality indicator.

### 6.2.3	Ultra-Reliable Low-Latency Communications (uRLLC)

Performance of uRLLC is in addition to low latency also reflected by its reliability. The lower the latency or higher the reliability of uRLLC, the higher its performance is. 3GPP discuss this in TR 28.913 [i.3], that an operator may want to assess energy efficiency with respect to reliability of uRLLC, e.g. for communication services requiring very high reliability, such as Cloud/Edge/Split Rendering, Gaming or Interactive Data Exchanging, wireless road-side infrastructure backhaul etc. Reliability is a very important performance metric for such use cases. Reliability can be considered to be a service quality indicator.

## 6.3	Service availability

### 6.3.1	Enhanced Mobile Broadband (eMBB)

The straight-forward indicator of service availability in an eMBB scenario would be the coverage, e.g. the coverage area where a targeted service quality can be maintained or guaranteed. At the same time, we already use the coverage area as the service volume indicator for coverage sites, which need to be considered in the continued analysis. One way forward could be to use a coverage quality indicator (CoA_Q) as service availability metric, as mentioned in section 6.2.1 above. Further availability indicators could be cell availability or RAN availability, which provides the average time availability duration per cell or the entire RAN, respectively [i.2].

### 6.3.2	Massive Machine Type Communications (mMTC)

For mMTC, the same service availability performance indicators as for eMBB can be considered.

### 6.3.3	Ultra-Reliable Low-Latency Communications (uRLLC)

Also here, the same service availability performance indicators as for eMBB can be considered.

## 6.4	Summary

Table 1 below summarizes the key performance indicators, mapped to performance dimensions, for the different use case families.

Table 1 – Main performance indicators for use case families

| Use case family | Main performance indicators, divided on performance dimensions |  |  |
| --- | --- | --- | --- |
|  | Service volume | Service quality | Service availability |
| Enhanced mobile broadband (eMBB) | Data volume (capacity sites) Coverage area (coverage sites) | User throughput (capacity sites) Coverage quality (coverage sites) | Coverage (area or quality) alternatively Cell or RAN availability |
| Massive machine type communications (mMTC) | Number of connections or subscribers | Connection quality | See eMBB |
| Ultra-reliable low latency communications (uRLLC) | Data volume 1/Latency | Reliability | See eMBB |

# 7	Multi-dimensional network energy efficiency metric proposals

## 7.1	General formulation

To allow for a more comprehensive network energy efficiency evaluation, and to better reflect the full performance of the network in the energy efficiency metric, the network energy efficiency metrics should take multiple performance dimensions into account. A general formulation, based on the high-level definition of energy efficiency in Chapter 4 above, can be as follows.

Energy Efficiency= f (service volume, service quality, service availability, …)Energy consumption

This formulation indicates that the numerator should be a function of the service volume, service quality, service availablility (and potential other performance dimensions), hence reflecting several performance dimensions.

There are several reasons for why this could be beneficial. One is that 5G networks, not only target at carrying more data volume and providing a better coverage, but also at providing differentiation via Quality-of-service (QoS). The current volume-based network energy efficiency metrics fail to capture MNOs’ efforts to provide such QoS differentiation. Another is network optimization, where energy saving algorithms in certain cases may be misled by the volume-based energy efficiency metrics. Having the quality dimension reflected in the network energy efficiency metrics would solve many of these issues.

The service function in the numerator can be defined in different ways, and the following sections will outline some alternatives with their pros and cons.

## 7.2	Metric based on adjustment factor(s)

One straight-forward approach to formulate the multi-dimensional metric is to take the existing volume-based metrics as starting point, and to introduce adjustment factors to reflect the other performance dimensions. The adjustment factors would be dimensionless, in order not to affect the unit of the metric.

As an example, we here illustrate how such an adjusted energy efficiency metric could be formulated for the eMBB use case family, i.e. how to adjust the network data energy efficiency metric to take service quality into account:

Energy Efficiency= Data volumeEnergy consumption×Factorquality

The adjustment factor, in this case Factorquality, should represent the service quality. In this simple example, we define it as the average downlink user throughput in relation to the targeted downlink user throughput that the MNO wants to offer to its customers:

Factorquality= Average downlink user throughputAverage target downlink user throughput

Hence, it describes how well the network is able to deliver the intended service quality, in this case, downlink user throughput. The average downlink throughput is easy to calculate based on standardized counters available in the operation and maintenance (O&M) system (3GPP TS 32.450 [i.4] and 3GPP TS 28.554 [i.2]), which is an advantage. For details about this, see Annex A.

To exemplify, we can take the networks introduced in Table 2 below, and assume that the average targeted downlink user throughput is 10 Mbit/s. We can then calculate the adjustment factors (in this case, Factorquality) and the adjusted energy efficiencies for the different networks. The results are summarized in Table 2. We can see that network A, which is the second best of the three in terms of network data energy efficiency, but is rather poor in delivering the targeted downlink user throghput of 10 Mbit/s, becomes the worst of the three in terms of QoS-adjusted energy efficiency. Network B, which is slightly worse that network A in terms of network data energy efficiency, but good in delivering the targeted downlink user throughput, is better than network A, when we compare the QoS-adjusted energy efficiencies. Finally, network C is the best both before and after QoS adjustment. However, note that its delivered downlink user throughput is much higher than the targeted downlink user throughput. In order not to benefit from this over-provisioning, we have limited the adjustment factor to 1.

Table 2 – Comparison of network data energy efficiency, user throughput, and QoS-adjusted energy efficiency in different networks.

| Network | Network Data Energy Efficiency (GB/kWh) | Average DL User Throughput (Mbit/s) | Factorquality | QoS-adjusted Network Data Energy Efficiency (GB/kWh) |
| --- | --- | --- | --- | --- |
| A | 6.4 | 6.3 | 0.63 | 4.0 |
| B | 6.2 | 9.6 | 0.96 | 6.0 |
| C | 6.7 | 20.1 | 1 | 6.7 |

Similarly, the network coverage energy efficiency metric can be adjusted to take service quality into account:

Energy Efficiency= Coverage areaEnergy consumption×Factorquality

In this case, the adjustment factor, Factorquality, should represent the coverage quality. As mentioned earlier, there is already a definition of coverage quality provided in ES 203 228 [i.1]. However, that definition is focused on the limit where the service drops, and is already used when calculating the coverage area. Hence, it can be seen as an indication of the service availability. What we want here, is a comparative measure of the quality of the coverage. A further discussion on this difference can be found in Annex B.

To design this comparative measure of the quality of the coverage, we here use the cell edge user throughput in relation to the targeted cell edge user throughput:

Factorquality= Cell edge user throughputTarget cell edge user throughput

Hence, it describes how well the network is able to deliver the intended coverage quality, in this case a certain cell edge user throughput. However, to measure cell edge user throughput in a live network is not straight-forward, and a better approach could be to use RSRP reports to represent the coverage quality (e.g., the amount of ”good” RSRP reports in relation to the total amount of RSRP reports). Further details on this can be found in Annex A, however, in the present section we continue to use cell edge throughput for illustrative purposes.

So far, the examples presented has taken one additional performance dimension into account in the adjustment factor, but it is straight-forward to generalize the metric based on adjustment factors to reflect more performance dimensions. For example, the network data energy efficiency metric can be generalized to take both user throughput and coverage quality into account by defining the adjustment factor, Factorquality, as

Factorquality= Factorthoughput × Factorcoverage = 

Average downlink user throughputAverage target downlink user throughput × Cell edge user throughputTarget cell edge user throughput

As an example, in Table 3 we illustrate how the network data energy efficiency can be adjusted based on both user throughput and coverage quality for three different networks. The targeted user throughtput in this example is 10 Mbit/s and the targeted cell edge user throughput is 1 Mbit/s.

Note that networks A and B have average user throughput and cell edge user throghput above the target values, and in order to not benefit from this overprovisioning we limit the adjustment factors to 1. Network C, on the other hand, who originally has the best network data energy efficiency of 8 GB/kWh, delivers both average user througput and cell edge throughput below the targeted values. This results in adjustment factors < 1, and the adjusted network data energy efficiency becomes 6.5 GB/kWh.

Table 3 – Comparison of network data energy efficiency, user throughput, coverage quality and adjusted energy efficiency in different networks.

| Network | Network Data Energy Efficiency (GB/kWh) | Average User Throughput (Mbit/s) | Factortp | Cell Edge User Throughput (Mbit/s) | Factorcov.q | Adjusted Network Data Energy Efficiency (GB/kWh) |
| --- | --- | --- | --- | --- | --- | --- |
| A | 5 | 13 | 1 | 1.2 | 1 | 5*1*1 = 5 |
| B | 7 | 12 | 1 | 1.1 | 1 | 7*1*1 = 7 |
| C | 8 | 9 | 0.9 | 0.9 | 0.9 | 8*0.9*0.9 = 6.5 |

Finally, in the most generalized form, one can also introduce weighting factors for the different performance dimensions used in the adjustment factor. However, exactly how to do this need further study. Two options are

Factorquality=i=1nFiwi

or

Factorquality=i=1nwi×Fi

where 𝐹𝑖 is the adjustment factor of different performance dimensions, e.g. user throughput, coverage quality, etc, and 𝑤𝑖 is the weighting factor of the different performance dimensions, which can be flexibly defined for different scenarios. Note that these two options provide different results, and it need to be further studied which one is to be preferred when more than one performance dimension is used for adjustment.

Similar multi-dimensional metric formulations based on adjustment factor(s) can be obtained for the mMTC and uRLLC use case families.

### 7.2.1	Limitation of adjustment factors

In the examples in Table 2 and Table 3 above, we have limited the adjustment factors to 1. In other words, when the adjustment factor is calculated to be larger than 1, which means that the measured quality is higher than the targeted quality, we have limited the adjustment factor to 1. In the case for DL user throughput, this means we use this formula:

Factorquality=min (1 , Average measured downlink user throughputAverage target downlink user throughput )

Depending on the use case for the network energy efficiency metric, other options could also be considered. The formulation above is suitable when assessing and comparing different cases as one could argue that there should be no benefit from what could be seen as an over-dimensioning. In fact, this could even be taken one step further by inverting the adjustment factor when it becomes larger than 1. The motivation would be that since more resources than necessary have been used in the network (to deliver better quality than targeted), the network is operating with non-optimal resource usage.

However, there could also be use cases for the network energy efficiency metric where such limitation is not desirable. For example, with network optimization algorithms that activate, de-activate, and also re-distribute resources in the network it is necessary to have the full picture of what happens in the network, also when and where quality is over-provisioned, in order to take the correct decisions regarding de-activation and/or re-distribution of resources. An example is given in Appendix III.

Hence, whether limitation of the adjustment factors should be used or not, need to be considered depending on the use case of the network energy efficiency metric.

## 7.3	Multiplicative metric

Another option to formulate a multi-dimensional metric is to formulate the numerator of the general energy efficiency definition in section 7.1 as a multiplication of the performance dimensions. An example of such a metric is mentioned in section 6.1.3 above, 3GPP has in their Technical Specification TS 28.554 [i.2] defined an uRLLC energy efficiency metric based on both latency and data volume, (DVMN /Te2e,MN). The energy efficiency metric is in this case formulated as:

EEMN,DV,L= DVMNTe2e,MN*ECMN

where EEMN,DV,L is expressed in bits/s*J. This KPI is applicable for for the cases where, for example, the uRLLC is deployed and operators want to evaluate the Energy Efficiency at different periods of time, such as the busy hours in the morning and the idle hours in the mid night, in which both latency performance and the data volume performance can vary.

## 7.4	Separate metrics per performance dimension

A third approach could be to not try to capture multiple performance dimensions in a single energy efficiency metric, but instead let multiple single-dimensional energy efficiency metrics represent the different performance dimensions in the network energy efficiency evaluation.

Similar to the present document, 3GPP has in TR 28.880 identified network data traffic, network quality, and network availability as performance dimensions. These correspond to what in the present document is referred to as service volume, service quality, and service availability.

For the network data traffic dimension, 3GPP propose to use the already existing energy efficiency metrics based on data volume (for eMBB) and the number of connections (for mMTC). In ETSI, that would correspond to the already existing energy efficiency metrics in ETSI ES 203 228 [i.1], that are based on service volume. These metrics are also introduced in chapter 5 in the present document.

When assessing network energy efficiency from the network quality dimension, 3GPP propose in TR 28.880 to use the average DL user throughput and the average UL user throughput, respectively, to reflect the network quality dimension. However, note that the network quality performance include also other aspects such as coverage quality, or connection quality, etc, even though here the focus is only on the user throughput aspect.

The resulting new EE KPIs, which are evaluated as the ratio between performance (i.e., user throughput KPIs) and energy consumption, are captured in Table 4 below.

Table 4 - New EE KPIs evaluated from network quality performance dimension

| Performance dimension to evaluate EE KPIs |  | New EE KPIs | Note |
| --- | --- | --- | --- |
| Network quality | UE throughput as the network quality performance indicator | DL RAN UE throughputECper gNB | The unit of this KPI is kbit/(s*J) This new EE KPI is obtained by DL RAN UE throughput divided by the average energy consumption per gNB over the same observation period. The average energy consumption per gNB is obtained by summing up the Energy Consumption (ECgNB) of all the gNBs and then dividing the result by the total number of gNBs. ECgNB is defined in clause 6.7.3.4.2 of TS 128 554 [i.2]. |
|  |  | UL RAN UE throughputECper gNB | The unit of this KPI is kbit/(s*J). This new EE KPI is obtained by UL RAN UE throughput divided by the average energy consumption per gNB over the same observation period. The average energy consumption per gNB is obtained by summing up the Energy Consumption (ECgNB) of all the gNBs and then dividing the result by the total number of gNBs. ECgNB is defined in clause 6.7.3.4.2 of TS 128 554 [i.2]. |

For the availability performance dimension, existing KPI:s in ETSI TS 128 554 [i.2] are cell availability and RAN availability, which provides the average availability time duration per cell or the entire RAN. Hence, it is possible to define new EE KPI:s for the availability dimension by dividing these KPI:s with energy consumption according to Table 5 below.

Table 5 - New EE KPIs evaluated from network availability performance dimension

| Performance dimension to evaluate EE KPIs |  | New EE KPIs | Note |
| --- | --- | --- | --- |
| Network availability | RAN Availability as the availability performance indicator | Radio access network availabilityECper cell | The unit of this KPI is s/J. This new EE KPI is obtained by dividing Radio access network availability KPI by the average energy consumption per cell of the RAN over the same observation period. The average energy consumption per cell of the RAN is obtained by considering summation of energy consumption of all gNB/CU in the RAN for a given time period and dividing it by total number of cells in the RAN that are operational in same time period.. NG-RAN EC is defined in clause 6.7.3.4.1 of TS 128 554 [i.2]. |
|  | Cell Availability as the availability performance indicator | Cell availability divided by average energy consumption per cell in a gNB | The unit of this KPI is s/J. This new EE KPI is obtained by considering Cell Availability KPI in numerator and dividing it by the average energy consumption per cell in a gNB over the same observation period. The average energy consumption per cell in a gNB is obtained by considering energy consumption of gNB for a given time period and dividing it by total number of cells in a gNB that are operational in same time duration/period. ECgNB is defined in clause 6.7.3.4.2 of TS 128 554 [i.2]. |

With the new energy efficiency metrics in Table 4 and Table 5, we have together with the existing energy efficiency metrics in ETSI ES 203 228 [i.1] a collection of single-dimensional energy efficiency metrics reflecting the three performance dimensions service volume, service quality, and service availability. Together, they can provide a picture of multi-dimensional energy efficiency of a network.

# 8	Analysis of metric proposals

## 8.1	Use cases

ETSI ES 203 228 [i.1] aims to provide a better understanding of the Energy Efficiency (EE) of mobile networks in particular considering the networks' evolution in different periods of time. The focus is on metrics for energy efficiency and methods of assessing (and measuring) energy efficiency in operational networks. There are several potential users of this standard, e.g., MNO / CSP (Mobile Network Operators / Communication Service Providers) (and indirectly the suppliers of their network equipment and analytics), telecom regulators, neutral hosts, tower companies, B2B consumers, B2C consumers, etc.

Therefore, in order to properly frame the discussion on the analysis of the potential changes to the metrics and methodology to consider multiple performance dimensions proposed in chapter 7 above, it is relevant to consider some potential relevant mobile network EE assessment use cases.

### 8.1.1		Mobile Network EE based network optimization

Description:  Trace, compare and optimize the EE of a mobile network or a sub-network thereof over time, considering its evolution or changes e.g. traffic, feature activation, HW modernization, parameter/configuration optimization, ...

Key drivers: OPEX reduction, CO2e emission reduction.

### 8.1.2	Mobile Network EE based service delivery optimization

Description:  Trace, compare and optimize the EE of a mobile network or a sub-network thereof over time, considering the service(s) delivered to customers and subscribers.

Challenges: It can become more complicated when the MNO has many different types of customers/subscription/service offering and also with the transversal implementation of e.g. network/service slices, or coverage of special customers like industrial sector.

Key drivers: revenue/cost optimization (and customer satisfaction at lowest possible energy cost) as well as CO2e emission reduction, possibly per service delivered.

### 8.1.3	Mobile Network EE assessment between different types of mobile network architecture

Description: Track and compare EE of different architecture or major configuration of mobile networks or sub-networks thereof within live conditions, e.g. between different deployed clusters with for instance cloud RAN, O-RAN, …

Key drivers: consider EE in term of impacts versus performances as another input criteria for network architecture decision.

### 8.1.4	EE monitoring and reporting

Description: voluntary or mandatory reporting of energy efficiency metric(s), with a focus on delivering wireless connectivity, broadband and services, at low/lower energy/CO2e impacts.

Key drivers: Environmental impact assessment as well as possibly energy dependency for connectivity infrastructure and services.

< to be further developed >

# 9	Concluding remarks

< to be developed >

# Annex A:
Required measurements for adjustment factors

In section 7.2 above, two quality adjustment factors for the eMBB use case were discussed, Factorthroughput, and Factorcoverage. They were defined as follows:

Factorthroughput= Average downlink user throughputAverage target downlink user throughput

and

Factorcoverage= Cell edge user throughputTarget cell edge user throughput

## A.1	Throughput measurement

If we start with Factorthroughput, that is based on the average downlink user throughput. The average downlink throughput is easy to calculate based on standardized counters available in the operation and maintenance (O&M) system, as described in clause 6.3.1 in [i.4] for LTE and clause 6.3.6 in [i.2] for 5G-NR.

To achieve a DL throughput measurement that is independent of file size and gives a relevant result it is important to remove the volume and time when the resource on the radio interface is not fully utilized. (Successful transmission, buffer empty in Figure 1 below). To achieve a throughput measurement that is independent of bursty traffic pattern, it is important to make sure that idle gaps between incoming data is not included in the measurements. That shall be done as considering each burst of data as one sample.

Figure 1 – Illustration of calculation of average DL throughput (source: ETSI TS 132 450 / ETSI TS 128 554).

A summary of the KPI calculation based on counters is provided in Table 6.

Table 6 – Summary of KPI calculation based on ETSI TS 132 450 / ETSI TS 128 554.

| RAT | Counter | Description | KPI calculation | KPI |
| --- | --- | --- | --- | --- |
| LTE | L.Thrp.bits.DL | Total downlink PDCP-layer traffic volume in a cell | (L.Thrp.bits.DL - L.Thrp.bits.DL.LastTTI) / L.Thrp.Time.DL.RmvLastTTI | User Downlink Average Throughput (4G） |
|  | L.Thrp.bits.DL.LastTTI | Downlink PDCP-layer traffic volume sent in the last TTIs before the buffer is empty |  |  |
|  | L.Thrp.Time.DL.RmvLastTTI | Data transmission duration excluding the last TTIs before the downlink buffer is empty |  |  |
| NR | N.ThpVol.DL | Total volume of downlink data sent at the RLC layer in a cell | (N.ThpVol.DL - N.ThpVol.DL.LastSlot) / N.ThpTime.DL.RmvLastSlot | User Downlink Average Throughput (5G） |
|  | N.ThpVol.DL.LastSlot | Downlink volume of RLC SDUs sent in the last timeslot during which the buffer becomes empty |  |  |
|  | N.ThpTime.DL.RmvLastSlot | Data transmission duration excluding the last timeslot during which the downlink buffer becomes empty |  |  |

## A.2	Coverage quality measurement

If we then continue with Factorcoverage, that was in section 7.2 (and in the equation above) based on cell edge user throughput. However, cell edge user throughput cannot easily be calculated from standardized counters as the average downlink user throughput discussed above. As mentioned in section 7.2, an alternative is to use RSRP reports to represent the coverage quality (e.g., the amount of ”good” RSRP reports in relation to the total amount of RSRP reports). Due to the difficulties to establish cell edge user throughput measurements based on standardized counters, and the fact that RSRP reports are readily available in any vendor eNB/gNB, we propose to define Factorcoverage,

Factorcoverage= Coverage quailtyTarget average coverage quality

where the Coverage quaility can be defined as the amount of RSRP reports above a certain threshold divided by the total amount of RSRP reports. This calculation can be simplified by utilizing counters that measure the distribution of RSRP reports within a cell w.r.t. pre-defined ranges, e.g. 0 to n, where the worst reports are categorized in range 0 and the best reports are categorized in range n. KPI calculation can then be carried out as in Table 7, in which index k is chosen to represent the threshold range for sufficiently good RSRP reports.

Table 7 – Summary of coverage quality KPI calculation.

| RAT | Counter | Description | KPI calculation | KPI |
| --- | --- | --- | --- | --- |
| LTE | L.MeasRpts.RSRP.Index0~n | Number of samples with the measured reports RSRP ranging within index 0~n | j=knL.MeasRpts.RSRP.Indexjj=0nL.MeasRpts.RSRP.Indexj | Coverage Quality (4G） |
| NR | N.MeasRpts.RSRP.Index0~n | Number of samples with the measured reports RSRP ranging within index 0~n | j=knN.MeasRpts.RSRP.Indexjj=0nN.MeasRpts.RSRP.Indexj | Coverage Quality (5G） |

This means that in the KPI calculation, the numerator includes all RSRP samples greater than a given threshold (e.g. -110dBm) and the denominator includes all the RSRP samples. Hence the KPI takes a value between 0 and 1, the higher the better coverage quality.

# Annex B:
Comparison of existing and proposed quality adjustment factors

## B.1	Introduction

The objective of the present TR, “Study of multi-dimensional network energy efficiency metrics”, is to investigate how in particular differences in service quality can be reflected in the service-volume based energy efficiency metrics that typically are used today. The motivation being that using metrics only based on service volume may lead to incorrect conclusions, and also that energy efficiency optimizing methods may be mis-lead if only the service volume is considered. One proposal to capture the service quality dimension, introduced in section 7.2, is to introduce adjustment factors and multiply the service volume based metric with them.

However, ETSI ES 203 228 [i.1] already contain some factors and methods that are supposed to reflect quality, e.g., there is a factor for coverage quality and a similar factor for circuit-switched data volume (typically voice traffic). In addition, it can also be observed that the measured packet data volume in ETSI ES 203 228 [i.1] considers QoS classes.

This appendix review and discuss these service quality factors, and try to explain the fundamental difference between the already existing factors for some services in ETSI ES 203 228 [i.1] and the ones being proposed in section 7.2.

## B.2	Adjustment factors in the present document

For the eMBB use case, there exist two energy efficiency metrics in ETSI ES 203 228 [i.1], one based on data volume (data volume / energy consumption) and one based on coverage area (coverage area / energy consumption).

In the present TR, one proposal is to take these existing volume-based metrics as starting point, and to introduce an adjustment factor to reflect the service quality dimension. The adjustment factor would be dimensionless, in order not to affect the unit of the metric. Hence, a metric taking service quality into account would look like this:

Energy Efficiency= Service volumeEnergy consumption×Factorquality

The adjustment factor, Factorquality, should represent the service quality. If we consider data volume, a reasonable service quality indicator is the user throughput. To achieve a dimensionless metric, the adjustment factor is defined as the average downlink user throughput in relation to the targeted downlink user throughput that the MNO wants to offer to its customers:

Factorquality= Average downlink user throughputAverage target downlink user throughput

Hence, it describes how well the network is able to deliver the intended service quality, in this case, downlink user throughput. The average downlink throughput is easy to calculate based on standardized counters available in the operation and maintenance (O&M) system (ETSI TS 132 450 [i.4] and ETSI TS 128 554 [i.2]), see also Annex A for details.

To exemplify, we can take the networks introduced in Table 8 below, and assume that the average targeted downlink user throughput is 10 Mbit/s. We can then calculate the adjustment factors (in this case, Factorquality) and the adjusted energy efficiencies for the different networks. The results are summarized in Table 8. We can see that network A, which is the second best of the three in terms of network data energy efficiency, but is rather poor in delivering the targeted downlink user throghput of 10 Mbit/s, becomes the worst of the three in terms of QoS-adjusted energy efficiency. Network B, which is slightly worse that network A in terms of network data energy efficiency, but good in delivering the targeted downlink user throughput, is better than network A, when we compare the QoS-adjusted energy efficiencies. Finally, network C is the best both before and after QoS adjustment. However, note that its delivered downlink user throughput is much higher than the targeted downlink user throughput. In order not to benefit from this over-provisioning, the adjustment factor is in this case limited to 1:

Table 8 – Comparison of network data energy efficiency, user throughput, and QoS-adjusted energy efficiency in different networks.

| Network | Network Data Energy Efficiency (GB/kWh) | Average DL User Throughput (Mbit/s) | Factorquality | QoS-adjusted Network Data Energy Efficiency (GB/kWh) |
| --- | --- | --- | --- | --- |
| A | 6.4 | 6.3 | 0.63 | 4.0 |
| B | 6.2 | 9.6 | 0.96 | 6.0 |
| C | 6.7 | 20.1 | 1 | 6.7 |

For the network energy efficiency metric based on coverage area, it is natural to assume that the adjustment factor, Factorquality, should represent the coverage quality. One such measure could be the cell edge throughput in relation to the targeted cell edge user throughput. However, it is difficult to measure the cell edge throughput based on counters in the O&M system, instead there is a proposal to use RSRP reports to represent the coverage quality (e.g., the amount of ”good” RSRP reports in relation to the total amount of RSRP reports), see Table 9.

Table 9 – Summary of coverage quality calculation.

| RAT | Counter | Description | KPI calculation | KPI |
| --- | --- | --- | --- | --- |
| LTE | L.MeasRpts.RSRP.Index0~n | Number of samples with the measured reports RSRP ranging within index 0~n | j=knL.MeasRpts.RSRP.Indexjj=0nL.MeasRpts.RSRP.Indexj | Coverage Quality (4G) |
| NR | N.MeasRpts.RSRP.Index0~n | Number of samples with the measured reports RSRP ranging within index 0~n | j=knN.MeasRpts.RSRP.Indexjj=0nN.MeasRpts.RSRP.Indexj | Coverage Quality (5G) |

This means that in the coverage quality calculation, the numerator includes all RSRP samples greater than a given threshold (e.g. -110dBm) and the denominator includes all the RSRP samples. Hence the coverage quality based correction factor always takes a value between 0 and 1, the higher the better coverage quality.

## B.3	Existing factors in ETSI ES 203 228

Already in the current version of ETSI ES 203 228 [i.1] there exist a sort of quality adjustment factors for circuit-switched (CS) service volume and for the coverage area.

### B.3.1	CS traffic

For CS traffic (e.g. CS voice or VoLTE), the data volume is considered as the number of minutes of communications during the time T multiplied by the data rate of the corresponding service and the call success rate. The call success rate is equal to 1 minus the sum of blocking and dropping rates, i.e.:

Call Success Rate=(1-dropping rate)×100 [%]

The dropping includes the intra-cell call failure (rate of dropping calls due to all the causes not related to handover) and the handover failure:

1-dropping rate=1-intracell failure rate(1-handover failure rate)

### B.3.2	Coverage area

Similarly, also the designated coverage area is to be multiplied by a sort of quality factor in ETSI ES 203 228 [i.1].

The coverage quality factor for a base station is based on network failure reports of the UE. The coverage quality factor shall be measured based on coverage failures reported by the appropriate network counters:

CoA_Qdes = 1 - "percentage of users/sessions with coverage failure" 	(9)

The following indicators shall be used to calculate the coverage failure:

- RRC setup failure ratio (Call setup failure ratio).

- RAB setup failure ratio (UE-BS radio interface failure).

- RAB release failure ratio (UE-BS radio interface failure).

A further factor which can indicate a coverage issue is the handover drop ratio. However, a handover drop can have multiple reasons (cell overload, UE speed, etc.). Furthermore, the handover drop rate depends on the network structure (number of neighbour cells). Its calculation requires several additional network parameters and complicates the data collection and analysis significantly. This factor is therefore omitted.

The coverage quality factor for a site is defined as follows:

CoA_Qdes = (1 - RRC setup failure ratio) (1 - RAB setup failure ratio) (1 - RAB release failure ratio)

The needed parameters are specified by ETSI standards and the results can be obtained from the network management and supervision.

## B.4	Observations and discussion

The existing quality factors for CS traffic and coverage area in ETSI ES 203 228 [i.1] are focused on the situation where the service cease to exist, i.e., when the CS call drops or the coverage is so bad that it is impossible to setup a connection. It is fully clear that these situations shall not be included when calculating the service volume (i.e. CS traffic volume and coverage area).

Furthermore, the packet data volume in ETSI ES 203 228 [i.1] is calculated using counters taking the QoS Class Identifier (CQI) into account. However, the effective result of this is also focused on the situation where the service cease to exist (i.e., when the required minimum QoS for a service is not fulfilled). It should also be noted that CQI only applies to certain types of services, and not to best effort services such as web browsing (better QoS means web pages load faster) or video streaming (where a better QoS results in e.g. higher resolution).

To conclude, the already existing factors in ETSI ES 203 228 [i.1] are only focused on the situation when the service cease to exist (by removing these situations when calculating the service volume), but are not able to capture differences in service quality which is the objective of the metrics being studied in the present TR.

Referring to Figure 2 below, the already existing factors in ETSI ES 203 228 [i.1] cut away the red part where there is no service, but can after that only distinguish in the service volume (horizontal) dimension, resulting in that A is better then B despite the fact that B delivers slightly better quality than the targeted service quality while A is far below the targeted service quality. The adjustment factors in the present TR are able to capture the difference in service quality (the vertical dimension) and “punish” A for its poor service quality below the targeted service quality.

Figure 2: Illustrative diagram of service volume vs service quality and what existing adjustment factors and new adjustment factors capture.

# Annex C:
Example application of metric based on adjustment factor

In this contribution, an example of how a multi-dimensional energy efficiency metric is used to analyze and optimize a mobile network in a European metropolitan area is provided. The metric considered is the QoS-adjusted metric introduced in section 7.2, and also shown here:

Energy Efficiency= Data volumeEnergy consumption×Factorquality

with

Factorquality= Average downlink user throughputAverage target downlink user throughput

## C.1	Methodology

Cell-level statistics is collected from all sites in the area under study in order to derive the average downlink data volume and user throughput per site as well as its energy consumption.

Once these statistics is available, the QoS-adjusted energy efficiency metric is computed per site.

The QoS-adjusted energy efficiency metric per site is plotted versus site traffic, and a benchmark curve is fitted to the data points (note that this curve over time need to be updated for future optimization exercises to accommodate for traffic and network changes).

With the generated plot and the benchmark curve, low energy efficiency sites (and root cause) can be identified by comparing the QoS-adjusted energy efficiency metric of each site with the fitted network energy efficiency benchmark.

Energy efficiency improving actions targeted towards the different root causes are carried out on the low energy efficiency sites.

New cell-level statistics is collected, analyzed, and improvements calculated.

## C.2	Reference implementation

Cell-level statistics were collected from a large number of sites in a European metropolitan area, and the QoS-adjusted energy efficiency metric was computed per site, in this case using 20 Mbps as average target downlink user throughput.

Figure 3 illustrates the results of this process, where the QoS-adjusted energy efficiency metric is plotted versus site traffic. Every orange dot represents one site, and the green lines represent current and future energy efficiency benchmark curves.

Figure 3 – QoS-adjusted energy efficiency plotted versus traffic.

Also shown in Figure 3 are the identified low energy efficiency sites and proposed actions to address the root causes and improve the QoS-adjusted energy efficiency. In summary, these can be explained as follows:

- Type I: Low traffic and high downlink user throughput, where network energy saving solutions are not fully utilized / activated.

- Type II: High traffic and low downlink user throughput, where energy saving solutions are fully utilized, and the hardware generation is old.

- Type III: High traffic and high downlink user throughput, where energy saving solutions are fully utilized, and the hardware generation is new.

For Type I sites, the QoS-adjusted energy efficiency metric can be improved by activating energy saving solutions more aggressively. By doing this, the energy consumption is reduced which improve the QoS-adjusted energy efficiency metric. Probably the downlink user throughput is decreased, but the adjustment factor is not affected as it is capped to 1.

For Type II sites, the QoS-adjusted energy efficiency metric can be improved by hardware refresh. This will decrease the energy consumption as new hardware generations typically are more efficient, and probably also improve the downlink user throughput. Both these attributes contributes to improve the QoS-adjusted energy efficiency metric.

For Type III sites, the QoS-adjusted energy efficiency metric can be improved by migrating traffic towards newer RAT generations that are more energy efficient, e.g. 5G.

## C.3	Optimization performance

In this section, it is shown how the mentioned actions for the different site types indicated earlier contributed to network energy efficiency optimization in a trial carried out on the sites presented in Figure 3. Since the traffic mitigation from 2G/3G/4G to 5G is more complex to optimize and the gains more complex to quantify, the focus is on Type I and Type II sites in the following.

For Type I sites, and to quantify the gains brought by the network energy saving solution optimization, two phases were conducted in the trial. During the first phase, in order to estimate the baseline network energy consumption, all the network energy saving solutions were deactivated while taking measurements. In the second phase, the network energy saving solutions were activated and tuned for the measurement campaign.

Figure 4 shows the network energy consumption before and after optimization. It can be seen that the energy saving solution activation and optimization at the Type I sites saves ~1500 kWh per day. Furthermore, Figure 5 shows how these network energy consumption reductions are achieved while maintaining stable key performance indicators (KPIs) related to network performance and user service quality. As a result, the QoS-adjusted energy efficiency metric in the trial area was improved by 17.5%, as can be seen in Figure 6.

Figure 4 – Network energy consumption before (yellow colour) and after (green colour)
Type I site optimization.

Figure 5 – LTE and NR network performance KPIs kept stable.

Figure 6 – QoS-adjusted energy efficiency metric gain due to Type I site optimization.

For Type II sites, hardware refresh was applied. In the trial, two specific sites were selected to quantify the gains obtained by the hardware refresh. The results of the analysis, shown in Figure 7 and Figure 8, indicate that the hardware refresh not only resulted in 35-40% energy saving gain per site (Figure 7), but also an improvement in user service quality as reflected in the QoS-adjusted energy efficiency metric (Figure 8).

Figure 7 – Energy saving due to hardware refresh of Type II sites.

Figure 8 – QoS-adjusted energy efficiency metric gain due to hardware refresh of 
Type II sites.

Annex:
Bibliography

Annex:
Change History

| Date | Version | Information about changes |
| --- | --- | --- |
| 2023-02-10 | 0.0.1 | Early draft created (based on current agreed ITU-T L.Sup.MDEE draft) |
| 2023-06-30 | 0.0.2 | Draft created after joint ETSI/ITU meeting in June |
| 2023-11-23 | 0.0.3 | Draft created after ETSI and ITU meetings in October/November |
| 2024-06-05 | 0.0.4 | Some edits and addition of multiplicative metric, to be discussed at EEPS#65 |
| 2025-06-17 | 0.0.5 | Addition of separate metric per dimension approach, discussion on quality factors, re-arranging some content, etc, as discussed in e-meetings Nov-June. |
| 2025-06-24 | 0.0.6 | Addition of use case chapter based on EEPS(25)000025. Removed resolved comments. |

# History

| Document history |  |  |
| --- | --- | --- |
| <Version> | <Date> | <Milestone> |

Latest changes made on 2022-03-14