3GPP TSG-SA5 Meeting #162	S5-253426

Goteborg, Sweden, 25 - 29 August 2025

Source:	ZTE Corporation

Title:	Pseudo-CR on TR 28.881 Add Key Issue on Intent feasibility check enhancement to support resource reservation

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	V0.0.0

Work Item:	FS_IDMS_MN_Ph4

Comments

This contribution proposes to add structure for TR 28.881 based on SP-250861.

Proposed Changes

* * * First Change * * * *

4	Issue investigations and potential solutions for new areas

Editor's note: this clause will contain issues and potential solutions related to the new scenarios, requirements and generic capability for intent driven management.

4.X Issue#Num: Intent feasibility check enhancement to support resource reservation

4.X.1 Description

In today’s highly dynamic and heterogeneous network environment, Intent-Driven Management Service (IDMS) is emerging as the foundational paradigm for advanced network operations and automation.  Nevertheless, practical deployments reveal a critical capability gap: the absence of robust support for dynamic resource reservation and release.

Intent expression, translation, negotiation, and fulfilment are intrinsically coupled to resource allocation—e.g., bandwidth, capacity and spectrum.  Because these resources are highly time-variant, the current IDMS lacks the ability for the MnS Consumer to proactively reserve or release them on demand.  This shortcoming results in inefficient resource utilisation and reduced flexibility in intent fulfilment. Additionally, after an MnS Producer has classified an intent as “FEASIBLE” following the initial feasibility check, subsequent changes in network resource can render the intent “INFEASIBLE” before intent fulfilment is requested.  This temporal inconsistency, caused by the absence of continuous feasibility validation, will lead to fulfilment failures, undermining the reliability of intent-based operations and degrading overall network performance. To address these deficiencies, IDMS should be enhanced as follows:

- Resource Reservation Support

When the MnS Producer supports resource reservation, the MnS Consumer should be able to include explicit reservation requirements within the expressed intent, ensuring that the necessary resources are reserved prior to execution and thereby guarantee the successful fulfilment.

- Dynamic Resource Monitoring and Real-time Feedback

When the MnS Consumer performs an Intent Feasibility Check without requesting resource reservation, the MnS Producer should continuously monitor resource changes and promptly report updated resource states to the MnS Consumer.  This enables the MnS Consumer to adjust the intent based on feedback before requesting intent fulfilment, preventing potential fulfilment failures caused by resource fluctuations.

4.X.2 Potential requirements

REQ-IDMS_Resource-1: The intent driven MnS should have the capability to enable the MnS consumer to express requirement of resource reservation in an intent.

REQ-IDMS_Resource-2: The intent driven MnS should have the capability to monitor resource changes and provide the feedback related to resource fluctuations.

4.X.3 Potential solutions

4.X.4  Evaluation of potential solutions

* * * End of Changes * * * *