3GPP TSG-SA5 Meeting #159	S5-253461

# Goteborg, Sweden, 25 – 29 August 2025

# Source:	Ericsson

# Title:	DP on Nationwide coverage support of network slicing

# Document for:	Endorsement

Agenda Item:	6.4.27

# 1	Decision/action requested

The group is asked to discuss and endorse the proposal

# 2	References

[1]	3GPP TS 28.622, “Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”, v19.4.0

[2]	3GPP TS 28.541, “Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and Stage 3”, v19.4.0

[3]	GSMA White Paper, “Infrastructure Sharing: An Overview”, June 2019 [Online]. Link: https://www.gsma.com/solutions-and-impact/technologies/networks/gsma_resources/infrastructure-sharing-an-overview/

[4]	BEREC, “Common Position on Mobile Infrastructure Sharing”, June 2019 [Online]. Link: https://www.berec.europa.eu/sites/default/files/files/document_register_store/2019/6/BoR_%2819%29_110_CP_Infrastructure_sharing.pdf

[5]	3GPP TS 23.501, “System architecture for the 5G System (5GS)”, v19.2.0

[6]	3GPP TS 32.130, “Telecommunication management; Network sharing; Concepts and requirements”, v18.3.0

# 3	Rationale

## 3.1 	Background

## 3.1.1 	Network Slicing coverage

One of the service requirements that a network slice shall fulfil is coverage area, i.e. the geographic regions where the slice will be accessible. At the NSI level, this requirement is represented in ServiceProfile, with “coverageArea” attribute. At the NSSI level, this requirement is represented in SliceProfile, with one of the following attributes:

- “TopSliceSubnetProfile.coverageaAreaTAList”, if target NSSI is a E2E NSSI.

- “RANSliceSubnetProfile.coverageaAreaTAList”, if target NSSI is a R-NSSI.

- “CNSliceSubnetProfile.coverageaAreaTAList”, if target NSSI is a C-NSSI.

The network slice provisioning MnS producer needs to map “coverageArea” attribute value (input requirement included in the NSI allocation request) into corresponding “coverageAreaTAList” attribute values (input requirement in the NSSI allocation requests). This mapping is not straightforward since both attributes are of different type.  On the one hand, “coverageArea” attribute is of GeoArea type (3GPP TS 28.622 [1], clause 4.3.55); this means that the coverage requirement is specified as a geographical area that can be represented using a convex polygon. On the other hand, “coverageAreaTAList” attribute is of Tai type (3GPP TS 28.622 [1], clause 4.3.39); in this case, the coverage is specified using a list of TAIs. The “coverageAreaTAList” attribute contained in the SliceProfile allows a network slice subnet provisioning MnS producer to know which TAs are in scope of the requested NSSI. The RAN slice subnet provisioning MnS producer sends this information to corresponding RAN controllers, for them to configure gNBs and cells accordingly.

A gNB can be configured to support multiple slices. This configuration is done with the RAN, and is based on the following principles:

- The gNB serves one or more cells. A cell belongs to a Tracking Area (TA), which is defined by the Tracking Area Code (TAC). TACs are defined within a PLMN. In network sharing scenarios, TAs are uniquely identified by the tuple {PLMN ID, TAC}, which represents the Tracking Area Identifier (TAI). This tuple is specified in the Generic NRM with the Tai datatype (TS 28.622 [1], clause 4.3.39).

- Network slices are defined within a PLMN. A network slice is uniquely identified by the tuple {PLMN ID, S-NSSAI}. This tuple is specified in the 5G NRM with the PLMNInfo datatype (TS 28.541 [2], clause 4.3.41).

- A network slice is linked to a TA. This is because S-NSSAIs are managed per tracking area.

Based on the above principles, it can be noticed that all cells belonging to the same TA must serve the same set of network slices.

## 3.1.2 	Single-operator and multi-operator networks

Network slicing can be provisioned in single-operator (i.e., non-shared) networks or multi-operator (i.e., shared) networks.

As the network densification gets increased and need to keep legacy still prevails, more and more operators are adopting network sharing. Despite the benefits that network sharing brings (increased rollout speed, lower CAPEX/OPEX, quickly expanding coverage, sharing low traffic areas, etc.), it also comes with implications on the operations on the network: alignment on operational priorities, common network planning/evolution strategy, sharing performance data and alarms, etc.

Network sharing can be materialized with different solutions. These solutions can be classified into two main categories:

- Passive sharing: it is the common use by two or more operators of passive elements (unpowered components) of their respective networks. This category can be further divided into:

  - Site sharing

  - Backhaul sharing

- Active sharing: is the common use by two or more operators of active elements of their respective networks. This category can be further divided into:

  - Multi-Operator Radio Access Network (MORAN)

  - Multi-Operator Core Network (MOCN)

  - Core network sharing

  - National roaming

Further details on these solutions and more profound analysis of their pros/cons, from technology, business and regulation standpoints, can be found in GSMA [3] and BEREC [4]

Though all the listed solutions are technically viable in commercial networks, by the end of Rel-18, MOCN is the only mobile network sharing solution supported for 3GPP 5G system. Below there is a summary of MOCN references:

- 3GPP TS 23.501 [5], clause 5.18 details the functionality and network-layer capabilities of 5G MOCN.

- 3GPP TS 32.130 [6] details the impact of 5G MOCN on the management layer. This specification also clarifies the existence of two organizational roles: Master Operator (MOP) and Participant Operator (POP).  The MOP is the actor entrusted with the deployment and operation of shared network elements, while providing network and OAM services to one or more POPs. There is only one MOP per shared RAN.

- The RAN controller takes part in the Master Operator Shared RAN Domain Manager (MOP-SR-DM) [6]

## 3.2 	Problems with existing solution

One of the scenarios foreseen for 5G network slicing is to use this feature to provide isolation with SLA guarantees for services requiring nationwide coverage. Examples of customers asking for the provisioning of dedicated slices servicing a national coverage can be public safety operators or railway operators. This would represent one of the biggest opportunities for operators to monetize slicing, with these customers willing to pay money for slices accessible from any region within the country.

The support of these scenarios is not clear with the current solution that Network Slice NRM fragment offers. There are two main issues in this regard, resulting in problems that are further elaborated below.

## 3.2.1 	Issue #1: A MnS consumer is unable to check whether nationwide coverage is feasible or not

National geography is organized into Tracking Areas (TAs), each uniquely identified by TAI. The collection of TAs forming a given nation/country is defined by the space denoted by T.

A network slice customer (e.g., public safety operator) signs an agreement with a given MNO, for this MNO to become network slice provider. The selected MNO receives from the customer a request to provision a nationwide slice.  With this coverage area requirement, the network slice MnS producer will act depending on whether the mobile network is shared or not with other MNOs.

On the one hand, there is the multi-operator network case, which is aligned with the network sharing described in Section 3.1.2. In this scenario, the selected MNO provides nationwide coverage using cell sites of another MNO(s), i.e. partner MNOs. Figure 1 presents an example of this scenario, with three MNOs operating in the country (MNO #1, #2 and #3).

Figure 1: Nationwide slice provisioning in a multi-operator network.

The following bullets present a summary of these scenario:

- The MNO selected to become network slice provider (MNO #1) is one of the three MNOs operating in the country.

- MNO #1 has a RAN sharing agreement with MNO#2 and MNO#3, who become partner MNOs. This RAN sharing agreement is based on MOCN.

- Each MNO implement one or more network slice subnet MnS producer(s), which collaboratively define the administrative domain corresponding to that domain. The admin domain of an MNO #i is defined by Ti ∈T.

- Ti,j ∈T represents the subset of TAs that are covered by network slice subnet MnS producer #j from MNO #i. ThIn this regard:

  - MNO #1 serves a subset of TAs, defined by T1= jT1,j, with T1∈T.  MNO #1 becomes MOP for T1, with MNO #2 and MNO #3 becoming POPs.

  - MNO #2 serves a subset of TAs, defined by T2= jT2,j, with T2∈T.  MNO #2 becomes MOP for T2, with MNO #1 and MNO #3 becoming POPs.

  - MNO #3 serves a subset of TAs, defined by T3= jT3,j, with T3∈T.  MNO #3 becomes MOP for T3, with MNO #1 and MNO #2 becoming POPs.

  - T1, T2 and T3 are not overlapping.

- The NSMF performs the role of network slice MnS producer and network slice subnet MnS consumer. The NSSMFs perform the roles of network slice subnet MnS producers.

When NSMF receives the request to allocate a nationwide slice, it shall proceed to check whether the sum of serving NSSMFs allows covering the whole nation. This means that the network slice subnet MnS consumer shall have the logic to check whether the i=13jTi,j=T,  where  Ti,j can be read from “NetworkSliceSubnetProviderCapabilities.coverageTAList” attribute set by MnS producer #j from MNO #i. This knowledge is required for the selected MNO to assure it is able to deliver nation-wide coverage capability, and if not, to establish RAN agreements with other MNOs to cover missing geographies.

On the other hand, there is the single-operator network case. In this scenario, the selected MNO is able to provide nationwide coverage without relying on partner MNOs; the assumptions are the same as the ones made for sharing scenario, with the exception that i=1. Figure 2 shows an example of this scenario.

Figure 2: Nationwide slice provisioning in a single-operator network.

Problem: Today, a network slice subnet provisioning MnS consumer is not mandated to have the capability to check whether the sum of all the TAs covered by subscribed network slice subnet provisioning MnS producer(s) allows covering the whole nation geography. Likewise, a MnS producer does not have the capability to report a MnS consumer on whether it is able to provide nationwide slices. In this situation, a MnS consumer has no means to know whether nationwide slice provisioning support is feasible or not.

## 3.2.2 	Issue #2: Long list of TAIs in big countries

The “coverageAreaTAList” attribute  specifies the list of TAIs supporting the requested network slice subnet. This attribute is present in the NetworkSliceSubnetProviderCapabilities IOC (set by network slice subnet provisioning MnS producer) and in the SliceProfile (set by network slice subnet provisioning MnS consumer).

Problem: For big countries, this list can become extremely long (due to large geography), causing handling and visualization challenges on the consumer and producer side. The problem exists for both network and non-network sharing scenarios

## 3.3 	Solution Description

It is proposed to add new attributes to the Network Slice NRM fragment to support the specification of nationwide slice provisioning, in both single and multi-operator networks. The attributes belong to the following constructions:

- ServiceProfile, so that network slice provisioning MnS consumer can univocally express the requirement to provision an NSI offering nationwide coverage. Further details on the solution are found in clause 3.3.1

- SliceProfile, so that a network slice subnet provisioning MnS consumer can univocally express the requirement to provision an NSSI supporting nationwide coverage. Further details on the solution are found in clause 3.3.1

- NetworkSliceSubnetProviderCapabilities IOC, so that a slice subnet provisioning MnS producer can univocally express its capability to provide nationwide coverage. The way the MnS producer get this capability (whether solely or with the help of partner network slice subnet MnS producers, whether these MnS producers are from the same or different MNOs, etc.) shall not be disclosed to the MnS consumer. Further details on the solution are found in clause 3.3.2

## 3.1.1 	Changes in ServiceProfile and SliceProfile

It is proposed to add an attribute that allows representing the nationwide coverage area requirement for NSI (and supporting NSSIs). A name proposal for this new attribute is “nationwideCoverageRequested”. This attribute shall be supported in ServiceProfile (TS 28.541 [5], clause 6.3.3) SliceProfile.CNSliceSubnetProfile (TS 28.541 [2], clause 6.3.23), SliceProfile.RANSliceSubnetProfile (TS 28.541 [2], clause 6.3.24) and SliceProfile.TopSliceSubnetProfile (TS 28.541 [2], clause 6.3.25)

The attribute properties are specified below.

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nationwideCoverageRequested | O | T | T | F | T |

The attribute description is as follows.

| nationwideCoverageRequested | This attribute specifies whether nationwide coverage is required for the requested network slice or network slice subnet. In ServiceProfile: If “nationwideCoverageRequested” attribute is TRUE and the attribute “coverageArea” is present, then the system will ignore “coverageArea” attribute. In CNSiceSubnetProfile, RANSiceSubnetProfile or TopSliceSubnetProfile:If “nationwideCoverageRequested” attribute is TRUE and attribute “coverageAreaTAList” is present, then the system will ignore “coverageAreaTALIst” attribute. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| --- | --- | --- |

As seen, the addition of “nationwideCoverageRequested” attribute allows addressing issue #2.

In addition, the notes in attribute description table clarify relationship of this attribute with “coverageArea” and “coverageAreaTAList”.

## 3.3.2	Changes in NetworkSliceSubnetProviderCapabilities

It is proposed to add an attribute that allows representing the capability of a network slice subnet provisioning MnS producer to support nationwide coverage. A name proposal for this new attribute is “nationwideCoverageSupported”. This attribute shall be present in NetworkSliceSubnetProviderCapabilities IOC (TS 28.541 [2], clause 6.3.36).

The attribute properties are specified below.

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nationwideCoverageSupport | M | T | F | F | T |

The attribute description is as follows.

| nationwideCoverageSupported | This attribute specifies whether it is achievable for the network slice subnet MnS producer to support nationwide coverage. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: N/A isNullable: False |
| --- | --- | --- |

The MnS producer can set the attribute value to TRUE or FALSE, depending on the coverage achievable by the MNO implementing this MnS producer. This achievable coverage depends on the MNO’s planning activities and potential commercial agreements with other partner MNOs. These agreements, if existing, do not need to be disclosed to the MnS consumer (for example, in the Figure 6, the public safety operator is not aware of the commercial agreements that MNO#1 has with other MNOs).

The attribute “nationwideCoverageSupported” allows addressing issue #1.

## 3.4 	Advantages of the proposed solution

- It allows a network slice (subnet) provisioning MnS producer to announce to subscribed MnS consumers about its capability to provision NSI (NSSI) supporting nationwide coverage, without disclosing information to consumers on how this is done.

- It allows a network slice (subnet) provisioning MnS consumer to automatically check the capability of a network slice (subnet) MnS producer to support nation-wide slicing features, with a model-driven approach.

- It allows a network slice (subnet) provisioning MnS consumer to request a NSI (NSSI) supporting nationwide coverage, without the need to list of all TAs associated to such nationwide coverage.

Advantages a) and b) are both related to issue #1. Finally, advantage c) is related to issue #2.

# 4	Detailed proposal

The group is asked to:

- Agree on the issues listed in clause 3.2.

- Endorse the solution proposed in clause 3.3. To that end, the following CR has been prepared.

| TDoc | Title |
| --- | --- |
| S5-253462 | R19 CR TS 28.541 Enhance Network Slice NRM fragment for nationwide coverage support |