3GPP TSG-SA5 Meeting #162	S5-253471

# Goteborg, Sweden, 25 – 29 August 2025

Source:	Ericsson

Title:	Rel-19 pCR TS 28.579 Add solution for authorization of external MnS consumer to access management services

Document for:	Approval

Agenda item:	6.19.21.1

Spec:	3GPP TS 28.579

Version:	1.0.0

Work Item:	MExpo

Comments

This pCR proposes to add a solution for the “Authorization of the external MnS consumer to access management services” use case into TS 28.579.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: " Management and orchestration; Architecture framework".

[3]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[4]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; stage 3".

[5]	3GPP TS 28.622: “Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”.

[6]	TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[7]	Void.

[8]	IETF RFC 7231: “Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content”.

[9]	3GPP TS 29.501: "5G System; Principles and Guidelines for Service Definition; stage 3".

[x1]	3GPP TS 33.122: “Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs”

[x2]	3GPP TS 28.319: “Management and orchestration; Access control for management services”.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

#### 6.X	Authorization of external MnS consumer to access management services

#### 6.X.1	Introduction

To authorize an external MnS consumer to access management services, there are a lot of procedures that need to be executed beforehand. These procedures require the participation of the following entities:

- Admin entities: Operator (in charge of system administrative tasks).

- CAPIF entities: external MnS consumer (API invoker), CCF, and OAuth2.0 server. Whether the OAuth2.0 server is co-located or not with CCF is a deployment choice.

- 3GPP management system entities: MSED (API provider domain), authentication and authorization service producers, MnS producer.

The procedures involved in this use case can be arranged into two groups.

On the one hand, the procedures that constitute pre-conditions for this use case to work. These include:

- Publishing (involved entities: operator, CCF, MSED). The solution for this procedure is described in clause 6.X.2.1

- MSED configuration (involved entities: operator, MSED, authentication & authorization service producers). The solution for this procedure is described in clause 6.X.2.2

- API invoker onboarding (involved entities: API invoker, CCF). The solution for this procedure can be found in clause 5.5.2.2 of TS 29.222 [4] and clause 6.1 of TS 33.122 [x1].

- Service API discovery (involved entities: API invoker, CCF). The solution for this procedure can be found in clause 5.2.2.2 of TS 29.222 [4].

On the other hand, the procedures that define the actual use case, as described in clause 5.2.X.1. These include:

- Service API authorization request (involved entities: API invoker, CCF, OAuth2.0 server). The solution for this procedure is described in clause 6.X.3.

- Service API invocation request (involved entities: API invoker, CCF, OAuth2.0 server, MSED, authentication & authorization service producers, MnS producer). The solution for this procedure is described in clause 6.X.4.

As noted, only the procedures that require the involvement of 3GPP management system entities (and therefore may have an impact on 3GPP management system) are described in detail. The exception is the Service API authorization request (clause 6.X.3), whose illustration is needed to understand the solution for Service API invocation request (clause 6.X.4).

* * * Next Change * * * *

#### 6.X.2	Pre-condition procedures

#### 6.X.2.1	Publishing

The solution for this procedure is illustrated in Figure 6.X.2-1.

Figure 6.X.2-1: Publishing procedure.

1. The operator decides the specific management service information (i.e. which resources/operations/data from which management services) to be made available for external consumption.

2. The operator decides how the selected management service information will be published into the CAPIF. For example, the operator can decide to publish a single management service as one or more service APIs.

3. The operator configures the decisions made in steps 1-2 into the MSED.

NOTE: Steps 1-3 are offline and therefore not subjected to standardization.

4. The MSED APF publishes service API information onto the CCF,  by invoking Publish_Service_API (clause 8.2 of TS 29.222 [4]) over the CAPIF-4 interface. The payload carries ServiceAPIDescription data type (see clause 6.2 of the present document).

#### 6.X.2.2	MSED configuration

This procedure aims to configure authorization information for MSED AEF, for it to become an internal MnS consumer. This is needed for MSED’s AEF to interact with MnS producers, something that occurs when service API invocations from external MnS consumers, once authorized, need to be serviced by target MnS producers. It is worth noting that the requests sent from MSED AEF to a MnS producer will never contain resources/operations that fall outside the specific management service information that the operator has decided to publish in CAPIF; therefore, the permissions assigned to AEF needs to be always based on the published management service information.

The solution for this procedure is illustrated in Figure 6.X.2-2.

Figure 6.X.2-2: MSED configuration procedure

1. The operator configures MSED AEF authorization information using MSAC feature (TS 28.319 [x2]). In particular, the operator creates one Identity resource (NOTE 1), and associates it to one or more Roles, each in turn bound to one or more AccessRule resources (NOTE 2).  This information is configured into the MSED.

NOTE 1: Identity attributes are defined in clause 7.3.1 in TS 28.319 [x2]. The Identity resource created for the MSED AEF is uniquely identified in this example with “aef-msed”.

NOTE 2: The AccessRule attributes are defined in clause 7.3.3 in TS 28.319 [x2].  The “dataNodeSelector” and “operations” attributes of an AccessRule needs to be within the scope of the published management service information (step 1 in Figure 6.X.2-1).

2-7. MSED AEF authentication. The workflow matches steps 101-106 in TS 28.533 [2], Annex D, with AEF taking the role of MnS consumer.

8-10. The MSED AEF authorization. The workflow matches steps 201-203 in TS 28.533 [2], Annex D, with AEF taking the role of MnS consumer. After this procedure, the AEF receives an access token (“token_aef-msed”), defining the permissions of the AEF.

NOTE: The identity parameter present in steps 2, 8 and 9 corresponds to the Identity resource created in step 1.

#### 6.X.3	Service API authorization request

The solution for this procedure is illustrated in Figure 6.X.3-1.

Figure 6.X.3-1: Service API authorization request procedure

1. The external MnS consumer establishes a TLS session with CCF over CAPIF-1e interface, based on mutual authentication.

2. To get access to one or more service APIs, the external MnS consumer needs an access token from CAPIF OAuth2.0 server. To that end, the external MnS consumer sends an access token request to the CCF, using CAPIF_Security_API (clause 8.5 of TS 29.222 [4]). The payload carries AccessTokenReq data type (clause 8.5.4.2.6 of TS 29.222 [4]), which includes “api-invoker-id” (the external MnS consumer’s identity) and optionally “scope” (the requested scope, i.e. the list of service APIs for which the consumer requests access to).

NOTE 1: The “api-invoker-id” is the unique identifier of the external MnS consumer. It is assigned by the CCF during onboarding.

NOTE 2: If the “scope” attribute is not present, the CCF understands that the token is requested to all the service APIs that the external MnS consumer is subscribed to.

3. Upon receiving the request, the CCF communicates with the OAuth2.0 server to check whether the external MnS consumer is authorized, by checking AccessTokenReq attribute values. If authorized, the OAuth2.0 server generates an access token (“token_ext_consumer”) defining the permissions of the external MnS consumer for the service API(s) requested in step 2.

4. The CCF sends the access token response to the external MnS consumer. The response payload carries AccessTokenRsp data type (clause 8.5.4.2.7 of TS 29.222 [4]), which includes “token_ext_consumer”,  “expires_in” and “scope” (the granted scope, i.e. the list of service APIs the consumer has been granted access to).

5. The TLS session is torn down.

#### 6.X.4	Service API invocation request

The solution for this procedure is illustrated in Figure 6.X.4-1.

Figure 6.X.4-1: Service API invocation request procedure.

1. The external MnS consumer establishes a TLS session with MSED AEF over CAPIF-2e interface, based on mutual authentication.

2. The external MnS consumer sends a service API invocation request to the MSED AEF. This request includes the requestor’s identity (“api-invoker-id”), the requestor’s access token (“token_ext_consumer”) and API call (i.e. service API, resource, operation).

3. The MSED AEF validates the request, by checking consumer’s authorization information. The authorization information can be checked with CCF, using CAPIF_Security_API (clause 8.5 of TS 29.222) over CAPIF-3 interface.

4. The MSED AEF sends the access token to the OAuth2.0 server for its verification. The AEF gets notified if verification is successful or not.

5. If token verification is successful, the MSED’s AEF identifies the MnS producer that is responsible for servicing    the service API invocation, and sends it an MnS request. This request includes the requestor’s identity (“aef-msed”), the requestor’s access token (“token_aef-msed”) and API call (i.e. MnS name, resource, operation).

6. Upon receiving the request from MSED, the MnS producer contacts the authentication and authorization service producer for access control purposes. If requestor’s access token is valid, then the authorization service producer finds the Identity resource matching the requestor’s identity, and checks whether any of its constituent AccessRule resources for that Identity fulfils the API call, leading to a successful authorization.

7. If successful authorization, the MnS producer services the MnS request.

8. The MnS producer sends MnS response to the MSED AEF.

9. The MSED AEF sends service API invocation response to the external MnS consumer, informing it about successful/failed service API invocation.

10. The TLS session is torn down.