3GPP TSG-SA5 Meeting #162	S5-253540

Goteborg, Sweden, 25 - 29 August 2025

Source:	Huawei

Title:	pCR TS 28.579 Fix wrong reference

Document for:	Approval

Agenda item:	6.19.21.1

Spec:	3GPP TS 28.579

Version:	1.0.0

Work Item:	MExpo

Comments

In Clause 5.1.2, the original placement of the reference "(defined in TS 29.222 [4])" at the sentence ending could lead to potential misinterpretation, as it might be incorrectly associated with the CAPIF-3, CAPIF-4 and CAPIF-5 interfaces which are defined in TS 23.222, rather than being properly limited to the CAPIF-defined information elements in TS 29.222.

Proposed Changes

* * * First Change * * * *

# 5	Specification level requirements

## 5.1	General

### 5.1.1	Introduction

To expose management services to external MnS consumers using CAPIF, there is a need to define the functional entity responsible for providing the CAPIF API provider domain functions defined in clause 6 of TS 23.222[3]. These functions include the API Exposing Function (AEF), API management function (AMF) and the API publishing function (APF). For the present document, the functional entity responsible for providing these API provider domain functions is referred to as the Management Services Exposure Domain (MSED).

### 5.1.2	Management Services Exposure Domain (MSED)

MSED provides the API provider domains by consuming the CAPIF-3, CAPIF-4 and CAPIF-5 interfaces as shown in Figure 5.1.2-1. The API invoker shown in Figure 5.1.2-1 outside the PLMN trust domain is the external MnS consumer. To enable the exposure of management services using CAPIF, the 3GPP management system is responsible for providing the mapping logic between the management service information and the CAPIF-defined information elements (defined in TS 29.222 [4]) exchanged on the CAPIF-3, CAPIF-4 and CAPIF-5 interfaces (defined in TS 23.222 [3]).

The MSED publishes the service APIs (as a result of mapping the management service information to the CAPIF-defined service API information) to the CAPIF core function (CCF) using the CAPIF-4 interface. The API invokers can discover the published service APIs via the CAPIF-1e interface. The AEF of the MSED enables the API invokers  to consume the published management services as service APIs via the CAPIF-2e interface as shown in Figure 5.1.2-1.

Editor's Note: The interactions or relation between the AEF of the MSED and the MnS producers is FFS

Figure 5.1.2-1: MSED interactions with the CCF and the API invokers (i.e., the external MnS consumers)

As shown in Figure 5.1.2-1, MSED does not implement the CAPIF core function(CCF) but is a consumer of the CAPIF-3, CAPIF-4 and CAPIF-5 interfaces, which are defined outside the 3GPP management system.

Editor's Note: The possible deployment options for MSED are FFS

Traceability: REQ-MEXPO-FUN-01

* * * End of Changes * * * *