3GPP TSG-SA5 Meeting #162	S5-253572

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.201 | CR | 0027 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.201 Add QoS level in Network Sharing Container Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH_MOCN_NetShare |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It should be clarified that the PLMN Identifier in the Network Sharing Charging Information is POP PLMN Identifier not MOP, because the MOP PLMN Identifier has been captured in the charging data request. The measurement of PDCP SDU Data Volume is calculated per PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI as specified in TS 28.552, it’s also necessary to add QoS level in the Network Sharing Container Information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Update PLMN Identifier to POP PLMN Identifier 2.Add QoS level in Network Sharing Container Information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Network Sharing Charging Information is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 6.2.1.4, 6.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [100] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [100].

CEF	Charging Enablement Function

CGF	Charging Gateway Function

CHF	Charging Function

MnS	Management Service

NSPA	Network Slice Performance and Analytics

NWDAF	Network Data Analytics Function

PEC	Post Event Charging

POP                     Participating Operator

Next Change

#### 6.2.1.4	Definition of Network Sharing Charging Information

Specific charging information used for 5G MOCN network sharing charging is provided within the Network Sharing Charging Information.

The detailed structure of the Network Sharing Charging Information can be found in table 6.2.1.4.1.

Table 6.2.1.4.1: Structure of Network Sharing Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| PLMN Identifier | M | This field holds POP PLMN that the performance information is related to. |
| Single NSSAI | OC | This field holds network slice information the performance analytics information is related to. |

Next Change

#### 6.2.1.5	Definition of Network Sharing Container Information

Specific charging information used for 5G MOCN network sharing charging is provided within the Network Sharing Container Information.

The detailed structure of the Network Sharing Container Information can be found in table 6.2.1.5.1.

Table 6.2.1.5.1: Structure of Network Sharing Container Information

| Information Element | Category | Description |
| --- | --- | --- |
| Downlink Data Volume | OC | This field holds downlink cell PDCP SDU Data Volume as described in clause 5.1.3.6.2 of TS 28.552 [270]. |
| Uplink Data Volume | OC | This field holds uplink cell PDCP SDU Data Volume as described in clause 5.1.3.6.2 of TS 28.552 [270]. |
| QoS level | OC | This field holds QoS level as described in clause 5.1.3.6.2 of TS 28.552 [270] which is obtained from the Filter of QoS flow. |
| Number of PDU Sessions requested | OC | This field holds the number of PDU sessions requested by the gNB as described in clause 5.1.1.5.1 of TS 28.552 [270]. |
| Number of PDU Sessions successful | OC | This field holds the number of PDU sessions successfully setup by the gNB from AMF as described in clause 5.1.1.5.2 of TS 28.552 [270]. |

End of Change