3GPP TSG-SA5 Meeting #162	S5-253641

Goteborg, Sweden, 25 - 29 August 2025

Source:	Huawei

Title:	Rel-19 pCR TS 28.561 Update the use case in 5.2.2.2, 5.3.2.2, 5.4.2.3

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS 28.561

Version:	1.0.0

Work Item:	NDT

Comments

This contribution is proposed to update the use cases.

In clause 5.2.2.2, the main purpose is to use NDT to evaluate high-risk network operations which cannot be conducted in the physical network. The example in the last paragraph is not aligned with this scenario.

In clause 5.3.2.2, the main purpose is to use NDT to evaluate how the network responds of events. NDT can model network behaviors and provide information on potential impact of network events. However, the statement given in signaling storm is not aligned with this use case. Examples are given to describe how the signaling storm event can be modelled in NDT considering different scenarios.

In clause 5.4.2.3, it is unclear about what kind of result are generated by NDT and how to use it to measure user satisfaction.

Proposed Changes

* * * First Change * * * *

5.2.2.2	Support for evaluation of high-risk network operations

Each operation for network optimization and maintenance on mobile network may cause potential network failures, including network congestion and network breakdown. High-risk operations are the operations with increased risk of failure, such as potentially dangerous configuration modification, policy modification, software version upgrade, and board switching. Operators should not carry out the high-risk network operations or perform direct optimization in the physical network. The NDT can be used for evaluation of high-risk network operations.

Using NDT, high-risk operations causing potential network failures can be evaluated. NDT MnS Producer can also be requested to create an instance for optimization and verification of network policies and solutions for the further risk avoidance. After simulating/emulating a scenario, NDT MnS Producer can generate the results and report them back to the MnS consumer. Based on results, the MnS consumer can generate possible network policies and solutions to minimize the impact of high-risk operations.

* * * Next Change * * * *

5.3.2.2	Verification of network response to events – NDTVER_02

The network can experience different kinds of events, including failure and non-failure events. A failure event refers to the situation where the network does not behave as expected, e.g., where users in the network are completely unable to get service from the network. A non-failure event refers to the situation where the network experiences an unusual occurrence, but users are still able to get service. The NDT can be used to evaluate how the network responds to any of these events. The NDT can model network behaviors and provide information on potential impact of network events. The NDT can be used to evaluate if a particular event will cause a network failure or whether the network can withstand the event.

The following is an example of events that can be supported:

- 	Signaling storms: A signaling storm is a failure events where a large number of signaling messages suddenly surge in the mobile communication network, resulting in the network processing capacity overload, thus affecting the network performance and stability. During this period, users will repeatedly try to establish the connection until reconnected, thus generating a large number of signaling messages surge suddenly, causing signaling storm. In this scenario, NDT is used to simulate/emulate the network behaviour of surges in signaling requests on the network. For example, NDT can simulate/emulate a sudden surge in the number of UE requests in the scenario of major holidays or network interruption recovery. Operators can also select the multiplication factor for the surge in signaling requests to be injected in NDT to evaluate the network resilience capability in the face of different severity levels of events.

* * * Next Change * * * *

5.4.2.3	Using NDT to generate user experience data

For operators, it is important to accurately measure user satisfaction with the network services from a network usage perspective. When the performance metrics related to customer satisfaction are low, operators are eager to identify the underlying causes and figure out ways to boost performance. Multiple factors influence user satisfaction, including user service quality, network usage experience etc. Digital twin technology can be applied here to model and simulate end - user behaviors within the network, providing valuable insights for improving network services by generating simulation results, e.g. KPIs/PMs related to service experience of end user. The results may be provided to other network functionality, e.g. MDA, for further analysing the customer satisfaction level for network services.

The satisfaction of network service is affected by combining factors, and the digital twin technology is desirable to model and simulate the user usage experiences by integrating multi-domain data sources which include network performance and user experience data, fault prediction. This proactive approach allows CSPs to identify potential detractors who are not satisfied with services on the entire network, monitor the end user journey, gain deeper insights into the end user's needs, and perform refined experience management based on user groups.

* * * End of Changes * * * *