3GPP TSG-SA5 Meeting #162	S5-253913

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0247 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for IAB-node configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MOBILE_IAB_OAM |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is nomative work for Rel-19 study on management of IAB nodes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure IAB-node configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Miss the procedure for IAB-node configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.x, C.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.525: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Requirements".

[3]	ETSI GS NFV-IFA 013 (V4.5.1) (2023-09): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Os-Ma-nfvo reference point - Interface and Information Model Specification".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP TS 22.261 "Service requirements for next generation new services and markets".

[6]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[7]	Void.

[8]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[9]	GSMA NG.116 Generic Network Slice Template v9.0 (2023-04-27) - NG.116-v9.0.pdf (gsma.com).

[10]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects;System Architecture for the 5G System;Stage 2".

[11]	3GPP TS 38.300: "Technical Specification Group Radio Access Network;NR; NR and NG-RAN Overall Description;Stage 2".

[12]	ETSI GS NFV-IFA 014 (V4.2.1) (2021-05): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Network Service Templates Specification".

[13]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[14]	ETSI GS NFV-IFA 011 (V4.3.1) (2022-06): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; VNF Descriptor and Packaging Specification".

[15]	ETSI GS NFV-IFA 008 (V4.3.1) (2022-05): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[16]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[17]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[18]	3GPP TS 38.401: "Technical Specification Group Radio Access Network; NG-RAN; Architecture description".

[x]	3GPP TS 28.315: “Management and orchestration; Plug and Connect; Procedure flows”.

2nd Change

## 7.x	Procedure of IAB-node configuration

The Figure 7.x-1 illustrates the procedure of IAB-node configuration based on its location.

Figure 7.x-1 IAB-node configuration

1) After IAB-node connecting to 3GPP management system via PnC (see TS 28.315 [x] clause 5.6), the provisioning MnS producer of IAB-node sends location information via any of the following notifications:

- notifyMOICreation

- notifyMOIAttributeValueChanges

- notifyMOIChanges

- notifyMOIDeletion

2) The provisioning MnS consumer (management system) perfoms configuration mapping with IAB-node location.

3) The provisioning MnS consumer (management system) provisions IAB-node configuration via any of the following operations upon the reception of notifications:

- createMOI

- modifyMOIAttributes

- deleteMOI

- changeMOIs

4) When IAB-node moves to a new location which may connect to a different management system based on its location (see TS 28.315 [x] clause 5.6), procedures of step 1) to 3) as above are repeated for mobile IAB-node to obtain configuration based on its location.

2nd Change

# C.x	Procedure of IAB-node configuration

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

participant “MS_P” as MnS_Producer

participant "MS_C" as MnS_Consumer

alt MnS producer of IAB-node sends notifyMOICreation with locationInfo to management system (MnS consumer)

MnS_Producer -> MnS_Consumer: 1-a. notifyMOICreation (locationInfo)

End

alt or MnS producer of IAB-node sends notifyMOIAttributeValueChanges with locationInfo to management system (MnS consumer)

MnS_Producer -> MnS_Consumer: 1-b. notifyMOIAttributeValueChanges (locationInfo)

End

alt or MnS producer of IAB-node sends notifyMOIChanges with locationInfo to management system (MnS consumer)

MnS_Producer -> MnS_Consumer: 1-c. notifyMOIChanges(locationInfo)

End

alt or MnS producer of IAB-node sends notifyMOIChanges with locationInfo to management system (MnS consumer)

MnS_Producer -> MnS_Consumer: 1-d. notifyMOIDeletion(locationInfo)

End

MnS_Consumer -> MnS_Consumer: 2. Performs configuation mapping with IAB-node location

alt management system (MnS consumer) provisions the configuration via createMOI() to MnS producer of IAB-node

MnS_Consumer -> MnS_Producer: 3-a. createMOI()

End

alt or management system (MnS consumer) provisions the configuration via modifyMOIAttributes to Mn producer of IAB-node

MnS_Consumer -> MnS_Producer: 3-b. modifyMOIAttributes()

End

alt or management system (MnS consumer) provisions the configuration via modifyMOIAttributes to Mn producer of IAB-node

MnS_Consumer -> MnS_Producer: 3-c. deleteMOI()

End

alt or management system (MnS consumer) provisions the configuration via modifyMOIAttributes to Mn producer of IAB-node

MnS_Consumer -> MnS_Producer: 3-d. changeMOIs()

End

opt If IAB-node moves to new location

ref over MnS_Producer, MnS_Consumer

4. Procedures as above 1-3

end ref

end opt

@enduml

End of change