3GPP TSG-SA5 Meeting #162	S5-254031

Goteborg, Sweden, 25 - 29 August 2025

Source:	Huawei

Title:	pCR TS 28.579 Update use case of logging the management service API invocations to the CCF

Document for:	Approval

Agenda item:	6.19.21.1

Spec:	3GPP TS 28.579

Version:	1.0.0

Work Item:	MExpo

Comments

This pCR proposes to:

- Enhance the use case logging the management service API invocations to the CCF

Proposed Changes

* * * First Change * * * *

### 5.2.3	Logging the management service API invocations to the CCF

#### 5.2.3.1	Definition

When exposing service APIs to external MnS consumers, the operator can benefit from monitoring information related to the service API invocations. The logs of the service API invocations can be consumed by authorized consumers (e.g., the AMF of the MSED or auditing purposes and the charging functions).The monitoring information can include:

- The invoked service API details (e.g., the name, resource(s), and operations)

- The identity of the external MnS consumer who performed the service API invocation (i.e., the API invoker ID)

- The result of the service API invocation (e.g., success or failure)

- The time and duration of the service API invocation

Traceability: REQ-MEXPO-LOG-01, REQ-MEXPO-LOG-02.

* * * Next Change * * * *

## 5.3	Requirements

REQ-MEXPO-FUN-01: 3GPP management system should support the capability to expose management services to external MnS consumers using CAPIF.

REQ-MEXPO-REG-01: The exposure of management services using CAPIF shall have the capability to provide the required mappings between the management service-related information and the CAPIF-defined API provider enrolment related information to enable the registration of MSED to the CCF.

REQ-MEXPO-REG-02: The exposure of management services using CAPIF shall provide the capability to register MSED to the CCF.

REQ-MEXPO-REG-03: The exposure of management services using CAPIF shall provide the capability to deregister MSED from the CCF.

REQ-MEXPO-REG-04: The exposure of management services using CAPIF shall provide the capability to update the registration details of MSED at the CCF.

REQ-MEXPO-PUB-01: The exposure of management services using CAPIF shall have the capability to map the management service-related information into the service API related information to enable the publishing of management services to the CCF.

REQ-MEXPO-PUB-02: The exposure of management services using CAPIF shall have the capability to publish the service APIs to the CCF.

REQ-MEXPO-PUB-03: The exposure of management services using CAPIF shall have the capability to retrieve the published service APIs at the CCF.

REQ-MEXPO-PUB-04: The exposure of management services using CAPIF shall have the capability to update the published service APIs at the CCF.

REQ-MEXPO-PUB-05: The exposure of management services using CAPIF shall have the capability to unpublish the service APIs from the CCF.

REQ-MEXPO-LOG-01: The exposure of management services using CAPIF shall support the capability to create logs based on the service API invocations by the external MnS consumers.

REQ-MEXPO-LOG-02: The exposure of management services using CAPIF shall support the capability to log the service API invocations to the CCF.

* * * End of Changes * * * *