3GPP TSG-SA5 Meeting #162	S5-254071

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.405 | CR | 0044 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28405 Rapporteur’s clean up |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TraceQoE_OAM |  |  |  |  |  | Date: |  |  | 2025-08-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clean up Rel-19 before the end of Rel-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed references according to 21.801. Removed abbreviations in clause 3.2 that exist in 21.905. Editorial changes. Correction of references. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification would not conform to 21.801 and 21.905. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 3.1, 3.3, 4.1.1, 4.1.2.1, 4.1.2.2, 4.2.1, 4.2.2.1, 4.2.2.2, , 4.3.1, 4.3.3.1, 4.4.2.2, 4.5.1, 4.5.2, 4.5.4, 4.6.1.1, 4.6.2.1, 5.5, 5.8, and 5.15. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of S5-253694. |  |  |  |  |  |  |  |  |

1st change

# 1	Scope

The present document addresses the mechanisms used for the function Quality of Experience (QoE) measurement collection in 3GPP networks . The measurements that are collected are DASH (see TS 26.247 [6]), MTSI MTSI (see TS 26.114 [7]) and Virtual Reality (VR) (see TS 26.118 [13]) measurements.

The function includes collecting QoE information from UEs frequenting a specified area or an individual UE for a specified end user service/end user service type. The document describes the activation and deactivation of a network request session, UE request session and recording session and also the reporting of recorded information , see TS 28.404 [2].

Next change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.404: "Telecommunication management; Quality of Experience (QoE) measurement collection; Concepts, use cases and requirements".

[3]	3GPP TS 28.308: "Management of Quality of Experience (QoE) measurement collection Integration Reference Point (IRP); Information Service (IS)".

[4]	3GPP TS 25.331: "Radio Resource Control (RRC) protocol specification".

[5]	3GPP TS 27.007: "AT command set for User Equipment (UE)".

[6]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[7]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[8]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[9]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[10]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[11]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[12]	3GPP TS 38.413: " NG-RAN; NG Application Protocol (NGAP)".

[13]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[14]	Void

[15]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

Next change

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Network session: A network session is used when an NF requests a UE to perform QoE Measurement Collection for a specified end user service type. Supported end user service types are defined in clause 5.8.

Next change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

DM	Domain Manager

MCE	Measurement Collector Entity

NB	Node B

QMC	QoE Measurement Collection

Next change

### 4.1.1	Activation of measurement collection job and reporting of collected information in UTRAN

The parameters for the network request session are sent from the management system to the RNCs that host the cells that are included in the collection job request in the activateAreaQMCJob operation, see TS 28.308 [3]. The RNC starts a network request session, with the Network request session id, see TS 28.308 [3] given in activateAreaQMCJob operation, see TS 28.308 [3]. For the duration of the network request session, the RNC(s) checks for connections where the UE has the UE Application Layer Measurement Capability, see TS 25.413 [10]. The UE Application Layer Measurement Capability is sent from the UE to RNC (via the core network) in message INITIAL UE MESSAGE, see TS 25.413 [10].

When a session is found that has a UE with the UE Application Layer Measurement Capability, the RNC starts a UE request session by sending a MEASUREMENT CONTROL message, see TS 25.331 [4] to the UE via the NB.

The AT command +CAPPLEVMC, see TS 27.007 [5] activates provisioning of measurement reporting from the lower layers to the application within the UE by an unsolicited result code +CAPPLEVMC. When information about a measurement report is received in the UE from the network, the unsolicited result code +CAPPLEVMC contains the relevant parameters (service_type, start_stop-indication and config-file & config-file length if applicable) that are provided from the lower layers in the UE to the application.

Figure 4.1.1-1: QMC activation and reporting in UTRAN

1.	The NM sends activateAreaQMCJob to DM/EM that controls the impacted RNC(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qMCTarget, qoEReference and QMC configuration file.

2.	The DM/EM forwards activateAreaQMCJob to impacted RNC(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qMCTarget, qoEReference and QMC configuration file.

3.	The RNC checks for connections where the UE has the UE Application Layer Measurement Capability, see TS 25.413 [10] that match the criteria for serviceType in the activateAreaQMCJob.

4.	When a connection is found that has the UE Application Layer Measurement Capability, see TS 25.413 [10], the RNC start a UE request session and stores the associated QoECollectionEntityAddress, sends the message RRCConnectionReconfiguration to the UE, and includes the following: serviceType, qoEReference and QMC configuration file.

5.	The access stratum in the UE sends the AT command +CAPPLEVMC to application level and includes the following: serviceType, qoEReference and QMC configuration file.

6.	When the application in the serviceType starts, the QMC is initiated.

7.	The application layer sends the AT command +CAPPLEVMR, see TS 27.007 [5], including a recording session indication that a session has started to the access stratum.

8.	The UE sends the message MeasurementReport including the recording session indication to the RNC.

9.	The RNC sends a notification including the recording session indication to the NM.

10.	When the QMC is completed, the recorded information is collected in a QMC report, see TS 26.247 [6] for DASH or TS 26.114 [7] for MTSI, including qoEReference and recordingSessionId. The qoEReference, Client Id, see TS 26.247 [6] for DASH or TS 26.114 [7] for MTSI, in the reporting container (that represent the UE request session), and recordingSessionId are needed in the QMC collection entity for post processing purposes.

11.	The application layer sends the AT command +CAPPLEVMR, see TS 27.007 [5], including qoEReference and the QMC report to the access stratum.

12.	The UE sends the message MeasurementReport including qoEReference and the QMC report to the RNC.

13.	The RNC sends the QMC report to the MCE associated to the qoEReference.

Next change

#### 4.1.2.1	Handover between cells within an RNC

When handover is made, the measurement area is checked.

-	If the mobile is inside the measurement area after the handover, the RNC sends in an indication that the UE is inside the measurement area in the RRCConnectionReconfiguration message, see TS 25.331 [4] to the UE. The UE send the +CAPPLEVMC AT command, see TS 27.007 [5] to the application to inform that recording shall start if the application becomes active (when the UE moves into the measurement area).

-	If the mobile has moved outside the measurement area, there is no indication that the UE is inside the measurement area in the RRCConnectionReconfiguration message, see TS 25.331 [4] from the RNC to the UE. The UE sends the +CAPPLEVMC AT command, see TS 27.007 [5] to the application to inform that recording shall not be started even if the application becomes active.

Next change

#### 4.1.2.2	Handover between RNCs

The figure 4.1.2.2-1 and the text below describes the handling at handover between RNCs.

Figure 4.1.2.2-1: Handling of QMC activation in case of handover in UTRAN

1.	Source RNC sends the message HandoverRequest to Target RNC and includes areaScope, qoECollectionEntityAddress and a recording session indication indicating whether a recording session has started.

2.	If a recording session has not started, the target RNC checks if the target cell is inside the areaScope or not.

3.	The target RNC sends the message Handover Request Ack to the source RNC.

4.	The source RNC sends the message RRCReconfiguration to the UE.

5.	The access stratum in the UE sends the AT command +CAPPLEVMC, see TS 27.007 [5], to application level.

6.	The UE sends the message RRCReconfigurationComplete to the RNC.

7.	When the application becomes active, QMC is initiated if the UE is inside the areaScope.

8.	The application layer sends the AT command +CAPPLEVMR, see TS 27.007 [5],  including a recording session indication to the access stratum.

9.	The UE sends the message MeasurementReport including the recording session indication to the RNC.

10.	The RNC sends a notification including the recording session indication to the NM.

11.	When the QMC is completed, the recorded information is collected in a QMC report, see TS 26.247 [6] for DASH, TS 26.114 [7] for MTSI including qoEReference and recordingSessionId. The qoEReference, Client Id , see TS 26.247 [6] for DASH, TS 26.114 [7] for MTSI in the reporting container (that represent the UE request session) and recordingSessionId are needed in the QoE collection entity for post processing purposes.

12.	The application layer sends the AT command +CAPPLEVMR, see TS 27.007 [5], including qoEReference and the QMC report to the access stratum.

13.	The UE sends the message MeasurementReport including qoEReference and the QMC report to the RNC.

14.	The RNC sends the QMC report to the MCE associated to the qoEReference.

Next change

### 4.2.1	Activation of measurement collection job and reporting of collected information in LTE

The parameters for the network request session are sent from the management system to the eNBs that host the cells that are included in the collection job request in the activateAreaQMCJob operation, see TS 28.308 [3]. The eNB starts a network request session, with the Network request session id, see TS 28.308 [3], given in activateAreaQMCJob operation, see TS 28.308 [3]. For the duration of the network request session, the eNB(s) checks for connections where the UE has the QoE-MeasReport capability, see TS 36.331 [8], for collection of streaming services or the QoE-MTSI-MeasReport capability, see TS 36.331 [8], for collection of MTSI services. The UE capability is sent from the UE to eNB via the core network in message UE CAPABILITY INFO INDICATION, see TS 36.413 [9].

When a session is found that has a UE with the wanted UE capability, the eNB starts a UE request session and sends a RRCConnectionReconfiguration, see TS 36.331 [8], to the UE.

The AT command +CAPPLEVMC, see TS 27.007 [5], activates provisioning of measurement reporting from the lower layers to the application within the UE by an unsolicited result code +CAPPLEVMC, see TS 27.007 [5]. When information about a measurement report is received in the UE from the network, the unsolicited result code +CAPPLEVMC, see TS 27.007 [5], contains the relevant parameters (service_type, start_stop-indication and config-file & config-file length if applicable) that are provided from the lower layers in the UE to the application.

Figure 4.2.1-1: QMC activation and reporting in LTE

1.	The NM sends activateAreaQMCJob to DM/EM that controls the impacted eNB(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file.

2.	The DM/EM forwards activateAreaQMCJob to impacted eNB(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file.

3.	The eNB checks for connections where the UE has the UE capability, see TS 36.413 [9], that match the criteria for serviceType in the activateAreaQMCJob.

4.	When a connection is found that has the wanted UE capability, see TS 36.413 [9], the eNB starts a UE request session and stores the associated QoECollectionEntityAddress, sends the message RRCConnectionReconfiguration to the UE, and includes the following: serviceType, qoEReference and QMC configuration file.

5.	The access stratum in the UE sends the AT command +CAPPLEVMC, see TS 27.007 {5], to application level and includes the following: serviceType, qoEReference and QMC configuration file.

6.	When the application in the serviceType starts, the QMC is initiated.

7.	The application layer sends the AT command +CAPPLEVMR, see TS 27.007 [5] including a recording session indication that indicates that a session is started to the access stratum.

8.	The UE sends the message MeasReportAppLayer including the recording session indication to the eNB.

9.	The eNB sends a notification including the recording session indication to the NM.

10.	When the QMC is completed, the recorded information is collected in a QMC report, see TS 26.247 [6] for DASH, TS 26.114 [7] for MTSI, including qoEReference and recordingSessionId. The qoEReference, Client Id, see TS 26.247 [6] for DASH, TS 26.114 [7] for MTSI, in the reporting container (that represent the UE request session), and recordingSessionId are needed in the QMC collection entity for post processing purposes.

11.	The application layer sends the AT command +CAPPLEVMR, see TS 27.007 [5], including qoEReference and the QMC report to the access stratum.

12.	The UE sends the message MeasReportAppLayer including qoEReference and the QMC report to the eNB.

13.	The eNB sends the QMC report to the MCE associated to the qoEReference.

Next change

#### 4.2.2.1	Handover between cells within an eNB

When handover is made and no Recording session id is provided, the measurement area is checked.

-	If the mobile is inside the measurement area after the handover, the eNB sends in an indication that the UE is inside the measurement area in the RRCConnectionReconfiguration message, see TS 36.331 [8], to the UE. The UE send the +CAPPLEVMC AT command , see TS 27.007 [5], to the application to inform that recording shall start if the application becomes active (when the UE moves into the measurement area).

-	If the mobile has moved outside the measurement area, there is no indication that the UE is inside the measurement area in the RRCConnectionReconfiguration message, see TS 36.331 [8], from the eNB to the UE. The UE sends the +CAPPLEVMC AT command, see TS 27.007 [5], to the application to inform that recording shall not be started even if the application becomes active.

Next change

#### 4.2.2.2	Handover between eNBs

The figure 4.2.2.2-1 and the text below describe the handling at handover between eNBs.

Figure 4.2.2.2-1: Handling of QMC activation in case of handover in LTE

1.	Source eNB sends the message HandoverRequest to Target eNB and includes areaScope and qoECollectionEntityAddress.

2.	The target eNB checks if the target cell is inside the areaScope or not.

3.	The target eNB sends the message HandoverRequestAcknowledge to the source eNB.

4.	The source eNB sends the message RRCConnectionReconfiguration to the UE.

5.	The access stratum in the UE sends the AT command +CAPPLEVMC, see TS 27.007 [5], to application level.

6.	The UE sends the message RRCConnectionReconfigurationComplete to the eNB.

7.	When the application becomes active, QMC is initiated if the UE is inside the areaScope.

8.	The application layer sends the AT command +CAPPLEVMR, see TS 27.007 [5], including a recording session indication indicating that a session has started to the access stratum.

9.	The UE sends the message MeasReportAppLayer including the recording session indication to the eNB.

10.	The eNB sends a notification including the recording session indication to the NM.

11.	When the QMC is completed, the recorded information is collected in a QMC report, see TS 26.247 [6] for DASH, TS 26.114 [7] for MTSI, including qoEReference and recordingSessionId. The qoEReference, Client Id, see TS 26.247 [6] for DASH, TS 26.114 [7] for MTSI, in the reporting container (that represent the UE request session) and recordingSessionId are needed in the QoE collection entity for post processing purposes.

12.	The application layer sends the AT command +CAPPLEVMR, see TS 27.007 [5], including qoEReference and the QMC report to the access stratum.

13.	The UE sends the message MeasReportAppLayer including qoEReference and the QMC report to the eNB.

14.	The eNB sends the QMC report to the MCE associated to the qoEReference.

Next change

### 4.3.1	Activation of measurement collection job in UTRAN

In Signalling based activation, the QoE measurement collection is sent to the UE from the OAM via CN. The messages from EMS propagated to HSS, MSE Server/SGSN and RNC. QoE measurement collection activation request is propagated to UE.

When HSS activates the QoE measurement collection for a job, the MSC Server/SGSN can send the QoE measurement collection to the UE through RNC, the following configuration parameters shall be included in the message: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file.

1)	The NM sends activateAreaQMCJob to DM/EM that controls the impacted RNC(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qMCTarget, Individual UE Id, qoEReference and QMC configuration file.

2)	The DM/EM sends activateAreaQMCJob to HSS that controls the impacted RNC(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, Individual UE Id,qoEReference and QMC configuration file.

3)	The HSS sends an Insert Subscriber Data to the MSC Server/SGSN that is registered in the HSS in a previous Update Location Request message.

4)	The MSC Server/SGSN starts finding the UE and check if that the UE capability matches the criteria for ServiceType in activateAreaQMCJob

5)	The MSC Server/SGSN sends message CN INVOKE TRACE to the RNC and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget,qoEReference and QMC configuration file.

6)	The RNC checks for connections where the UE has the UE Application Layer Measurement Capability that match the criteria for serviceType in the activateAreaQMCJob.

7)	The RNC start a UE request session and stores the associated QoECollectionEntityAddress, sends the message RRCConnectionReconfiguration to the UE, and includes the following: serviceType, qoEReference and QMC configuration file.

8)	The access stratum in the UE sends the AT command +CAPPLEVMC, see TS 27.007 [5], to application level and includes the following: serviceType, qoEReference and QMC configuration file.

9)	When the application in the serviceType starts, the QMC is initiated.

10)	The application layer sends the AT command +CAPPLEVMR, see TS 27.007 [5], including a recording session indication that a session has started to the access stratum.

11)	The UE sends the message MeasurementReport including the recording session indication to the RNC.

12)	The RNC sends a notification including the recording session indication to the NM.

13)	When the QMC is completed, the recorded information is collected in a QMC report, including qoEReference and recordingSessionId. The qoEReference, Client Id, in the reporting container (that represent the UE request session), and recordingSessionId are needed in the QMC collection entity for post processing purposes.

14)	The application layer sends the AT command +CAPPLEVMR, see TS 27.007 [5], including qoEReference and the QMC report to the access stratum.

15)	The UE sends the message MeasurementReport including qoEReference and the QMC report to the RNC.

16)		The RNC sends the QMC report to the MCE associated to the qoEReference.

Next change

#### 4.4.2.2	Activation of QoE measurement task after UE attachement to the network

The QoE measurement collection is sent to the UE from the OAM via CN. The messages from EMS propagated to HSS, MME and eNodeB are the same as previous section. QoE measurement collection activation request is propagated to UE finally.

When MME can send the QoE measurement collection to the UE through eNodeB, the following configuration parameters shall be included in the message: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file. QoE measurement collection activation request is propagated to UE.

1)	The EMS sends activateAreaQMCJob to HSS that controls the impacted eNB(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file.

2)	The HSS inserts subscriber related data and forwards it to the MME.

3)	The MME forwards activateAreaQMCJob to impacted eNB(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, pLMNTarget, qoETarget, qoEReference and QMC configuration file.

4)	The eNB checks if the UE capability that matches the criteria for serviceType in the activateAreaQMCJob.

5)	If the UE has the wanted UE capability, the eNB starts a UE request session and stores the associated QoECollectionEntityAddress, sends the message RRCConnectionReconfiguration to the UE, and includes the following: serviceType, qoEReference and QMC configuration file.

6)	The access stratum in the UE sends the AT command +CAPPLEVMC, see TS 27.007 [5], to application level and includes the following: serviceType, qoEReference and QMC configuration file.

7)	When the application in the serviceType starts, the QMC is initiated.

8)	The application layer sends the AT command +CAPPLEVMR, see TS 27.007 [5], including a recording session indication that indicates that a session is started to the access stratum.

9)	The UE sends the message MeasReportAppLayer including the recording session indication to the eNB.

10)	The eNB sends a notification including the recording session indication to the NM.

11)	When the QMC is completed, the recorded information is collected in a QMC report, see TS 26.247 [6] for DASH, TS 26.114 [7] for MTSI, including qoEReference and recordingSessionId. The qoEReference, Client Id, see TS 26.247 [6] for DASH, TS 26.114 [7] for MTSI, in the reporting container (that represent the UE request session), and recordingSessionId are needed in the QMC collection entity for post processing purposes.

12)	The application layer sends the AT command +CAPPLEVMR, see TS 27.007 [5], including qoEReference and the QMC report to the access stratum.

13)		The UE sends the message MeasReportAppLayer including qoEReference and the QMC report to the eNB.

14)		The eNB sends the QMC report to the MCE associated to the qoEReference.

Next change

### 4.5.1	Activation of measurement collection job and reporting of collected information in NR

Management Based Activation enables collection of application layer measurements from the UEs in the specified area for specified end user service type.

The parameters for the network request session are sent from the MnS Consumer via the management system to the gNBs that host the cells that are included in the QoE Measurement Collection job request in the Create MOI QMC operation, see TS 28.622 [15]. A QMC Job is activated by creating a QMCJob object instance in the MnS producer.

Figure 4.5.1-1 and the text below describe the activation of QoE measurement collection.

Figure 4.5.1-1: QMC activation example for Management Based Activation and Reporting in NR

1)	A QoE Measurement Collection Job begins when the MnS Consumer sends createMOI request for QMCJob to the MnS Producer serving the impacted gNB(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, qoEReference, mDTAlignmentInformation, availableRANqoEMetrics and qMCConfigFile, and mBSCommunicationServiceType. Step 2 and step 3 are asynchronous in time with respect to Step 1; they may occur before or after step 1.

NOTE: The RAN visible QoE measurements are supported for the DASH streaming and VR services.

2)	Application level measurement configuration. The Application in the UE shall send AT command +CAPPLEVMCNR,see TS 27.007 [5] containing the parameter <n> set to 1 to the Access Stratum. This enables the the Access Stratum to present an unsolicited result code to the Application at a later time. Step 2 and step 3 are asynchronous in time with respect to Step 1; they may occur before or after step 1.

3)	UE registration procedure. The UE shall register QMC capability with the gNB by setting qoe-Streaming-MeasReport to “supported” in UE-NR-Capability, see TS 38.331 [11]. Step 2 and step 3 are asynchronous in time with respect to Step 1; they may occur before or after step 1.

4)	The gNB starts a network request session, with the QoE Reference, see TS 28.622 [15]given in createMOI for QMCJob, see TS 28.622 [15]. For the duration of the network request session, the gNB(s) checks for connections where the UE has qoe-Streaming-MeasReport set to “supported”, see TS 38.331 [11,] in step 2.

5)	If a UE has the wanted capability, the gNB stores the associated qoECollectionEntityAddress and starts a UE request session by sending RRCReconfiguration to the UE including serviceType, measConfigAppLayerId (corresponding to the value of qoEReference from step 4), measConfigAppLayerContainer, ran-VisibleParameters and availableRANqoEMetrics.

If QoE measurement configuration pertains to MBS communication service, the gNB translates the qoECollectionEntityAddress into qoECollectionEntityIdentity and includes qoECollectionEntityIdentity in the RRCReconfiguration.

NOTE: The IE measConfigAppLayerId indicates the identity of the application layer measurement configuration, see TS 38.331 [11].

6)	The access stratum in the UE sends an unsolicited response to the Application including app-meas_service_type, meas_config_app_layer_id and app-meas_config_file. These IEs map directly to the IEs in step 5. The unsolicited response is for the AT command +CAPPLEVMCNR, see TS 27.007 [5], which is sent from Application in step 2.

7)	The QMC Job remains inactive until the Application for the specified serviceType initiates service.

8)	When the Application begins service, it sends the AT command +CAPPLEVMRNR, see TS 27.007 [5], including meas_config_app_layer_id and qoe_measurement_status to indicate a recording session has started, to the access stratum.

9)	The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId, appLayerSessionStatus, and qoECollectionEntityIdentity to the gNB to indicate a recording session has started. If mDTAlignmentInformation is received in step 1, the gNB may activate the MDT measurements that are to be aligned with the QoE measurements performed by the UE.

10)	While active, the Application collects measurement reports. When the QMC is completed or at the end of period for periodic report, the recorded information is formatted in a QMC report, see TS 26.247 [6] for DASH, TS 26.114 [7] for MTSI or TS 26.118 [13] for VR.

11)	When a formatted measurement report is available, the Application sends the AT command +CAPPLEVMRNR, see TS 27.007 [5] including meas_config_app_layer_id, qoe_measurement_status, and app-meas_report to the Access Stratum. If the QMC Job has ended, qoe_measurement_status is set to indicate the Job has ended.

12)	When the AT command +CAPPLEVMRNR, see TS 27.007  is received from the Application, the Access Stratum sends the message MeasurementReportAppLayer including measConfigAppLayerId, appLayerSessionStatus, measReportAppLayerContainer, and qoECollectionEntityIdentity to the gNB.

13)	The gNB translates the qoECollectionEntityIdentity into qoECollectionEntityAddress if it is needed, and sends the QMCRecord to the MCE associated to the qoEReference. The report contains the QoEReference from step 4, a time stamp, and measReportAppLayerContainer from step 12.

Next change

### 4.5.2	Void

Next change

### 4.5.4	Temporary stop and restart of QoE information reporting during RAN overload in NR

In case of overload in RAN, the gNB may temporarily stop the reporting from the UE by sending the RRCReconfiguration message [11] to relevant UEs. The RRCReconfiguration message, see TS 38.331 [11], includes pauseReporting set to TRUE in appLayerMeasConfig to temporarily stop application layer measurement reporting. The access stratum continues recording further information until the data storage capacity for the reporting is fully used or the QoE measurement session ends. Then the recorded data is kept until it is reported to the gNB or when the QoE measurement configuration is released.

When the overload situation in RAN is ended the gNB restart the reporting from the UE by sending the RRCReconfiguration message, see TS 38.331 [11], to relevant UEs. The RRCReconfiguration message is including pauseReporting set to FALSE in appLayerMeasConfig to restart application layer measurement reporting.

The UE application level is not directly affected by the RAN overload.

Next change

#### 4.6.1.1	Activation of QoE measurement task after completion of UE registration procedure

Figure 4.6.1.1-1 and the text below describe the activation of QoE measurement collection after completion of UE registration procedure.

Figure 4.6.1.1-1: QMC activation and reporting example in NR after UE is registered

1.	The MnS Consumer sends createMOI request for QMCJob to UDM that controls the impacted gNB(s), and includes the parameters: serviceType, sliceScope,  areaScope, qoECollectionEntityAddress, qoETarget, qoEReference, mDTAlignmentInformation, availableRANqoEMetrics, qMCConfigFile, and mBSCommunicationServiceType.

2.	The UDM inserts subscriber related data and forwards it to the AMF.

3.	The AMF forwards the configuration parameters serviceType, areaScope, sliceSupportListQMC, measCollEntityIPAddress, qoEReference, nGRANTraceId, availablerANqoEMetrics and containerForAppLayerMeasConfig in message UE Context Modification Request to the impacted gNB.

4.	The gNB checks if the UE capability matches the criteria for serviceType in the QoE measurement configuration information.

5.	If the UE has the wanted UE capability, the gNB starts a UE request session and stores the associated qoECollectionEntityAddress, sends the message RRCReconfiguration to the UE including serviceType, measConfigAppLayerId, transmissionOfSessionStartStop, ran-VisibleParameters, measConfigAppLayerContainer, and availableRANqoEMetrics.

If QoE measurement configuration pertains to MBS communication service, the gNB translates the qoECollectionEntityAddress into qoECollectionEntityIdentity and includes qoECollectionEntityIdentity in the RRCReconfiguration.

NOTE:	The IE measConfigAppLayerId indicates the identity of the application layer measurement configuration, see TS 38.331 [11].

6.	The access stratum in the UE sends an unsolicited response to the application level including app-meas_service_type, meas_config_app_layer_id, transmission_of_session_start-end, ran_visible_periodicity, number_of_buffer_level_entries, report_playout_delay_for_media_startup, and app-meas_config_file. The unsolicited response is for the AT command +CAPPLEVMCNR, see TS 27.007 [5], which is sent from UE Application Level to UE Access Stratum during Registration procedure.

7.	When the application for the specified serviceType starts, the QMC is initiated. To specify the session which is started, the application generates a recordingSessionId.

8.	The application layer sends the AT command +CAPPLEVMRNR, see TS 27.007 [5], including meas_config_app_layer_id and qoe_measurement_status that indicates that a session is started to the access stratum.

9.	The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId,  appLayerSessionStatus and qoECollectionEntityIdentity to the gNB. If mDTAlignmentInformation is received in step 1, the gNB may activate the MDT measurements that are to be aligned with the QoE measurements performed by the UE.

10.	When the QMC is completed or at the end of period for periodic report, the recorded information is collected in a QMC report, see TS 26.247 [6] ] for DASH, TS 26.114 [7] for MTSI or TS 26.118 [13] for VR.

11.	The application layer sends the AT command +CAPPLEVMRNR, see TS 27.007 [5], including meas_config_app_layer_id, playout_delay_for_media_startup, number_of_buffer_level_entries, qoe_measurement_status indicating that session has ended and app-meas_report including recordingSessionId to the access stratum.

12.	The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId, appLayerSessionStatus, ran-VisibleMeasurements and measReportAppLayerContainer including recordingSessionId and qoECollectionEntityIdentity to the gNB.

13.	The gNB translates the qoECollectionEntityIdentity to the qoECollectionEntityAddress, if it is needed, and sends the QMC report to the MCE associated to the qoEReference. The report contains the qoEReference, a time stamp, and the RAN transparent container including the recordingSessionId and slice, which contains only the S-NSSAI used. Note that the qoEReference is mapped to the measConfigAppLayerId at gNB on the previous step and is included in QMC report.

Next change

#### 4.6.2.1	NG Based Handover for Signalling Based Activation

The figure 4.6.2.1-1 and the text below describe the handling at NG Based handover between gNBs for Signalling Based Activation case.

Figure 4.6.2.1-1: Handling of QMC activation example in case of NG Based handover in NR for Signalling Based Activation

1. Source gNB sends the message HANDOVER REQUIRED to AMF. The message includes the Source to Target Transparent Container which contains qoEReference, serviceType, measConfigAppLayerId, areaScope, measCollEntityIPAddress, qoEMeasurementStatus, sliceSupportListQMC, nGRANTraceId, availableRANVisibleqoEMetrics, containerForAppLayerMeasConfig, ran-VisibleParameters, and mBSCommunicationServiceType..

2. AMF sends the message HANDOVER REQUEST to the Target gNB. The message includes the Source to Target Transparent Container which contains the same parameters as in step 1.

3. The target gNB checks if the measurements will be continued or not.

4. The target gNB sends the message HANDOVER REQUEST ACKNOWLEDGE to the AMF.

5. The AMF sends the message HANDOVER COMMAND to the source gNB.

6. The source gNB sends the message RRCReconfiguration to the UE.

7. The UE sends the message RRCReconfigurationComplete to the gNB.

8. When the ongoing QMC is completed or end of periodic reporting is reached, the recorded information is collected in a QMC report, see TS 26.247 [6] for DASH, TS 26.114 [7] for MTSI or TS 26.118 [13] for VR, at UE Application Level.

9. The application layer sends the AT command +CAPPLEVMRNR , see TS 27.007 [5], including meas_config_app_layer_id, qoe_measurement_status playout_delay_for_media_startup, number_of_buffer_level_entries, report_initial_playout_delay, and the app-meas_report to the access stratum.

10. The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId, appLayerSessionStatus, measReportAppLayerContainer, and qoECollectionEntityIdentity to the gNB.

11. The gNB translates the qoECollectionEntityIdentity to the qoECollectionEntityAddress, if it is needed, and sends the QMC report to the MCE associated to the qoEReference. The report contains the qoEReference, a time stamp, and the RAN transparent container including the recordingSessionId and slice (which contains only S-NSSAI that is used). Note that the qoEReference is mapped to the measConfigAppLayerId at gNB on the previous step and is included in QMC report.

Next change

## 5.5	QMC configuration file (container) (M)

The QMC configuration file is a container that is specified in Annex L of TS 26.247 [6], clause 16.5 of TS 26.114 [7] or clause 9.4 of TS 26.118 [13].

The corresponding parameter name for this container in Radio Resource Control (RRC) Protocol specification is measConfigAppLayerContainer in case of NR and LTE, see TS 38.331 [11], TS 25.331 [4] or "Container for application layer measurement configuration" in case of UTRAN, see TS 36.331 [8]

For NR, the QMC configuration file parameter in the management interface defines the location of the container..

Next change

## 5.8	Service type (M)

Which kind of service that shall be recorded.

-	DASH (0)

-	MTSI (1)

-	VR (2)

DASH measurements are specified in TS 26.247 [6]. MTSI measurements are specific in TS 26.114 [7]. VR measurements are specified in TS 26.118 [13].

Next change

## 5.15	MBS Communication Service Type (O)

The MBS Communication Service Type parameter specify which type of MBS communication service the QoE measurement configuration pertains to. See TS 38.413 [12]

End of changes