3GPP TSG-SA5 Meeting #163	S5-254282

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0576 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.623 Correct several issues for the IOCs related to MADCOL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20 |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Following issues observed in the TS28623_ManagementDataCollectionNrm.yaml and needs to be corrected: 1. ManagementData <<choice>> includes the CHOICE_1.1 mgtDataCategory and CHOICE_2.1 mgtDataName, however, such choice information is missing in TS28623_ManagementDataCollectionNrm.yaml. 2. The multiplicity for attribute targetNodeFilter is “*”, however targetNodeFilter only contain one NodeFilter in TS28623_ManagementDataCollectionNrm.yaml. 3. Several attributes of ManagementDataCollection is defined as mandatory attributes, including managementData, targetNodeFilter, collectionTimeWindow, reportingCtrl and jobId, however, such mandatory property is missing in TS28623_ManagementDataCollectionNrm.yaml. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the definition for ManagemenetDataCollection IOC in TS28623_ManagementDataCollectionNrm.yaml to align with stage 2 in TS 28.622. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incosistence between stage 2 and stage 3 for ManagementDataCollection IOC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Following YAML files normatively defined in the forge are updated: - OpenAPI/TS28623_ManagementDataCollectionNrm.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1916 at commit 47be9b241a29a77cbf8515fb05ffacde38e33ca6

*** START OF CHANGE 1 ***

*** OpenAPI/TS28623_ManagementDataCollectionNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Management Data Collection NRM

version: 19.5.0

description: >-

OAS 3.0.1 definition of the Management Data Collection NRM fragment

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.623; Generic NRM, Management Data Collection NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.623/

paths: {}

components:

schemas:

#-------- Definition of types for name-containments ------

SubNetwork-ncO-ManagementDataCollectionNrm:

type: object

properties:

ManagementDataCollection:

$ref: '#/components/schemas/ManagementDataCollection-Multiple'

#-------Definition of generic IOCs ----------#

#-------- Definition of types-----------------------------------------------------

ManagementData:

oneOf:

- type: array

items:

$ref: '#/components/schemas/MgmtDataCategory'

uniqueItems: true

- type: array

items:

$ref: '#/components/schemas/MgmtDataName'

uniqueItems: true

MgmtDataCategory:

type: string

enum:

- COVERAGE

- CAPACITY

- ENERGY_EFFICIENCY

- MOBILITY

- ACCESSIBILITY

MgmtDataName:

type: string

NodeFilter:

type: object

properties:

areaOfInterest:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AreaOfInterest'

networkDomain:

type: string

enum:

- CN

- RAN

cpUpType:

type: string

enum:

- CP

- UP

sst:

type: integer

objectInstances:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

#-------- Definition of concrete IOCs --------------------------------------------

ManagementDataCollection-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

managementData:

$ref: '#/components/schemas/ManagementData'

targetNodeFilter:

type: array

items:

$ref: '#/components/schemas/NodeFilter'

uniqueItems: true

collectionTimeWindow:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

reportingCtrl:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/ReportingCtrl'

dataScope:

type: string

enum:

- SNSSAI

- 5QI

- PLMN

condition:

type: string

processMonitor:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/ProcessMonitor'

consolidateOutput:

type: boolean

jobId:

type: string

required:

- managementData

- targetNodeFilter

- collectionTimeWindow

- reportingCtrl

- jobId

#-------- Definition of YAML arrays for name-contained IOCs ----------------------

ManagementDataCollection-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagementDataCollection-Single'

#-------- Definitions in TS 28.623 for TS 28.532 ---------------------------------

resources-mgmtDataCollectionNrm:

oneOf:

- $ref: '#/components/schemas/ManagementDataCollection-Single'

<CODE ENDS>

*** END OF CHANGE 1 ***

End of Changes