3GPP TSG-SA5 Meeting #163	S5-254310

Wuhan, China, 13 – 17 October 2025

Source:	Nokia

Title:	Pseudo-CR on CCL for Handover optimization

Document for:	Approval

Agenda item:	6.20.10

Spec:	3GPP TS28.889

Version:	0.0.0

Work Item:	Closed Control Loop Management phase2

Comments

This pCR is to add use case and requirements for CCL for Handover optimization

- WT-3: Study the need for CCL enhancements enabling automated, efficient network management.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[X]	3GPP TS 28.104: “Management and orchestration; Management Data Analytics (MDA)”

* * * Next Change * * * *

# 5. 			Use Cases

### 5.X	CCL for Handover control

5.X.1	Description

3GPP TS 28.104 [X], specified MDA reports on root cause analysis of ongoing issues, predictions of potential issues and corresponding relevant causes and recommended actions for preventions, and/or prediction of network and/or service demands. MDA reports include descriptive, diagnostic, predictive and prescriptive analysis for Handover optimization. The Mns consumer may want to compose MDA capabilities for descriptive, diagnostic, predictive and prescriptive Handover optimization analysis as part of a closed control loop for Handover optimization. The management system should enable the MnS consumer to compose the Handover optimization closed control loop.

#### 5.X.2	Potential requirements

REQ- CCL-ES-1: The management system should enable the MnS consumer to compose a Handover optimization closed control loop from descriptive, diagnostic, predictive and prescriptive analytics capabilities on handovers.

### 5.x.3	Possible solutions

-	introduce an IOC for handover optimization closed control loop.

-	The handover optimization closed control loop inherits from a closed control loop

-	Introduce an IOC for a data collection component for handover optimization data

-	Introduce IOCs for descriptive, diagnostic, predictive and prescriptive handover optimization analytics components

-	Introduce attributes on each component that take the analytics output of one component as input to the other components

Note: the attributes to capture the input-output relations of the handover optimization CCL components are FFS.

5.X.4			Evaluation of solutions

TBA

* * * End of Changes * * * *