3GPP TSG-SA5 Meeting #163	S5-254311

Wuhan, China, 13 – 17 October 2025

Source:	Nokia

Title:	Pseudo-CR on CCL for network slice resource optimization

Document for:	Approval

Agenda item:	6.20.10

Spec:	3GPP TS28.889

Version:	0.0.0

Work Item:	Closed Control Loop Management phase2

Comments

This pCR is to add use case and requirements for CCL for network slice resource optimization

- WT-3: Study the need for CCL enhancements enabling automated, efficient network management.

Proposed Changes

* * * First Change * * * *

# 5. 			Use Cases

### 5.X	CCL for network slice resource optimization

5.X.1	Description

3GPP TS 28.104 [X], specified MDA reports on root cause analysis of ongoing issues, predictions of potential issues and corresponding relevant causes and recommended actions for preventions, and/or prediction of network and/or service demands. MDA reports include descriptive, diagnostic, predictive and prescriptive analysis for network slicing. The MnS consumer may want to compose MDA capabilities for descriptive, diagnostic, predictive and prescriptive network slicing analysis as part of a closed control loop for network slice resource optimization. The management system should enable the MnS consumer to compose the network slice resource optimization closed control loop.

#### 5.X.2	Potential requirements

REQ- CCL-ES-1: The management system should enable the MnS consumer to compose a network slice resource optimization closed control loop from descriptive, diagnostic, predictive and prescriptive analytics capabilities on network slicing.

### 5.x.3	Possible solutions

-	introduce an IOC for network slice resource optimization closed control loop.

-	The network slice resource optimization closed control loop inherits from a closed control loop

-	Introduce an IOC for a data collection component for network slice resource optimization data

-	Introduce IOCs for descriptive, diagnostic, predictive and prescriptive network slice resource optimization analytics components

-	Introduce attributes on each component that take the analytics output of one component as input to the other components

Note: the attributes to capture the input-output relations of the network slice resource optimization CCL components are FFS.

5.X.4			Evaluation of solutions

.

* * * End of Changes * * * *