3GPP TSG-SA5 Meeting #163	S5-254355

Wuhan, China, 13 - 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.260 | CR | 0449 | rev | 0 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 32.260 IMS Call Charging Optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CSCN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSIMSCH |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For IMS offline charging through Nchf interface, only MRFC has “Expiration of Interim interval” as chargeable event but other IMS nodes don’t have this. Actually, other IMS nodes also need this chargeable event. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For IMS offline charging through Nchf interface, add “Expiration of Interim Interval” as chargeable event for other IMS nodes besides MRFC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without this charging event, CHF can’t generate time limit CDR. Also, CHF can’t determine a long duration call is still ongoing or actually it has already been terminated but Charging Data Request [Termination] is lost. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.2.1.2	Service based interface Nchf specific

The Charging Data Request and Charging Data Response are exchanged between the IMS nodes and the CHF using Nchf, based on either PEC or SCUR scenarios specified in TS 32.290 [45].

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in this clause.

The CHF address can be selected from IMS signalling as per the TS 24.229 [204] or can be configured locally.

The CHF services may be selected by local configuration. IMS nodes (except IMS-GWF) may consume offline only charging service via Nchf interface.

The IMS nodes for which SIP method a charging Data Request is sent shall be operator configurable. The tables below describe all possible Charging Data Requests that might be sent from IMS nodes.

The generation of a Charging Data Request message by the IMS nodes in response to a particular "triggering SIP method" shall be operator configurable (enable or disable).

Table 5.2.1.2-1: Default Trigger conditions in IMS nodes (except MRFC and AS)

| Trigger Conditions | Trigger level | Offline only charging default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| SIP 2xx acknowledging a SIP INVITE | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Initial] |
| SIP ACK acknowledging a SIP INVITE |  | Immediate | Not Applicable | Not Applicable |  |
| ISUP: ANM |  | Immediate | Not Applicable | Not Applicable |  |
| SIP 2xx acknowledging a RE-INVITE or SIP UPDATE [e.g. change in media components terminating identity change] | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Update] |
| SIP ACK acknowledging an initial RE-INVITE or SIP UPDATE | - | Immediate | Not Applicable | Not Applicable |  |
| SIP 1xx provisional response, mid-dialog requests, mid-dialog responses and SIP INFO embedding RTTI XML body | - | Immediate | Not Applicable | Not Applicable |  |
| ISUP charging ASE |  | Immediate | Not Applicable | Not Applicable |  |
| SIP response (4xx, 5xx or 6xx), indicating an unsuccessful SIP RE-INVITE or SIP UPDATE |  | Immediate | Not Applicable | Not Applicable |  |
| Expiration of Interim | - | Immediate | Not Applicable | Not Applicable |  |
| SIP BYE message (both normal and abnormal session termination cases) | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Termination] |
| SIP 2xx acknowledging a SIP BYE message (only when last user location information of originating/ terminating party is required by operator for legal purpose). | - | Immediate | Not Applicable | Not Applicable |  |
| SIP Final Response (4xx, 5xx or 6xx), indicating an unsuccessful SIP session set-up procedure | - | Immediate | Not Applicable | Not Applicable |  |
| ISUP: REL |  | Immediate | Not Applicable | Not Applicable |  |
| Aborting a SIP session set-up procedure, using an internal trigger, or a SIP CANCEL | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |
| Deregistration | - | Immediate | Not Applicable | Not Applicable |  |
| SIP Final/Redirection Response 3xx | - | Immediate | Not Applicable | Not Applicable |  |
| SIP NOTIFY | - | Immediate | Not Applicable | Not Applicable |  |
| SIP MESSAGE | - | Immediate | Not Applicable | Not Applicable |  |
| SIP REGISTER | - | Immediate | Not Applicable | Not Applicable |  |
| SIP SUBSCRIBE | - | Immediate | Not Applicable | Not Applicable |  |
| SIP REFER | - | Immediate | Not Applicable | Not Applicable |  |
| SIP PUBLISH | - | Immediate | Not Applicable | Not Applicable |  |
| SIP Final Response (4xx, 5xx or 6xx), indicating an unsuccessful session-unrelated procedure | - | Immediate | Not Applicable | Not Applicable |  |

Second change

5.2.1.2	Service based interface Nchf specific

For offline only charging, the following details of chargeable events and corresponding actions in the IMS nodes (except MRFC) are defined in Table 5.2.1.2-3:

Table 5.2.1.2-3: Chargeable events and their related actions in IMS nodes (except MRFC)

| Chargeable event | Conditions | IMS-GWF and AS action |
| --- | --- | --- |
| SIP 2xx acknowledging a SIP INVITE |  | Charging Data Request [Initial] |
| SIP ACK acknowledging a SIP INVITE |  | Charging Data Request [Initial] |
| ISUP: ANM |  | Charging Data Request [Initial] |
| SIP 2xx acknowledging a RE-INVITE or SIP UPDATE [e.g. change in media components terminating identity change] |  | Charging Data Request [Update] |
| SIP ACK acknowledging an initial RE-INVITE or SIP UPDATE |  | Charging Data Request [Update] |
| SIP 1xx provisional response, mid-dialog requests, mid-dialog responses and SIP INFO embedding RTTI XML body |  | Charging Data Request [Update] |
| ISUP charging ASE |  | Charging Data Request [Update] |
| SIP response (4xx, 5xx or 6xx), indicating an unsuccessful SIP RE-INVITE or SIP UPDATE |  | Charging Data Request [Update] |
| Expiration of Interim Interval |  | Charging Data Request [Update] |
| SIP BYE message (both normal and abnormal session termination cases) |  | Charging Data Request [Termination], indicating that charging session is terminated |
| SIP 2xx acknowledging a SIP BYE message (only when last user location information of originating/ terminating party is required by operator for legal purpose). |  | Charging Data Request [Termination], indicating that charging session is terminated |
| SIP Final Response (4xx, 5xx or 6xx), indicating an unsuccessful SIP session set-up procedure |  | Charging Data Request [Termination], indicating that charging session is terminated |
| ISUP: REL |  | Charging Data Request [Termination], indicating that charging session is terminated |
| Aborting a SIP session set-up procedure, using an internal trigger, or a SIP CANCEL |  | PEC: Charging Data Request [Event] |
| Deregistration |  | PEC: Charging Data Request [Event] |
| SIP Final/Redirection Response 3xx |  | PEC: Charging Data Request [Event] |
| SIP NOTIFY |  | PEC: Charging Data Request [Event] |
| SIP MESSAGE |  | PEC: Charging Data Request [Event] |
| SIP REGISTER |  | PEC: Charging Data Request [Event] |
| SIP SUBSCRIBE |  | PEC: Charging Data Request [Event] |
| SIP REFER |  | PEC: Charging Data Request [Event] |
| SIP PUBLISH |  | PEC: Charging Data Request [Event] |
| SIP Final Response (4xx, 5xx or 6xx), indicating an unsuccessful session-unrelated procedure |  | PEC: Charging Data Request [Event] |

End of change