3GPP TSG-SA5 Meeting #163	S5-254407

Wuhan, China, 13 - 17 October 2025

Source:	ZTE Corporation

Title:	Pseudo-CR on TR 28.881 Add Solution to Support to Express Guarantee Requirements in an Intent

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	V0.0.0

Work Item:	FS_IDMS_MN_Ph4

Comments

In SA5#162, the use case and requirements on support to express guarantee requirements in an intent is agreed. This contribution propose to add possible solutions.

Proposed Changes

* * * First Change * * * *

4.8 Use case #8: Support to express guarantee requirements in an intent

4.8.1 Description

In today’s highly dynamic and heterogeneous network environment, Intent-Driven Management Service (IDMS) is emerging as the foundational paradigm for advanced network operations and automation.  Nevertheless, practical deployments reveal a critical capability gap: the absence of robust support for dynamic resource reservation and release.

Intent expression, translation, negotiation, and fulfilment are intrinsically coupled to resource allocation—e.g., bandwidth, capacity and spectrum.  When the IDMS is used for communication service assurance in a specific time window, the MnS consumer should be allowed to express the guarantee requirements in the intent. Then, the MnS Producer can perform some actions (e.g., resource reservation) to guarantee the intent fulfilment in the future. Because these resources are highly time-variant, the current IDMS lacks the ability for the MnS Consumer to proactively reserve or release them on demand.  This shortcoming results in inefficient resource utilisation and reduced flexibility in intent fulfilment. Additionally, after an MnS Producer has classified an intent as “FEASIBLE” following the initial feasibility check, subsequent changes in network resource can render the intent “INFEASIBLE” before intent fulfilment is requested without actions taken for guarantee. This temporal inconsistency, caused by the absence of continuous feasibility validation, will lead to fulfilment failures, undermining the reliability of intent-based operations and degrading overall network performance.

4.8.2 Potential requirements

REQ-IDMS_Resource-1: The intent driven MnS should have the capability to allow MnS consumer to express guarantee requirement in an intent.

4.8.3 Potential solutions

To support to express guarantee requirements in an intent, enhancement on intentMgmtPurpose is proposed. Add a new  GuaranteeReq <<datatype>> in the Intent IOC to allow the MnS Consumer to specify Guarantee Requirements (e.g., resource reservation) for the intent. The GuaranteeReq <<datatype>> may include the following attributes:

- guaranteeIndicator, represents that the Consumer indicates the Producer should reserve resource to guarantee the intent fulfilment in the future.

- guaranteeContext, represents the context of guarantee, e.g., the related resource should be in place before the time point in the future.

4.8.4  Evaluation of potential solutions