3GPP TSG-SA5 Meeting #163	S5-254446

# Wuhan, China, 13 - 17 Oct 2025

# Source:	Samsung

# Title:		RET Analytics

# Document for:	   Agreement

# Agenda Item:	6.20.7

# 1	Decision/action requested.

The group is asked to endorse the proposal.

# 2	References

None

# 3	Rationale

This contribution provides a new UC and Requirements for RET analytics.

* * * First Change * * * *

# 5	Use cases, potential requirements and possible solutions

## 5.x	Remote Electrical Tilt (RET) and Transmission Power analytics (TP)

#### 5.x.1	Use cases

Operators establish their own guidelines for deploying antenna tilts, both mechanical and electrical, in cells, primarily based on LTE/NR layers and traffic management strategies. Initial tilt and power configurations may not be optimal for all cells, necessitating ongoing fine-tuning. The initial tilt may not account for changes in subscriber movement or infrastructure developments. Factors influencing optimal tilt and power can evolve over time, including:

Subscriber Movement: Dynamic changes in user behavior, such as daily commutes from office to home or weekend outings to recreational areas, can impact network performance requiring the modification in the tilt.

Infrastructure Growth: The addition of new cell sites can introduce interference with neighboring cells. Interference arises when signals from the new site overlap with or disrupt existing signals. This would effect performance and can be mitigead with modification in the tilt.

Weather Conditions: Atmospheric phenomena like rain, snow, and fog can affect radio wave propagation, leading to signal attenuation or distortion. Tilt modfification may minimize the impact.

It is desirable to use MDA analytics to generate optimal tilt and power configuration over the time. It is essential to follow initial antenna tilt deployment with subsequent fine-tuning using MDA analytics. The producer employs a machine learning-based model that leverages strong correlations between performance, fault and configuration data of the network. This AI/ML model analyzes the impacts of coverage and interference in relation to user traffic behavior, including mobility and weather conditions. It generates recommendations for fine-tuning RET and optimizing transmission power per cell.

#### 5.x.2	Potential requirements

REQ-RET-DATA-1: MDA capability for RET analysis shall include providing analytics for indicating if a change in RET and TP is required for a specific cell.

REQ-RET-DATA-1: MDA capability for RET analysis shall include providing analytics for indicating the optimal value for RET and TP.

REQ-RET-DATA-1: MDA capability for RET analysis shall include providing analytics for indicating the context (timestamp, weather) under which the recommended change in RET and TP shall be performed.

#### 5.x.3	Possible solutions

TBD

* * * Next Change * * * *

* * * End of Change * * * *