3GPP TSG-SA5 Meeting #163	S5-254491

Wuhan, China, 13 - 17 October 2025

# Source:	Huawei

# Title:	Rel-20 pCR 32.872 Introduce the background for roaming charging

# Document for:	Approval

Agenda Item:	7.5.2

1	Decision/action requested

This is a pCR to add background in TR 32.872.

2	References

[1]	3GPP TR 32.872: "Study on 5GA roaming charging reliability enhancement".

3	Rationale

This pCR proposes to introduce the background for roaming charging in TR 32.872[1].

4	Detailed proposal

The following changes are proposed to be incorporated into TR 32.872 [1].

First change

4	Background

4.1	General

5G roaming charging are specified in middle tier charging specifications. Take data connectivity as an example, the home routed roaming charging, LBO roaming charging with N47 and LBO roaming charging with N107 are specified in 3GPP TS 32.255[2]. The common failure handling includes CTF detected failure, CHF detected failure, CHF as NF Consumer detected failure, Retry handling and Response code handling are specified in clause 5.5 of 3GPP TS 32.290[3]. In order to support more failure handling scenarios in roaming, the enhancement of failure handling will be studied.

4.2	Home routed Roaming charging

Figure 4.2.1 illustrates the Home Routed roaming charging architecture depicted in clause 4.2 of 3GPP TS 32.255[2]. The N40 reference point is defined for the interactions between H-SMF and H-CHF and between V-SMF and V-CHF in the reference point representation.

Figure 4.2.1: 5G data connectivity converged charging architecture in roaming Home Routed reference point representation

4.3	Local Breakout Roaming charging

Figure 4.3.1 illustrates the Local Breakout roaming charging with N47 architecture, as specified in clause 4.2.6 of 3GPP TS 32.255[2]. The N47 reference point is defined for the interactions between V-SMF and H-CHF.

Figure 4.3.1: 5G data connectivity converged charging architecture in Local Breakout V-SMF to H-CHF scenario reference point representation

Figure 4.3.2 illustrates the Local Breakout roaming charging with N107 architecture, as specified in clause 4.2.6a of 3GPP TS 32.255[2]. The N107 reference point is defined for the interactions between V-CHF and H-CHF.

Figure 4.3.2: 5G data connectivity converged charging architecture in Local Breakout Inter-CHF scenario reference point representation

End of change