3GPP TSG-SA5 Meeting #163	S5-254494

Wuhan, China, 13 - 17 October 2025

# Source:	Huawei

# Title:	Rel-20 pCR 32.872 Introduce the usecase3 on Local Breakout Inter CHFs scenario

# Document for:	Approval

Agenda Item:	7.5.2

1	Decision/action requested

This is a pCR to TR 32.872 Introduce a usecase on Local Breakout Inter CHFs scenario.

2	References

[1]	3GPP TR 32.872: "Study on 5GA roaming charging reliability enhancement".

3	Rationale

The message flow of PDU session charging in roaming with local breakout scenarios is specified in clause 5.2.2.18.5, 5.2.2.18.6, 5.2.2.18.7 of 3GPP TS 32.255[2], the V-CHF sends a Charging Data Request to the H-CHF after receiving the Charging Data Request from the V-CTF, but it is undefined how the V-CHF handles the trigger, support feature and timer in the Charging Data Request, i.e., how does the V-CHF determine the values of the trigger, supported features, and timer in the Charging Data Request, which may lead to misalignment between the two charging sessions.

This pCR proposes to introduce a new use case related to charging session alignment on the topic of Local Breakout Inter CHFs scenario.

4	Detailed proposal

Propose to incorporate the following change into the TR 32.872 [1]

First change

#### 5.x.1.5	Use Case #3: Association of two charging sessions

This use case focuses on the association of two charging sessions when inter-CHF communication applies, e.g.:

- 	charging session#1: the charging session between V-SMF(CTF) and V-CHF;

- 	charging session#2: the charging session between V-CHF and H-CHF.

As the V-CHF sits between the two charging sessions, it takes responsibility to associate the two charging sessions to avoid failure, e.g.,

- 	the V-CHF needs to determine the triggers in charging session#2, e.g., the triggers are intended for the CTF or the V-CHF;

- 	charging session#1 supports the URLLC feature, whereas charging session#2 does not, which may result in a URLLC charging failure;

- 	the negotiated value for the Unit Count Inactivity Timer of charging session #1 is 200 seconds, whereas for charging session #2 it is 100 seconds, which may result in the premature termination of charging session #2 while charging session #1 remains active.

Figure 5.x.1.5-1: An example of charging session unalignment

The potential charging requirements for this UC are: REQ-3GPPCH-LBIC-03.

### 5.x.2	Potential charging requirements

REQ-3GPPCH-LBIC-03: The V-CHF shall support the management of charging session alignment.

### 5.x.3	Key issues

#### 5.x.3.3	Key issue #3: Session alignment enhancement

This key issue is for investigating how to support REQ-3GPPCH-LBIC-03. This investigation covers the following:

-	Identification of the management for charging session alignment;

End of change