3GPP TSG-SA5 Meeting #163	S5-254525

Wuhan, China, 13th Oct 2025 - 17th Oct 2025

Source:	Nokia

Title:	Pseudo-CR on Rel-20 TR 28.885 Add new use case for enhancements to reporting EC and EE at per network slice granularity

Document for:	Approval

Agenda item:	6.20.5

Spec:	3GPP TR 28.885

Version:	0.0.0

Work Item:	FS_Energy_Ph4_OAM

Comments

To introduce a new use case to study for " Enhancements to support the information required by Energy Information Function (EIF)", related to WT-2.

This pCR introduces use case and potential requirements related to WT-2 of the SI FS_Energy_Ph4_OAM.

"

WT-2: Study 3GPP management system enhancements to support the information required by Energy Information Function (EIF) (defined in TS 23.501) as part of SA2 5GA Rel-20 SI FS_EnergySys_Ph2.

NOTE 4: Information required by EIF as identified by SA2 study (FS_EnergySys_Ph2) in TR 23.700-67.

"

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[A]	3GPP TS 22.261: "Service requirements for the 5G system".

[B]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[C]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[E] 	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[F]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[G]	3GPP TR 23.700-67: "Study on Energy Efficiency and Energy Saving; Phase 2".

[I]	ETSI ES 202 706-1 V1.7.1 (2022-08): "Environmental Engineering (EE); Metrics and measurement method for energy efficiency of wireless access network equipment; Part 1: Power consumption - static measurement method".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Energy Efficiency (EE): ratio between performance and energy consumption.

NOTE 1: the performance may be measured based on e.g. data volume, latency, number of active users, etc..

NOTE:	This definition is taken from 3GPP TS 28.310 [B].

Energy Consumption (EC): integral of power consumption over time.

NOTE 2: see ETSI ES 202 706-1 [I].

NOTE:	This definition is taken from 3GPP TS 28.310 [B].

Network Slice: A logical network that provides specific network capabilities and network characteristics.

NOTE:	This definition is taken from 3GPP TS 23.501 [F].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

EC	Energy Consumption

EE	Energy Efficiency

S-NSSAI	Single Network Slice Selection Assistance Information

* * * Next Change * * * *

### 5.2.C	Use case #<C>: Energy consumption and Energy Efficiency estimation and reporting at per network slice granularity

#### 5.2.C.1	Description

The requirements for energy consumption monitoring at per network slice granularity is specified in clause 6.15a.4.2 of TS 22.261 [A].

Several solutions identified for the key issues in TR 23.700-67 [G] include 5GC NFs to collect slice level energy consumption and energy efficiency from OAM (using SA WG5 defined services).

An S-NSSAI identifies a Network Slice. An S-NSSAI with a non-standard value identifies a single Network Slice within the PLMN with which it is associated (see clause 5.15.2.1 of TS 23.501 [F]).

Network Slice Energy Consumption (EC) KPI is specified in clause 6.7.3.3 of TS 28.554 [C]. This is reported at KPI object NetworkSlice IOC (defined in clause 6.2.1 of TS 28.541 [E]), while “network slice” in TS 23.501 [F] are identified using S-NSSAI and PLMN ID as described above.

Network slice Energy Efficiency (EE) KPIs are specified in clause 6.7.2 of TS 28.554 [C]. This is reported at KPI object NetworkSlice IOC (defined in clause 6.2.1 of TS 28.541 [E]), while “network slice” in TS 23.501 [F] are identified using S-NSSAI and PLMN ID as described above.

The NetworkSlice IOC is defined in clause 6.3.1 of TS 28.541 [E], with attribute serviceProfileList that holds a list of ServiceProfile (defined in clause 6.3.3 of TS 28.541 [E]) supported by the NetworkSlice MOI. ServiceProfile is a data type with attribute pLMNInfoList (a list of data type PLMNInfo defined in clause 4.3.41 of TS 28.541 [E]) that defines which PLMN and S-NSSAI combinations that are assigned for the service to satisfy service requirements represented by the ServiceProfile in case of network slicing feature is supported. A NetworkSlice MOI can be configured, in attribute serviceProfileList, with a list of ServiceProfile value, where the list can hold one or more than one element. Each element in the list attribute serviceProfileList can be configured with a list PLMN and S-NSSAI combinations in attribute pLMNInfoList. Hence a NetworkSlice MOI can be configured with a list of PLMN and S-NSSAI combinations.

This use case is to study the enhancements to support for energy consumption and energy efficiency monitoring, estimation and reporting at per network slice (as defined in TS 23.501 [F]) granularity, i.e., per S-NSSAI granularity and per PLMN-ID and S-NSSAI granularity in network sharing scenario.

Editor’s note: This use case could be updated subject to conclusions of SA2 study (FS_EnergySys_Ph2) in TR 23.700-67 [G].

#### 5.2.C.2	Potential requirements

PREQ-Energy_Reporting_Network_Slice-1: The 3GPP management system should be able to estimate and report the energy consumption at per network slice granularity.

PREQ-Energy_Reporting_Network_Slice-2: The 3GPP management system should be able to estimate and report the energy efficiency at per network slice granularity.

Editor’s note: The potential requirements in this clause could be updated subject to conclusions of SA2 study (FS_EnergySys_Ph2) in TR 23.700-67 [G].

#### 5.2.C.3	Potential solutions

#### 5.2.C.3.i	Potential solution #<i>: <Potential Solution i Title>

##### 5.2.C.3.i.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

##### 5.2.C.3.i.2	Description

Editor's Note:	This clause further details the potential solution, and any assumptions made.

#### 5.2.C.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions.

* * * End of Changes * * * *