3GPP TSG-SA5 Meeting #163	S5-254527

Wuhan, China, 13th Oct 2025 - 17th Oct 2025

Source:	Nokia

Title:	Pseudo-CR on Rel-20 TR 28.885 Add new use case to study measuring of EC of NE at per Energy Supply granularity

Document for:	Approval

Agenda item:	6.20.5

Spec:	3GPP TR 28.885

Version:	0.0.0

Work Item:	FS_Energy_Ph4_OAM

Comments

To introduce a new use case to study for "Enhancements to Energy Consumption and Energy Efficiency measurements and KPIs", related to WT-4.

This pCR introduces use case and potential requirements related to WT-4 of the SI FS_Energy_Ph4_OAM.

"

WT-4. Study enhancements to Energy Consumption and Energy Efficiency measurements and KPIs for Network Functions and Network Slice.

"

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[A]	3GPP TS 22.261: "Service requirements for the 5G system".

[B]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[X]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[C]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

* * * Next Change * * * *

### 5.4.A	Use case #<A>: Support EC measurement of NE at per Energy Supply granularity

#### 5.4.A.1	Description

Energy supply information can be associated with the Managed Elements (representing the Network Elements) using EnergyInfoGroup IOC (see clause 8.3.3. of TS 28.310 [B]). Energy-related information for each energy source (see EnergySourceInfo defined in clause 8.3.2 of TS 28.310 [B]) of the energy supply (see EnergySupplyInfo defined in clause 8.3.1 of TS 28.310 [B]) includes energy supply mix, carbon emission factor, renewable and non-renewable energy information.

Power, Energy and Environmental (PEE) measurements are defined in clause 5.1.1.19 of TS 28.552 [X], and PNF Energy consumption for ManagedElement is defined in clause 5.1.1.19.3 of TS 28.552 [X].

Energy consumption related KPIs are defined in clause 6.7.3 of TS 28.554 [C]. gNB Estimated Carbon Emission and NG-RAN Estimated Carbon Emission KPIs are defined in clause 6.7.7. of TS 28.554 [C].

A Network Element (represented by Managed Element) can be powered by one or more energy supplies (see TS 28.310 [B]). The Carbon Emission related KPIs defined in TS 28.554 [C] are currently restricted to the scenarios when a Network Element is powered by a single energy supply. This limitation is due to the lack of EC measurements at a per energy supply granularity for a Network Element.

Energy consumption of a Network Element is essential for monitoring and reporting of carbon emissions, carbon intensity, and network energy optimization use cases considering energy supply mix, and energy-related characteristics (defined in TS 22.261 [A]).

This use case is to study the enhancements to enable measurements and reporting of EC of a Network Element at a per energy supply granularity.

#### 5.4.A.2	Potential requirements

PREQ-Energy_Consumption-1: The 3GPP management system should be able to measure and report the energy consumption of a Network Element at a per energy supply granularity.

#### 5.4.A.3	Potential solutions

#### 5.4.A.3.i	Potential solution #<i>: <Potential Solution i Title>

##### 5.4.A.3.i.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

##### 5.4.A.3.i.2	Description

Editor's Note:	This clause further details the potential solution, and any assumptions made.

#### 5.4.A.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions.

* * * End of Changes * * * *